| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0110-v2 |
| --- | --- |

| Title: | EE2-2.1-2.5: Merge-based Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli, Ramin G. Youvalari, Alexandre Tissier, Yao-Jen Chang, Vadim Seregin, Marta Karczewicz | Email: | mabdoli@xiaomi.com yjchang@qti.qualcomm.com |
| Source: | Xiaomi Technologies, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-2.1 and EE2-2.5 exploration experiments, covering different implementation of the TIMD-merge mode. These two tests share a common aspect of proposing algorithms for creating a merge list based on the neighboring TIMD blocks. This list is then sorted based on the template cost and one (or possibly more) candidates are finally kept in the list. A CU-level flag then indicates whether a TIMD-merge candidate is used to derive the fusion IPMs of the CU. Both methods use the same adjacent/non-adjacent map to search for merge candidates in the neighbourhood.

Apart from their common aspects, the main difference between the single test in EE2-2.1 and group of tests in EE2-2.5 is that in EE2-2.5, DIMD blocks are also considered to fill the merge list, while in EE2-2.1, only TIMD neighbours are included.

Here is a summary of the results:

Test EE2-2.1: TIMD-merge mode:

AI { -0.07%, 0.05%, -0.01%, 100.9%, 100.3%}, RA { 0.00%, -0.00%, 0.00%, 100.0%, 100.0%}

Test EE2-2.5a: Intra-merge mode (1 candidate):

AI { -0.07%, 0.02%, 0.01%, 103.7%, 101.5%}, RA { -0.04%, 0.00%, 0.00%, 100.0%, 100.0%}

Test EE2-2.5b: Intra-merge mode without TIMD candidates:

AI { -0.06%, 0.00%, 0.00%, 103.5%, 101.6%}, RA { 0.00%, -0.00%, 0.00%, 100.0%, 100.0%}

Test EE2-2.5c: Intra merge mode with adjacent and non-adjacent neighbors used in current ECM intra tools:

AI { -0.06%, 0.02%, 0.02%, 103.6%, 101.2%}, RA { 0.00%, -0.00%, 0.00%, 100.0%, 100.0%}

Test EE2-2.5d: Intra merge mode with larger candidate list (3 candidates):

AI { -0.14%, 0.02%, 0.02%, 110.3%, 100.5%}, RA { 0.00%, -0.00%, 0.00%, 100.0%, 100.0%}

# Introduction

The TIMD process in ECM [1] is based on a template cost analysis in which several intra modes -- from a candidates list comprising MPMs, wide-angular, DC, horizontal, vertical, and extended modes are tested. Up to three intra modes (IPM1, IPM2,  IPM3) providing the smallest SATD costs are selected and blended in a fusion process according to their respective template cost.

Moreover, DIMD process is based on a template angular direction analysis, implemented by two Sobel filters that measure horizontal and vertical gradients. Then, a Histogram of Gradient (HoG) is generated by mapping computed angles into available IPMs and representing them with amplitudes, showing their similarity to the template texture. Finally, five IPMs with highest HoG amplitudes are selected, and along with the Planar mode, are fused to give the final prediction of the block.

## TIMD-merge mode (Test EE2-2.1)

The TIMD merge mode, originally proposed in JVET-AG0106 [2], inherits the TIMD modes from the previously TIMD coded blocks. For this, the adjacent and non-adjacent spatial neighboring blocks are scanned and if the scanned block is coded in TIMD or TIMD merge mode, then its TIMD information (prediction modes, fusion flag, fusion weights and wide-angle conditions of TIMD modes, MTS transform types) are added to a TIMD merge list. Up to five TIMD candidates are added into the list.

### Merge list construction

Figure 1 shows the non-adjacent spatial neighboring blocks that are used in TIMD merge list generation.

Fig 1. Adjacent and non-adjacent spatial neighboring candidates for TIMD merge mode (EE2-2.1) and intra merge mode (EE2-2.5)

Table 1: Initial TIMD merge list

| Cand | Intra modes | Fusion weights |
| --- | --- | --- |
| 1 | (IPM1,1, IPM1,2, IPM1,3) | (W1,1, W1,2, W1,3) |
| 2 | (IPM2,1, IPM2,2, IPM2,3) | (W2,1, W2,2, W2,3) |
| . . . | . . . | . . . |
| 5 | (IPM5,1, IPM5,2, IPM5,3) | (W5,1, W5,2 , W5,3) |

A list of five TIMD candidates is created from the neighborhood. After forming the TIMD merge list, the merge candidates are sorted based on the SAD cost over the template of the current block. For the cost calculation, the SAD costs of the tuples of TIMD modes are normalized using their fusion weights. For example, if a merge candidate has three intra modes (IPM1, IPM2, IPM3) with fusion weights (W1, W2, W3), the candidate’s cost is calculated as:

Cost = (SAD(IPM1) * W1 + SAD(IPM2) * W2 + SAD(IPM3) * W3 ) >> shift

The template size for sorting of TIMD merge candidates is decided to be one line from the immediate neighbourhood of the block.

In Test 2.1, the best candidate from the list is selected to be potentially included in encoder RDO.

The TIMD merge mode is applied to only luma blocks. Moreover, the mode is disabled for 4x4 block sizes.

In the encoder side, the SAD template costs calculation may reuse the SAD costs of the MPM list reordering in order to reduce the encoder runtime.

### Bitstream Signalling

Usage of the mode is signaled with a CABAC coded PU level flag. The TIMD merge mode is used as a sub-mode of TIMD tool.

## Intra-merge mode (Test EE2-2.5)

In the proposed method, a list of intra mode candidates is constructed using the intra modes from the neighbour blocks. It is similar to inter merge mode, but instead of motion vector the neighbour intra modes are added to the list. In this EE test, the merged neighbouring blocks are those coded by DIMD and TIMD modes. A flag is signalled to indicate whether to enable the proposed intra merge mode. If the flag is true, the intra merge list is constructed with 5 candidates at most. Then the best candidate is selected among these 5 candidates by comparing the template costs as described in Section 1.1. The neighbouring blocks considered for the intra merge list construction are as shown in Figure 1.

# Results

# Tables below summarize the coding efficiency and runtime impact of the proposed methods in ECM-13.0 [3] using the JVET common test conditions [4].

## Test EE2-2.1:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.10% | -0.02% | 101.1% | 100.3% | 100.8% | 100.4% |
| Class A2 | -0.07% | 0.03% | -0.02% | 100.3% | 100.3% | 100.3% | 100.4% |
| Class B | -0.07% | -0.04% | 0.00% | 100.9% | 100.0% | 98.8% | 100.2% |
| Class C | -0.08% | 0.09% | -0.01% | 101.3% | 99.9% | 100.8% | 100.2% |
| Class E | -0.06% | 0.14% | 0.01% | 100.5% | 101.3% | 100.2% | 100.2% |
| Overall | -0.07% | 0.05% | -0.01% | 100.9% | 100.3% | 100.1% | 100.3% |
| Class D | -0.04% | 0.02% | -0.05% | 100.2% | 94.4% | 100.2% | 100.1% |
| Class F | -0.04% | 0.07% | 0.06% | 101.0% | 100.1% | 99.1% | 100.2% |
| Class TGM | -0.04% | -0.01% | -0.03% | 100.3% | 100.1% | 102.3% | 100.4% |

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |  |
|  | Y | U | V |  | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -0.03% | 0.06% | -0.09% |  |  |  |  |  |
| Class C | -0.02% | 0.26% | 0.11% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D | 0.00% | -0.20% |  | 0.13% |  |  |  |  |
| Class F | 0.02% | 0.07% | 0.06% |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |

## Test EE2-2.5

Test EE2-2.5a:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | -0.01% | 0.03% | 103.2% | 102.0% | 100.1% | 100.1% |
| Class A2 | -0.08% | 0.02% | 0.06% | 103.3% | 102.0% | 100.1% | 100.2% |
| Class B | -0.06% | -0.03% | -0.09% | 104.3% | 101.6% | 100.1% | 100.1% |
| Class C | -0.05% | 0.10% | 0.11% | 103.7% | 100.8% | 100.1% | 100.0% |
| Class E | -0.06% | 0.04% | -0.01% | 103.8% | 101.0% | 100.0% | 100.0% |
| Overall | -0.07% | 0.02% | 0.01% | 103.7% | 101.5% | 100.1% | 100.1% |
| Class D | -0.03% | -0.04% | -0.08% |  |  | 100.1% | 100.0% |
| Class F | -0.06% | 0.12% | 0.05% |  |  | 100.1% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |

|  |  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-13.0 |  |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  | -0.06% | -0.07% | -0.28% |  |  | 100.1% | 100.3% |
| Class A2 |  | -0.02% | -0.16% | -0.06% |  |  |  |  |
| Class B |  | -0.04% | 0.03% | 0.08% |  |  | 100.0% | 100.1% |
| Class C |  | -0.04% | 0.13% | 0.14% |  |  | 100.4% | 100.0% |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  | -0.04% | 0.00% | 0.00% |  |  |  |  |
| Class D |  | -0.03% | -0.10% | -0.01% |  |  | 100.1% | 100.0% |
| Class F |  | 0.03% | 0.16% | 0.13% |  |  | 99.6% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |  |

Test EE2-2.5b:

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |  |
|  | Y | U |  | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% |  | -0.11% | 0.09% | 103.5% | 101.8% | 100.1% | 100.2% |
| Class A2 | -0.08% |  | 0.02% | 0.01% | 102.9% | 101.8% | 100.1% | 100.2% |
| Class B | -0.05% |  | -0.07% | -0.05% | 103.9% | 102.0% | 100.1% | 100.1% |
| Class C | -0.05% |  | 0.03% | 0.01% | 103.8% | 101.5% | 100.1% | 100.0% |
| Class E | -0.03% |  | 0.14% | 0.00% | 103.4% | 100.9% | 100.0% | 100.0% |
| Overall | -0.06% |  | 0.00% | 0.00% | 103.5% | 101.6% | 100.1% | 100.1% |
| Class D | -0.03% |  | 0.02% | -0.09% |  |  | 100.2% | 100.0% |
| Class F | -0.08% |  | 0.07% | 0.07% |  |  | 100.1% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |  |

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.01% | -0.03% | 0.21% |  |  | 100.1% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.02% | -0.16% | 0.45% |  |  | 100.1% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Test EE2-2.5c:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | 0.05% | 0.03% | 103.1% | 101.6% | 100.1% | 100.1% |
| Class A2 | -0.06% | 0.03% | 0.02% | 103.0% | 101.6% | 100.1% | 100.2% |
| Class B | -0.06% | -0.07% | -0.07% | 104.3% | 101.5% | 100.1% | 100.1% |
| Class C | -0.05% | 0.03% | 0.12% | 103.3% | 100.4% | 100.1% | 100.0% |
| Class E | -0.05% | 0.10% | 0.01% | 103.9% | 100.9% | 100.1% | 100.1% |
| Overall | -0.06% | 0.02% | 0.02% | 103.6% | 101.2% | 100.1% | 100.1% |
| Class D | -0.04% | 0.20% | 0.06% |  |  | 100.1% | 100.0% |
| Class F | -0.03% | -0.09% | 0.06% |  |  | 100.1% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.04% | -0.04% | -0.05% |  |  | 100.8% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.03% | 0.07% | 0.11% |  |  | 100.0% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Test EE2-2.5d:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.16% | 0.00% | 0.02% | 108.9% | 99.0% | 100.1% | 100.1% |
| Class A2 | -0.16% | 0.02% | 0.06% | 109.5% | 101.3% | 100.1% | 100.2% |
| Class B | -0.13% | -0.07% | -0.05% | 111.5% | 101.2% | 100.1% | 100.1% |
| Class C | -0.14% | 0.04% | 0.13% | 109.9% | 100.1% | 100.1% | 100.0% |
| Class E | -0.10% | 0.14% | -0.04% | 110.6% | 100.2% | 100.1% | 100.0% |
| Overall | -0.14% | 0.02% | 0.02% | 110.3% | 100.5% | 100.1% | 100.1% |
| Class D | -0.07% | 0.01% | -0.16% |  |  | 100.2% | 100.0% |
| Class F | -0.08% | -0.03% | 0.03% |  |  | 100.1% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.07% | 0.12% | 0.24% |  |  | 101.2% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.03% | 0.03% | 0.03% |  |  | 100.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# References

- Coban et.al, “Algorithm description of Enhanced Compression Model 13 (ECM 13)”, JVET-AH2025, April. 2024.

- R. G. Youvalari, M. Abdoli, “AHG 12: TIMD merge mode”, JVET-AG0106, Jan. 2024.

- ECM-13.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).