| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0131-v2 |
| --- | --- |

| Title: | AHG8: RPR for machine consumption |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Jacob Ström | Email: | christopher.hollmann@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution results for RPR for machine consumption using VTM-23.3 are presented. Results include the PandaSet, which was adopted at the previous meeting. The reported results are summarized as follows:

- 1.25x: −5.5% | −6.6% (SFU RA | LD); −4.2% | −3.8% (TVD RA | LD); −3.4% | −0.8% (Panda RA | LD)

- 1.5x: −3.1% | −4.7% (SFU RA | LD); −4.0% | 7.5% (TVD RA | LD); −5.8% | −2.0% (Panda RA | LD)

- 2.0x: −2.2% | 1.2% (SFU RA | LD); 1.5% | 18.9% (TVD RA | LD); −9.9% | −8.4% (Panda RA | LD)

It is proposed to expand the description of RPR in the TR on optimizations for encoders and receiving systems for machine analysis of coded video content.

In version 2 of this contribution missing results for the low delay configuration were added.

# Introduction

In this contribution results for using Reference Picture Resampling (RPR) in VTM-23.3 are presented. A previous contribution JVET-AH0130 [1] reported results for VTM-23.0. Compared to this contribution, there are several updates:

- VTM-23.3 is used instead of VTM-23.0

- The curve fitting method adopted at the meeting in Rennes was used to determine the results

- Results for the PandaSet, which was adopted at the meeting in Rennes, are presented

# Test results

Tests were performed according to the test conditions defined in JVET-AH2031 [2]. A summary is presented below, and detailed results can be found in the attached Excel documents.

| SFU-HW | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.25x | 1.5x | 2.0x | 1.25x | 1.5x | 2.0x |
| Class A | 22.3% | 27.8% | 28.0% | −8.5% | −11.0% | 0.5% |
| Class B | −17.9% | −9.4% | −24.2% | −8.1% | −5.7% | −12.8% |
| Class C | 0.1% | −3.1% | −3.0% | −3.2% | 1.9% | 3.0% |
| Class D | −5.9% | −4.6% | 13.0% | −8.1% | −8.5% | 13.6% |
| Average | −5.5% | −3.1% | −2.2% | −6.6% | −4.7% | 1.2% |

| TVD | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.25x | 1.5x | 2.0x | 1.25x | 1.5x | 2.0x |
| Average | −4.2% | −4.0% | 1.5% | −3.8% | 7.5% | 18.9% |

| PandaSet | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.25x | 1.5x | 2.0x | 1.25x | 1.5x | 2.0x |
| Class A1 | −7.9% | −7.0% | −15.0% | −0.1% | −2.9% | −4.9% |
| Class B1 | −3.3% | −5.2% | −8.5% | −1.3% | −1.4% | −8.4% |
| Class C1 | −6.3% | −10.1% | −8.8% | 1.2% | −5.6% | −8.8% |
| Class A2 | −5.5% | −6.1% | −11.6% | −6.8% | −9.1% | −14.6% |
| Class B2 | 0.4% | −1.4% | −8.3% | 2.4% | −1.0% | −8.7% |
| Class C2 | −2.3% | −7.8% | −11.6% | −3.6% | 4.1% | −5.6% |
| Average | −3.4% | −5.8% | −9.9% | −0.8% | −2.0% | −8.4% |

# Discussion

In this contribution we have shown that lowering the resolution using RPR brings a benefit in the domain of compressing video for machine consumption. It is of course possible to lower the resolution without using RPR by just encoding the entire sequence with a lower resolution. In our tests above that would have the same effect. However, we believe that it can be helpful to be able to change resolution over time in response to changes in the content. It is difficult show that with the current test sets, because even after adding PandaSet, there are only very few instances where the performance is hurt by lowering the resolution. In a real-world setting however, we believe there would be several situations where lowering the resolution would be hurting performance. Hence it seems reasonable to us that having the ability to change the resolution using RPR could be beneficial for the overall performance of the system.

We therefore propose to update the CDTR on optimizations for encoders and receiving systems for machine analysis of coded video content [3] with more information about the RPR tool. We propose to add the following short section about RPR to the section on spatial subsampling (added text in green):

  - “7.4 Spatial subsampling

If the analysis of a video shows that it contains primarily large objects, one way to increase the BD-rate performance can be to perform spatial subsampling on the input video. This will result in fewer pixels in subsampled frames to encode, and thus likely faster encoding and bit rate savings. The spatial subsampling can be applied adaptively, for example, based on the diversity characteristics of the visual content and the target bit rate. For example, depending on if the input video is captured by regular camera as natural scenes, or is captured by infrared sensor as thermal images, different spatial subsampling methods can be applied.

One tool that is part of the VVC standard that can be used for the purpose of spatial resampling is the reference picture resampling (RPR) tool. This tool allows the encoder to choose to code parts of the input video at different resolutions without the need of coding an intra picture at the specified changed resolution. Predictions can be made from all allowed pictures regardless of their resolution.”

A modified version detailing the proposed changes is also attached to this contribution.

# Conclusion

In this contribution we have shown that lowering the resolution using RPR brings a benefit to the domain of compressing video for machine consumption. We therefore propose adding a short section about RPR to the CDTR.

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

| [1] | C. Hollmann, J. Ström, P. Wennersten and K. Andersson, "AHG8: RPR for machine consumption," JVET-AH0130, Rennes, FR, April 2024. |
| --- | --- |
| [2] | S. Liu and C. Hollmann, "Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content," JVET-AH2031, Rennes, FR, April 2024. |
| [3] | C. Hollmann, J. Chen and S. Liu, "Optimization of encoders and receiving systems for machine analysis of coded video content (draft 5)," JVET-AH2030, Rennes, FR, April 2024. |