| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0134-v1 |
| --- | --- |

| Title: | AhG11: LOP3 filter with multiscale blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruiying Yang Francesco Cricri Maria Santamaria Honglei Zhang Jani Lainema Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Tel: Email: | {ruiying.yang, francesco.cricri, maria.santamaria_gomez, honglei.1.zhang, jani.lainema, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes the use of multiscale blocks in the backbone of the LOP3 filter, where the input signal to a residual block is processed at two different scales. It is reported that applying this approach in the RA configuration yields average BD-rates of -0.17% (Y), -0.37% (Cb), -0.77% (Cr) compared to NNVC-9.1 with NN intra tool ON and LOP3 tool ON. It is also reported that the BD-rates for the AI configuration are -0.11% (Y), -0.34% (Cb), -0.28% (Cr).

# Multiscale blocks

The architecture of LOP3 [1] is modified by extending the residual blocks of the backbone (also referred to as backbone blocks) to support multiscale processing. Figure 1 illustrates the proposed multiscale backbone block, where two different scales are used.

Figure 1. (top) The original backbone block. (bottom) The proposed multiscale backbone block.

# Training

The proposed model was trained based on the stage 3 training for LOP3.

Table 1 shows the details of NN information during the training stage.

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
|  | Framework: | PyTorch 2.1.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage 3: 60 epochs |
|  | Batch size: | 64 |
|  | Loss function: | L1 and MSE |
|  | Training time |  |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | The LOP2 stage 3 dataset |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage3: 2e-4, milestones [41, 51, 55] |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

# Quantization

The model was quantized by using the NNVC quantization scripts and default quantizers (those utilized for quantizing LOP3).

# Inference

Table 2 describes the loop-filter information at inference stage.

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 209716 |
|  | Total Number of Parameters (All Models) | 209716 |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 0.4MB |
|  | Multiply Accumulate (kMAC/pixel) | 16.97 |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128 / 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Experimental results

Table 3 shows the coding performance of the proposed approach (LOP3 with multiscale backbone blocks and NN-intra ON) in the RA configuration over NNVC-9.1 with LOP3 and NN-intra ON.

Table 3. Coding performance over NNVC-9.1 with LOP3 and NN intra ON in RA configuration

|  | BD-rate Over NNVC9.1 with LOP3 & NN intra ON |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -0,13 % | -0,41 % |  | -0,92 % |  | 0,06 % | -0,31 % | 0,29 % | 100 % | 121 % | 100 % |
| Class A2 | -0,12 % | -0,90 % |  | -0,42 % |  | 0,00 % | -1,08 % | 0,04 % | 100 % | 121 % | 100 % |
| Class B | -0,21 % | -0,32 % |  | -1,32 % |  | -0,12 % | -0,89 % | -0,46 % | 100 % | 120 % | 100 % |
| Class C | -0,18 % | -0,02 % |  | -0,25 % |  | 0,11 % | -0,22 % | 0,17 % | 101 % | 114 % | 100 % |
| Overall | -0,17 % | -0,37 % |  | -0,77 % |  | 0,00 % | -0,63 % | -0,05 % | 100 % | 119 % | 100 % |
| Class D | -0,23 % | -0,54 % |  | -0,07 % |  | 0,01 % | -1,96 % | 0,23 % | 99 % | 114 % | 100 % |
| Class F | -0,14 % |  | -0,69 % |  | 0,83 % | -0,08 % | -0,93 % | 1,71 % | 102 % | 103 % | 100 % |

Table 4 shows the coding performance of the proposed approach (LOP3 with multiscale backbone blocks and NN-intra ON) in the RA configuration over NNVC-9.1 with all NN tools OFF.

Table 4. Coding performance over NNVC-9.1 with all NN tools OFF in RA configuration

|  | BD-rate Over NNVC9.1-VTM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -8,93 % | -12,12 % | -13,33 % | -9,43 % | -13,66 % | -14,38 % | 125 % | 8078 % | 95 % |
| Class A2 | -7,67 % | -12,93 % | -8,42 % | -8,27 % | -13,81 % | -7,81 % | 125 % | 7881 % | 96 % |
| Class B | -7,09 % | -14,40 % | -13,22 % | -7,75 % | -16,21 % | -14,73 % | 123 % | 8400 % | 97 % |
| Class C | -6,70 % | -13,50 % | -12,11 % | -7,41 % | -14,36 % | -12,58 % | 123 % | 6893 % | 98 % |
| Overall | -7,47 % | -13,41 % | -11,99 % | -8,10 % | -14,73 % | -12,70 % | 124 % | 7806 % | 97 % |
| Class D | -6,93 % | -12,25 % | -11,20 % | -6,78 % | -13,71 % | -11,58 % | 121 % | 6240 % | 98 % |
| Class F | -3,95 % | -8,14 % | -5,30 % | -4,80 % | -10,84 % | -7,58 % | 136 % | 3162 % | 98 % |

Table 5 shows the coding performance of the proposed approach (LOP3 with multiscale backbone blocks and NN-intra ON) in the AI configuration over NNVC-9.1 with LOP3 and NN-intra ON.

Table 5. Coding performance over NNVC-9.1 with LOP3 and NN intra ON in AI configuration

|  | BD-rate Over NNVC9.1 with LOP3 & NN intra ON |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -0,13 % | 0,10 % | -0,12 % | -0,34 % | 0,22 % | 0,40 % | 101 % | 109 % | 100 % |
| Class A2 | -0,19 % | -0,85 % | -0,33 % | -0,34 % | -0,92 % | -0,22 % | 101 % | 106 % | 100 % |
| Class B | -0,16 % | -0,41 % | -0,63 % | -0,17 % | -0,64 % | -0,05 % | 103 % | 111 % | 100 % |
| Class C | -0,07 % | -0,47 % | -0,15 % | -0,14 % | -0,80 % | 0,00 % | 101 % | 111 % | 100 % |
| Overall | -0,11 % | -0,34 % | -0,28 % | -0,19 % | -0,47 % | 0,02 % | 101 % | 108 % | 100 % |
| Class D | -0,08 % | -0,52 % | -0,25 % | -0,14 % | -1,21 % | -0,21 % | 100 % | 113 % | 100 % |
| Class F | -0,12 % | -0,10 % | 0,81 % | -0,35 % | -0,41 % | 1,28 % | 100 % | 106 % | 100 % |

Table 6 shows the coding performance of the proposed approach (LOP3 with multiscale backbone blocks and NN-intra ON) in the AI configuration over NNVC-9.1 with all NN tools OFF.

Table 6. Coding performance over NNVC-9.1 with all NN tools OFF in AI configuration

|  | BD-rate Over NNVC9.1-VTM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -9,35 % | -14,01 % | -15,00 % | -9,81 % | -15,92 % | -15,96 % | 176 % | 5637 % | 96 % |
| Class A2 | -7,71 % | -14,23 % | -10,92 % | -8,59 % | -14,78 % | -10,23 % | 173 % | 4653 % | 97 % |
| Class B | -7,76 % | -14,41 % | -14,87 % | -8,20 % | -16,45 % | -16,16 % | 173 % | 4639 % | 97 % |
| Class C | -7,51 % | -13,80 % | -13,23 % | -8,46 % | -16,66 % | -15,61 % | 159 % | 3324 % | 98 % |
| Overall | -6,67 % | -11,77 % | -11,39 % | -7,23 % | -13,39 % | -12,32 % | 155 % | 2349 % | 98 % |
| Class D | -7,33 % | -12,06 % | -12,42 % | -7,75 % | -15,54 % | -15,57 % | 151 % | 3108 % | 98 % |
| Class F | -5,31 % | -9,58 % | -7,92 % | -5,88 % | -12,36 % | -10,78 % | 133 % | 3496 % | 97 % |

# Recommendation

It is recommended to include the proposed LOP3 with multiscale blocks as a test of the next EE1 round.

# Reference

- D. Liu, J. Ström, M. Damghanian, P. Wennerste, D. Rusanovskyy, Y. Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya. EE1-1.2: Joint LOP model with inputs transformed. JVET-AH0080. April 2024.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).