| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0141-v3 |
| --- | --- |

| Title: | EE2-2.4c: Combination of EE2 tests 2.2 and 2.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Döne Bugdayci Sansli Saverio Blasi Jani Lainema (Nokia) Karam Naser Ya Chen Fabrice Le Léannec Milos Radosavljević Kevin Reuzé (InterDigital) Jiaye Fu Jiaqi Zhang Siwei Ma (PKU) Ying Gao Cheng Huang(ZTE) | Email: | done.bugdayci_sansli@nokia.com saverio.blasi@nokia.com jani.lainema@nokia.com |
| Source: | Nokia, InterDigital, PKU, ZTE |  |  |

_____________________________

# Abstract

This contribution presents simulation results for the EE2 test 2.4c which is the combination of tests 2.4a, additional TIMD mode with different cost metric and 2.2, TIMD with enhanced block vectors deployment. The combined test allows deployment of block vector-based predictors both in conventional TIMD and in the newly proposed additional TIMD mode from Test 2.4. The following simulation results are reportedly found on top of ECM-13 anchor:

AI -0.10 %, -0.02 %, -0.04 %, 101.0%, 101.9%, 100.2%, 100.1%}

RA {%, %, %, %, %, %, %}

# Introduction

EE2 test 2.2 proposes the deployment of block vector-based predictors in TIMD, which may be used in place of any of the three TIMD predictors depending on the template cost [1]. The block vectors of the merge candidates of IntraTMP mode are used to create a block-vector based predictor in TIMD. This can be selected to replace one of the TIMD predictors if the block-vector template cost is smaller than the TIMD template costs. Additionally, block vector candidates generated by ARBVP process are included in the merge list of DIMD and TIMD, in the same manner as in IntraTMP.

On the other hand, EE test 2.4a proposes an additional TIMD based intra prediction mode, signaled as a sub-mode of TIMD. The TIMD process is altered compared to conventional TIMD allowing larger template sizes, different cost metric using Sum of Absolute Differences (SAD), and a different set of candidate intra modes, in order to differentiate the mode from conventional TIMD [2].

# Proposed Method

In this test, the two EE tests are combined orthogonally, i.e. all the features from both tests are integrated together. Additionally, block vector based TIMD predictor is allowed in the newly proposed TIMD mode as well, in the same manner as it is deployed in TIMD.

# Experimental Results

Simulation results are reported against ECM-13.0 anchor.

Table 1 Simulation results for Test 2.4c, All Intra configuration

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10 % | -0.04 % | 0.08 % | 101.2% | 101.0% | 100.2% | 100.1% |
| Class A2 | -0.12 % | -0.04 % | -0.02 % | 101.2% | 100.4% | 99.2% | 100.2% |
| Class B | -0.11 % | -0.03 % | -0.16 % | 101.4% | 101.9% | 101.1% | 100.3% |
| Class C | -0.09 % | 0.06 % | 0.02 % | 101.0% | 103.1% | 99.8% | 100.1% |
| Class E | -0.10 % | -0.04 % | -0.08 % | 100.0% | 102.7% | 100.1% | 100.1% |
| Overall | -0.10 % | -0.02 % | -0.04 % | 101.0% | 101.9% | 100.2% | 100.1% |
| Class D | -0.04 % | 0.00 % | -0.01 % | 101.9% | 104.1% | 100.1% | 100.0% |
| Class F | 0.01 % | 0.16 % | 0.25 % | 102.8% | 103.6% | 99.3% | 100.1% |

Table 2 Simulation results for Test 2.4c, Random Access configuration

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07 % | 0.05 % | 0.08 % | 100.8% | #NUM! | 100.4% | #NUM! |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01 % | -0.15 % | -0.09 % | 101.3% | #NUM! | 100.0% | #NUM! |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# References

- K. Naser, Y. Chen, F. Le Léannec, M. Radosavljević, K. Reuzé (InterDigital), J. Fu, J. Zhang, S. Ma (PKU), Y. Gao, C. Huang (ZTE), EE2-2.2: TIMD with enhanced block vectors deployment, JVET-AI0104, 2024/07, Sapporo

- D. Bugdayci Sansli, S. Blasi, J. Lainema (Nokia), EE2-2.4a/b: Additional TIMD mode with different cost metric, JVET-AI0113, 2024/07, Sapporo

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

PKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).