| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0165-v3 |
| --- | --- |

| Title: | EE2-4.4f: Test 4.1c from the previous EE |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang, Yue Yu, Haoping Yu, Dong Wang (OPPO) | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-4.4f. In EE2-4.4f, Test 4.1c from the previous EE is retested.

On top of ECM-13.0, simulation results are reported as follows:

AI {-0.07 % Y, 0.02 % U, -0.01 % V, 100.9 % EncT, 100.2 % DecT}

RA {-0.06 % Y, -0.09 % U, -0.07 % V, 100.5 % EncT, 99.7 % DecT}

Introduction

The multiple transform set selection (MTSS) method for intra blocks coded with LFNST/NSPT was proposed in JVET-AG0062 [1]. The proposed method was further investigated in AH EE2-4.1. AH EE2-4.1 was reported in JVET-AH0056 [2]. The proposed MTSS method allows CUs coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP to select one LFNST/NSPT transform set out of 2 candidate sets. When the current block is coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP in combination with LFNST/NSPT, one more bin is employed to indicate whether the 1st or the 2nd candidate transform set is selected.

Proposed method

Because OBIC was adopted into ECM-13.0, the MTSS method is extended to OBIC.

The derivation methods for transform sets are summarized below:

The 1st candidate transform set remains the same as the current ECM.

To derive the 2nd candidate:

For TIMD, DIMD, OBIC, if fusion is applied, the 2nd TIMD IPM is first considered;

For SGPM, the 2 IPMs that SGPM uses are first considered;

For MIP and EIP, PLANAR is first considered;

The 1st and 2nd candidate should have different transform sets or different transpose type. If the 2nd candidate is not yet decided by the above process, it is derived by the HoG of neighboring reconstructed pixels.

In EE2-4.4f, MTSS can be applied to CUs with number of pixels equal or larger than 256.

Test results

EE2-4.4f was implemented on top of the ECM-13.0 software [3] and tested based on the common test condition [4].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over 20240622_ECM13_4.4f/anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.18% | 0.13% | -0.01% | 101.3% | 100.3% | 100.1% | 100.1% |
| Class A2 | -0.08% | -0.08% | 0.00% | 101.8% | 100.4% | 100.3% | 100.0% |
| Class B | -0.05% | -0.04% | -0.05% | 100.9% | 100.1% | 101.6% | 100.1% |
| Class C | -0.01% | 0.05% | 0.06% | 99.5% | 99.6% | 100.1% | 100.0% |
| Class E | -0.05% | 0.05% | -0.02% | 101.5% | 100.6% | 99.0% | 100.0% |
| Overall | -0.07% | 0.02% | -0.01% | 100.9% | 100.2% | 100.3% | 100.0% |
| Class D | 0.01% | 0.00% | -0.14% | 101.5% | 101.6% | 100.2% | 100.0% |
| Class F | -0.05% | 0.03% | 0.10% | 99.9% | 99.7% | 99.2% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over 20240622_ECM13_4.4f/anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.17% | -0.24% | -0.29% | 100.6% | 99.2% | 100.2% | 99.8% |
| Class A2 | -0.03% | 0.05% | 0.00% | 101.0% | 99.5% | 99.9% | 99.8% |
| Class B | -0.05% | -0.09% | -0.08% | 100.3% | 99.8% | 100.2% | 100.2% |
| Class C | -0.03% | -0.10% | 0.07% | 100.3% | 100.0% | 100.5% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.09% | -0.07% | 100.5% | 99.7% | 100.2% | 100.0% |
| Class D | 0.01% | -0.17% | 0.00% | 100.1% | 100.5% | 100.0% | 100.0% |
| Class F | 0.01% | 0.25% | -0.12% |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

References

- F. Wang, Y. Yue, H. Yu, D. Wang, “Non-EE2: Multiple Transform Set Selection for LFNST/NSPT”, JVET-AG0062, January 2024

- F. Wang, Y. Yue, H. Yu, D. Wang, “EE2-4.1: Multiple Transform Set Selection for LFNST/NSPT”, JVET-AH0056, April 2024

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).