| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0169-v2 |
| --- | --- |

| Title: | Non-EE2: On sub-block merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Yue Yu Haoping Yu Dong Wang | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes to extend the usage of sub-block merge mode. The block size restriction is modified to allow more CUs applicable for sub-block merge mode. The sub-block merge mode flag is signaled depending on whether the neighboring blocks already been coded with sub-block merge mode or inter affine mode. On top of ECM-13.0, the performance of the proposed method is reported as follows:

RA {-0.09% Y, -0.09% U, -0.07% V, 101.2% EncT, 100.3% DecT}

LD {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT}

Instruction

In VVC and ECM, sub-block merge mode includes affine merge mode and SBTMVP mode. During the development of ECM, both the affine merge mode and the SBTMVP mode are enhanced. The CU size restriction of sub-block merge mode is specified as “cbWidth >= 8 && cbHeight >= 8”. ECM has released many size restrictions but the one for sub-block merge mode remains unchanged.

Proposed method

It is proposed to modify the CU size restriction of sub-block merge mode as “cbWidth * cbHeigh >= 32”. With this modification, CUs of size 4*8, 4*16, 4*32… and 8*4, 16*4, 32*4… can apply sub-block merge mode.

The usage of affine merge is highly related to the neighboring area of the current block. If current block is in an affine motion area, it has high probability to use affine merge. On the other hand, if current block is in a translational motion area, it has low probability to use affine merge.

It is proposed to use neighboring blocks’ coding information to determine whether sub-block merge mode is allowed for current block. If at least one of the spatial neighboring blocks and non-adjacent blocks is coded with sub-block merge mode or inter affine mode, sub-block merge mode is allowed for current block. Otherwise, sub-block merge mode is forbidden for current block. If sub-block merge mode is forbidden in current block, the encoder will not try sub-block merge mode and do not signal sub-block merge flag, the decoder will not parse this flag.

Simulation results

The proposed method is implemented on top of the ECM-13.0 software [1] and tested based on the common test condition [2]. The test results are summarized below.

Test 1:

In the following test, proposed aspect 1 is applied.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.06% | -0.06% |  |  |  |  |
| Class A2 | -0.08% | -0.16% | 0.01% |  |  |  |  |
| Class B | -0.07% | 0.01% | -0.05% |  |  |  |  |
| Class C | -0.18% | -0.12% | -0.07% | 103.4% | 101.6% | 99.5% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.08% | -0.07% | -0.05% |  |  |  |  |
| Class D | -0.27% | -0.19% | -0.11% | 103.3% | 101.7% | 99.8% | 100.0% |

Note that there is one point copy from anchor in Class A2.

Test 2:

In the following test, proposed aspect 2 is applied to the blocks newly enabled with sub-block merge mode by proposed aspect 1.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.18% | -0.11% | 100.5% | 100.0% | 100.2% | 99.3% |
| Class A2 | -0.10% | -0.15% | -0.06% | 101.3% | 99.9% | 100.1% | 100.1% |
| Class B | -0.06% | 0.04% | -0.14% | 101.5% | 100.2% | 99.3% | 100.0% |
| Class C | -0.16% | -0.15% | 0.04% | 101.3% | 101.0% | 99.9% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.09% | -0.09% | -0.07% | 101.2% | 100.3% | 99.8% | 99.9% |
| Class D | -0.26% | -0.11% | -0.15% | 102.0% | 101.7% | 99.9% | 100.0% |

Conclusions

This contribution proposes 2 modifications to sub-block merge mode. The proposed method brings -0.09% coding gain in RA with 1.2% EncT increase. It is recommended to investigate the proposed methods in EE2.

References

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AF2017, October 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).