| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0176-v2 |
| --- | --- |

| Title: | [EE1-2.1: On the detailed analysis of integer Transformer ILF with luma-chroma component balancing and complexity trade-off] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposed] |  |  |
| Author(s) or Contact(s): | Yun Li Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | yli30@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Based on the architecture of the HOP network, Transformer blocks were proposed in JVET-AF0158, and the inference was performed by using the Pytorch inference engine. This model has improved the coding gain significantly compared to HOP anchor. In addition, integrating HOP with transformer with SADL float/ int16 support was proposed in JVET-AH0205 and JVET-AH0206, which provides comparable performance compared to that of its respective floating-point model.

In this document, the results of integer-Transformer implementation in SADL with luma-chroma balancing are reported, the detail of the implementation and the analysis of the method are presented, and a complexity trade-off is also tested. The proposed model is based on the HOP4 architecture jointly integrated from JVET-AH0205 and JVET-AH0189, and the filter parameters are modified and constrained to the complexity and memory requirements of the HOP4 architecture. The training was conducted by using the NNVC9.1 HOP training guidelines, and SADL-int16 quantized model was used for the inference.

- Test1: Transformer model (int16) with luma/chroma rebalance:

Complexity of the model in kMAC/pixel: 468 (block-wise) with 1.39 M parameters.

The proposed filter (int16 model) with NNIntra ON provides: {-1.1%, 3.4%, 3.7%} for AI test configuration, and {-1.2%, 1.6%, 0.7%} for RA test configuration, versus the HOP anchor (NNIntra ON).

Compared to the VTM, the proposed filter provides: {-14.2%, -13.5%, -14.8%} for the AI configuration, and {-15.1%, -17.8%, -18.8%} for the RA configuration.

- Test4: Model with two Transformer blocks (int16):

Complexity of the model in kMAC/pixel: 465 (block-wise) with 1.35 M parameters.

The proposed filter (int16 model) with NNIntra ON provides: {-0.1%, -2.2%, -1.3%} for AI test configuration, and {-0.1%, -2.9%, -3.3%} for RA test configuration, versus the HOP anchor (NNIntra ON).

Compared to the VTM, the proposed filter provides: {-13.4%, -17.8%, -18.6%} for the AI configuration, and {-14.2%, -21.4%, -22.2%} for the RA configuration.

It is suggested to adopt the proposed method for HOP, and to fix the corresponding SADL operators in a later stage.

# Introduction

A unified filter architecture for the High-performance Operation Point (HOP) was defined in JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041. The HOP4 software integration, combining with Attention module (JVET-AH0205[1]) and wide activation of JVET-AH0189[2] was jointly trained by the proponents and presented during AhG11/14 telco. Complete inference results for HOP4 training are reported in JVET-AI0014.

Based on the architecture of the HOP network, Transformer blocks were introduced in JVET-AF0158 [3][4], and the inference was performed by using the Pytorch inference engine. This model has improved the coding gain significantly compared to HOP1/2/3. In addition, integrating HOP with transformer with SADL float/ int16 support was proposed in JVET-AH0205 and JVET-AH0206, which provides comparable performance compared to that of its floating-point model.

In this document, the results of integer-Transformer implementation in SADL with luma-chroma balancing are reported, the detail of the implementation and the analysis of the method are presented, and a complexity trade-off with two transformer blocks is also tested. This trade-off is with respect to the number of nonlinear layers and training time. The proposed model is based on the HOP4 architecture jointly integrated from JVET-AH0205 and JVET-AH0189, and the filter parameters are modified and constrained to the complexity and memory requirements of the HOP4 architecture.

The training was conducted by using the NNVC9.1 HOP training guidelines, and SADL-int16 quantized model was used for the inference.

Proposed method

## Overview of the method

EE1-2.1 contains for subtests, i.e., Test1: ILF with the Transformer block in every backbone residual block, and Test2: ILF with only two Transformer blocks.

Block diagram of the Test 1, full Transformer model, is shown in Figure 1. The transformer block contains an Attention network followed by a Feed Forward Network (FFN), which are shown in Fig. 2 and Fig. 3, respectively. The transformer block is integrated into the Residual Backbone Block (RBB) as illustrated in Fig. 4.

Figure 1 HOP with transformer blocks.

Figure 2 Attention block

Figure 3 Feed forward network

Figure 4 Backbone blocks with the transformer block

In this contribution, all the nonlinear layers, i.e. LayerNorm, Norm, and Softmax in Fig. 2 have been approximated with integer arithmetic operations, and they have been integrated into the SADL for inference. The approximation is an adaptation from existing techniques, for example, Newton’s method is utilized for computing the integer square root, and range reduction and polynomial approximation is applied to the calculation of exponential functions. In the proposal, these layers are quantized with 11 bits. A detailed implementation and analysis can be found in subsection 1.2 and 1.3. The luma-chroma balancing is achieved with training weight and chroma offset adjustment described in Section 2.

Test 2 and 3 are similar to Test1 but with a further reduction of the complexity. Our training validation loss indicates that Test1 has a better performance trade-off. Therefore, the results for Test2 and Test3 will not be reported in this document.

For the Test 4, only two Transformer Blocks are integrated to the 8th and the 15th HOP backbone blocks, whereas the rest of the HOP backbone blocks remains unchanged. To adjust the complexity to the constraint of HOP4, the network parameter C22 is set to 80, and the number of residual blocks to 22.

The information for the inference and the training stage is presented in Table 1 and 2 for Test1, respectively, and is illustrated in Table 3 and Table 4 for the Test4, respectively.

Table1. Network information for the inference (Full Transformer model)

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.36M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 468 (369 frame-wise) |
|  | Total Conv. Layers |  | 226 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 2.7/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training (Full Transformer model)

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | ~432h |
|  | Training data information: | DIV2K BVI TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Table3. Network information for the inference (model with two Transformer blocks)

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.4M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 464.3 (367 frame-wise) |
|  | Total Conv. Layers |  | 166 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 2.8/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 4. Network information for the training (model with two Transformer blocks)

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | ~336h |
|  | Training data information: | DIV2K BVI TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

## Implementation detail

The approximation of the non-linear operators is described in the following:

- LayerNorm

LayerNorm operator is defined by the Eq. 1, where x is a 4-D tensor (batch size, number of channels, height, width), i.e., (B, C, H, W), ε is a small value added to avoid division by zero, and the normalization is performed across the channel dimension. The  ε can be set as 1e-6 for floating point implementation and 4 for quantized representation of 11-bit.  In order to make this operator parameter free, Eq.1 can be simplified by Eq. 2 and followed by an 1x1 convolution, i.e., 1x1xCxC1 with input C and output C1 channels. The 1x1 convolution here compensates the functionality of scaling by γ and bias terms β in Eq. 1. A parameter-free LayerNorm facilitates the conversion to SADL as mentioned in Section 1.4.

y=x-E[x]Varx+ε*γ+β       (1)

y=x-E[x]Varx+ε                      (2)

The mean and variance computation of the Eq. 2 is well-defined and straightforward. However, the Square Root (sqrt) operation may not be defined identically across different computation platforms, and the integer sqrt may not be implemented either. Here, one approximation method from prior art is adapted for this system as follows.

Considering the full range of approximation of a 32-bit data precision, the sqrt is computed with Newton’ method. In order to compute the sqrt of N, it is equivalent to find the solution of x2-N=0. This can be computed iteratively with the equation xk+1=xk+nxk2, where k is initialized as zero, and x0 as a number larger than sqrt(N). A good option from [5] is to set it as  x0= 2nbt(N)/2 .  Here, nbt(N) is a function that returns the number of bits of N. The iteration stops when xk+1≥xk, and the solution is then xk, otherwise k=k+1. When N=0, the solution of zero will be returned directly without performing the iteration. In addition, the division of 2k, k≥ 0 is conducted with a right shift operation of k bits, where 2k is computed with a left shift of 1 by k bits.

It is found from our ablation study that this parameter-free Layer norm operation does not impact the filtering performance for the AI configuration.

- Norm

The normalization function of the Transformer in Figure 21 is defined in Eq. 3, where L2 Norm operator is computed as x2=k=0nxk2 , n is the number of elements., and the  ε can be set as 1e-6 for floating point implementation and 4 for quantized input of 11 bits.

y=xMAX(x2,   ε)     (3)

The computation of the sum of the square in L2 Norm can be performed by using integer arithmetic, whereas the integer sqrt operation is conducted by using the algorithm listed in the previous section.

- Softmax

The Softmax operator is illustrated in Eq. 4. x is a vector of real numbers or integer numbers (if x is quantized).

S(x)i=exi1Kexi                      (4)

In order to compute exi, it is proposed to exploit properties of the Softmax and exponential functions. For the former, due to the fact that S(x-x.max⁡())i=S(x)i, where x.max⁡() is the maximum value of x. Then exi-x.max⁡() can be computed in the negative region, where xi-x.max⁡()≤0. This not only reduces the range for approximation but also improves the numerical stability of the computation. For the latter, range reduction can be further applied on xshift_i=xi-x.max⁡() to reduce the range of the approximation region. With exshift_i=2-kexshift_i+k*ln2, where xshift_i+k*ln2 =mod (xshift_i, ln2) in the negative region, and k=-xshift_iln2. The remainder, mod (xshift_i, ln2), is within a much smaller range, in (-ln2, 0] to approximate. Therefore, we only need to compute emod(xshifti, ln2)≫k to obtain exi.

In case of x being integer-quantized, the approximation of ex would need to take into account the scaling factor of 2S (i.e., with s-bit quantization representation) for the input, such that the approximation is with respect to the dequantized signal of x2S. In this case, the computation similar to [5] is performed as follows: let sln(s)=2S*ln2, then k=-xshift_isln(s), mod (xshifti,  sln(s)) =xshifti+k*sln(s). We will need to compute emod (xshifti,   sln(s)) ≫k to obtain exi.

In this contribution, the ex function is approximated by using a high order polynomial, e.g., of second order. In particular, the function can be approximated such that  ex≈ax+b2+c, and the parameter a, b, and c may be estimated by solving the least square problem with any convex optimization approaches, e.g., gradient decent or Gauss-Newton. With a quantization of 11-bits for the input to the Softmax function, and  x=mod (xshifti,  sln(11)) after the shift and range reduction mentioned above, the polynomial can be rearranged to ex≈s1[x+bq2+cq]. The bq can be calculated and set to 2770, cq to 4024659, based on the estimated parameters presented in [5]. S1 is a scaling factor that will be cancelled out in Eq. 4. Therefore, it can be ignored in the calculation.

- Computing division and quantization

In general, a normalization process involves division in the form of y=ab. To quantize y with a N-bit quantized representation, it is computed such that y=a≪Nb. If b is already a very large number, the denominator b can be right shifted first such that y=a≪(N-N1)b≫N1, where 0≤N1≤N. In order to reduce the number of divisions, for a 32-bit computation precision, a division in some cases is computed first such that y=a*c>>(M-N), where c=1≪Mb, where M can be set to 31 or to a number as large as possible such that  a*c does not cause any overflow.

## An empirical analysis of the Newton’s method for solving square root

The Newton’s method is deterministic for square-root approximation in our case, assumed a deterministic starting point for the iteration and a deterministic integer division. An empirical study of computing the approximation of square root for the entire range of 32-bit precision verified that the results are a 100% match to that produced by using a standard library (after a floor operation). The number of iterations for 32-bit range is presented in the following:

For a more realistic range of 24-bit precision, the iterations needed for the computation is illustrated as follows:

It can be observed from the histogram that the average number of iterations is between 2 to 3 for 24-bit precision.

## SADL implementation

It was found that LayerNorm operator is only supported after ONNX 1.16, and the conversion from Pytorch to ONNX format is problematic with the SADL ONNX converter. A fix will likely involve a significant improvement from the SADL side. In addition, without converting the LayerNorm as an entire operator results in 6 to 7 other unknown operators in SADL, which will render the modelling of the quantization inconvenient.

Therefore, we have introduced a user-defined operator for LayerNorm and Norm operations, i.e. the unused DIV operator is utilized for this purpose. In this contribution, the DIV(x, 2) is defined as LayerNorm, and DIV(x, 3) as Norm. This facilitates both the implementation and quantization, and the implementation is also independent of ONNX versions.

## Complexity calculation

In the computation of nonlinear layers mentioned, when intensive addition, shift, and comparator operation are applied, each of these operations is counted as 1/30 of a multiplication. The calculation is based on the energy consumption of these operations, and it is estimated from prior research, e.g., [6]. The complexity comparison of a division is not that straight forward and is algorithm dependent. In this contribution, the complexity of a division is estimated based on a simple iterative subtraction algorithm for division. Each iteration requires a one’s complement operation, addition, and a carrier bit handling, which are estimated to be between 2 to 3 times more complex than a multiplication (note that the complexity varies drastically with different division algorithms). However, in machine learning literatures, a division is usually counted as a floating-point operation, which is the same for a multiplication.

# Simulation results

The method was integrated in the NNVC9.1 and tested following the EE1 CTC and evaluation procedure [7]. The proposed method has been trained following the EE1-0 training procedure (Stage 3 HOP training only) specified in JVET-AE0191 and updated for EE1 testing as specified in JVET-AG0041. The training is being conducted with a target of 20 epochs as specified by the guidance.

The proposed method was tested with NNVC9.1 reference software and the trained model is quantized to uniform 11-bit for the parameters with SADL, and the SADL modules for Transformer modules are added and modified for running the proposed models.

In addition, for Test1, the luma-chroma weight for the training is set to 6:1:1, whereas the chroma offset is set to 2 for the inference.

Summary of the inference results are reported below, complete results in CTC template are provided along with this document.

Test1:  Full Transformer model (int16):

Comparative results to the NNVC9.1 HOP4 filter are shown in Table 5. NN Intra was enabled for both the test and the anchor. Comparison against VTM (NNVC9.1) is shown in Table 6.

As observed in Tables 5, the proposed filter outperforms the HOP anchor with NNIntra ON by {-1.1%, 3.4%, 3.7%} for the AI configuration and {-1.2%, 1.6%, 0.7%} for the RA configuration, respectively.

Compared to VTM, shown in Table 6, the proposed method provides {-14.2%, -13.5%, -14.8%} for the AI configuration, and {-15.1%, -17.8%, -18.8%} for the RA configuration, respectively.

Table 5 Proposed method (int16 model) vs NNVC9.1 HOP 4, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.50% | 4.78% | 2.24% | -2.81% | 2.05% | -0.70% |
| Class A2 | -2.44% | 0.03% | 0.72% | -1.90% | -2.01% | -3.20% |
| Class B | -0.68% | 2.41% | 5.07% | -0.28% | 0.72% | 1.38% |
| Class C | -0.68% | 5.77% | 4.63% | -0.55% | 4.96% | 3.65% |
| Class E | -0.33% | 4.07% | 4.71% |  |  |  |
| Overall | -1.05% | 3.43% | 3.71% | -1.18% | 1.57% | 0.65% |
| Class D | -0.60% | 4.32% | 4.86% | -0.09% | 4.61% | 4.59% |
| Class F | -0.29% | 4.00% | 5.22% | -0.43% | 4.68% | 2.60% |

Table 6 Proposed method (int16 model) vs VTM, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -14.81% | -6.62% | -17.91% | -17.82% | -10.96% | -22.27% |
| Class A2 | -15.18% | -17.72% | -20.57% | -16.52% | -20.78% | -27.81% |
| Class B | -12.39% | -15.75% | -10.39% | -13.10% | -23.39% | -15.35% |
| Class C | -13.41% | -9.93% | -12.92% | -14.36% | -13.87% | -13.71% |
| Class E | -16.78% | -17.11% | -16.03% |  |  |  |
| Overall | -14.22% | -13.49% | -14.84% | -15.07% | -17.84% | -18.79% |
| Class D | -12.62% | -10.03% | -12.99% | -14.96% | -14.05% | -13.89% |
| Class F | -9.88% | -10.01% | -8.46% | -9.32% | -8.88% | -9.01% |

Test4:  Model with two Transformer blocks (int16):

Comparative results to the NNVC9.1 HOP4 filter are shown in Table 7. NN Intra was enabled for both the test and the anchor. Comparison against VTM (NNVC9.1) is shown in Table 8.

As observed in Tables 7, the proposed filter outperforms the HOP anchor with NNIntra ON by {-0.1%, -2.2%, -1.3%} for the AI configuration and {-0.1%, -2.9%, -3.3%} for the RA configuration, respectively.

Compared to VTM, shown in Table 8, the proposed method provides {-13.4%, -17.8%, -18.6%} for the AI configuration, and {-14.2%, -21.4%, -22.2%} for the RA configuration, respectively.

Table 7 Proposed method (int16 model) vs NNVC9.1 HOP4, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.10% | -2.45% | 0.14% | -0.61% | -3.76% | -3.54% |
| Class A2 | -0.14% | -3.01% | -1.85% | 0.09% | -2.59% | -3.30% |
| Class B | 0.01% | -2.05% | -0.92% | 0.15% | -3.61% | -3.41% |
| Class C | -0.03% | -1.52% | -2.01% | -0.24% | -1.56% | -2.85% |
| Class E | -0.09% | -2.29% | -1.70% |  |  |  |
| Overall | -0.06% | -2.20% | -1.27% | -0.11% | -2.89% | -3.26% |
| Class D | -0.03% | -2.12% | -2.09% | -0.23% | -2.62% | -2.82% |
| Class F | 0.17% | -0.20% | 0.31% | 0.29% | -0.99% | -1.52% |

Table 8 Proposed method (int16 model) vs VTM, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -13.58% | -13.37% | -19.33% | -16.01% | -16.24% | -24.74% |
| Class A2 | -13.18% | -19.54% | -22.33% | -14.81% | -21.08% | -27.89% |
| Class B | -11.78% | -18.67% | -15.17% | -12.73% | -26.34% | -19.49% |
| Class C | -12.85% | -15.64% | -18.04% | -14.09% | -19.13% | -19.28% |
| Class E | -16.59% | -22.02% | -20.78% |  |  |  |
| Overall | -13.35% | -17.82% | -18.63% | -14.16% | -21.35% | -22.16% |
| Class D | -12.12% | -15.29% | -18.47% | -15.08% | -20.01% | -20.35% |
| Class F | -9.46% | -13.14% | -12.46% | -8.65% | -13.42% | -12.47% |

# Conclusion

In this contribution, the results of EE1-2.1 tests, Int16 SADL implementation of HOP+Transformer with component balancing and complexity trade-off, are reported. Training weight of 6:1:1 and chroma offset of 2 were utilized to balance the luma-chroma components for the full Transformer model, and the performance of the filtering with only 2 Transformer blocks was also tested.

For the full transformer model (Test1), compared to NNVC-9.1 HOP anchor (NNIntra ON), the filter (int16 model) provides: AI: {-1.1%, 3.4%, 3.7%} and RA: {-1.2%, 1.6%, 0.7%}. Compared to VTM, it provides: AI: {-14.2%, -13.5%, -14.8%} and RA: {-15.1%, -17.8%, -18.8%}.

For the model with two Transformer blocks (Test4), compared to NNVC-9.1 HOP anchor (NNIntra ON), the filter (int16 model) provides: AI: {-0.1%, -2.2%, -1.3%} and RA: {-0.1%, -2.9%, -3.3%}. Compared to VTM, it provides: AI: {-13.4%, -17.8%, -18.6%} and RA: {-14.2%, -21.4%, -22.2%}.

It is suggested to adopt the proposed method (Test1: full Transformer model) for HOP, and fix the corresponding SADL operators in a later stage.

# References

- EE1-2.3 Integer implementation of HOP In-loop filter with Transformer blocks and Attention blocks, JVET-AH0205, Y. Li, D. Rusanovskyy, M. Karczewicz.

- EE1-2.2: Wide activation HOP model, JVET-0189, Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang.

- EE1-1.5 HOP In-loop filter with transformer blocks, JVET-AF0158, Y. Li, D. Rusanovskyy, T. Ryder, S. Eadie, M. Karczewicz.

- EE1-1.5 Report on implementation of HOP In-loop filter with Transformer blocks, JVET-AG0162, Y. Li, D. Rusanovskyy, M. Karczewicz.

- S. Kim, A. Gholami, Z. Yao, M. W. Mahoney, K. Keutzer, “I-BERT: Integer-only BERT Quantization”, arXiv:2101.01321v3, 2021

- Y.H. Wang, M.Q. Huang, K. Han, H.T. Chen, W. Zhang, C.J. Xu, and D.C. Tao. Addernet and its minimalist hardware design for energy-efficient artificial intelligence. arXiv preprint rXiv:2101.10015, 2021

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).