| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0182-v1 |
| --- | --- |

| Title: | AHG9: Bitdepth range information SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a Bitdepth range information SEI message to enable coding of high bit-depth content using Main profile by allocating the bits from an original picture into one or more ranges that are coded using separate picture layers, subpictures, or constituent rectangles. The proposed SEI message is particularly useful for coding of depth content, which can benefit from higher than 10 bits of bit-depth.

The Bitdepth range information SEI message can also be used to address bit-depth truncation use cases.

# Introduction

Depth content is typically represented in an inverse distance format, using Znear and Zfar parameters as described in the depth representation info SEI message. For content representing outdoor scenes, distances can be very large. To properly signal distances representing both close and distant objects, using only 10-bits is insufficient because of low precision for close objects,  which can lead to objectionable visual artifacts when using depth for reprojection. While HEVC and VVC profiles exist for coding higher bit-depths, encoder and decoders implementing those profiles are not widely available for easy integration into products.

An SEI message is proposed to enable allocating bits of high-bit depth content into bit ranges, with each range coded as a layer picture, subpicture, or constituent rectangle using an 8-bit or 10-bit profile of a video codec, such as HEVC Main, HEVC Main 10, or VVC Main profile. The semantics define a process for reconstructing a target picture with the same bit-depth as the original picture.

For each bit range, syntax elements signal the position of the first bit of that range in the original content, the number of bits allocated to that range, and the position of the first bit of the range in the coded picture/subpicture/constituent rectangle.

If coded pictures are used, each range is coded in a different layer, e.g. with a different value of nuh_layer_id. If subpictures or constituent rectangles are used, multiple ranges can be coded in different regions of single layer pictures.

The target use case uses lossy rather than lossless coding. Lossy coding leading to compression errors in the most significant bits from the original picture is a much more significant problem compression errors in the least significant bits. To reduce the likelihood and mount of compression errors for the most significant bits (MSBs), they can be placed in a higher bit position in the coded picture by left shifting the allocated bits.

With the proposed SEI message, an encoder has flexibility to select the number of bits and bit position offset in the coded picture(s) and can select those parameters to reduce the amount and likelihood of loss in the higher order bit range(s) relative to the low order bit range(s).  Placing the MSBs in a higher bit position in the coded picture reduces the amount of compression error in the significant bits. The encoder can also choose to reduce the QP.  Although shifting the significant bits to a higher bit position and reducing the QP are conceptually very similar, it can impact coding, as has been observed for coding 8-bit content as 10-bits because of the increase in internal bit precision. Encoder implementations treat content differently depending on which bit positions are occupied with significant data. Additionally, some encoder implementations do not enable assigning different QP values to different regions of the same picture, so when multiple ranges are included within separate regions within the same picture, it is useful to have a different effective QP value for the different ranges.

Depth content is typically flat and can be coded much more efficiently than texture video. If a use case requires guarantees that there will be no coding errors in the MSBs, the encoder can use trans quant bypass for the picture/subpicture/constituent rectangle containing the MSBs.

The proposed SEI message can also support bit-depth truncation in which bits are discarded from the original video, either using a single range or using multiple ranges.

The use case and target bitrate of the application will influence the parameter choices by the encoder, such as how many ranges to use, how many bits to allocate to each range, and the bit positions in which to place the allocated bits.

Several JCTVC contributions have proposed related technology for coding 16-bit content using 8-bit video codecs, including JCTVC-O0090, JCTVC-P0173, and JCTVC-P0162.

It is recommended to adopt the Bitdepth range information SEI to the VSEI TuC.

## High bit-depth content example for 16-bit depth maps

An example usage of the SEI message is provided for coding depth maps with 16-bit source using VVC Main profile with 10-bit bit depth.

The MIV common test conditions (WG04N0372) sequences include texture and depth for multiple views.  10-bit depth is used when coding content, in order to be able to use VVC Main profile. However, some of the MIV CTC test content has depth content available in 16-bit format.

The Chess sequence, as described in M50787, is a synthetic sequence which exercises a large depth range. A version with 16-bit depth is available at https://content.mpeg.expert/data/MPEG-I/Part12-ImmersiveVideo/test_material/m50787%20Chess/

View 5 is used as a test sequence, using the proposed SEI message. The depth map of frame 75 is shown in Figure 1, as visualized using YUView with Scale 12.

Figure 1. 16-bit Chess sequence Depth View 5 Frame 75, scaled by factor 12

As an alternative to using the proposed SEI message, to code the sequence with bit depth of 10 bits, the 6 least-significant bits can be truncated. A cropped region of the image is shown below, with the truncated version on the left, and the original version on the right. Bit truncation of the depth leads to contours which have a negative impact on quality when the depth is used for rendering of novel views.

Figure 2. Zoomed section, bit truncation to 10 bits on left, original 16 bits on right

Using the proposed SEI message, two ranges are defined, with the 6 MSBs allocated to range 0, and the 10 LSBs allocated to range 1, as illustrated in Figure 3. The MSBs allocated to range 0 are shown on the left and the LSBs allocated to range 1 are shown on the right.

Figure 3. Range 0 (MSBs) on left, Range 1 (LSBs) on right

Figure 4(a) shows the allocation of bits to ranges. 6 bits are allocated to Range 0, taken from the most signficant bits of the original content, and the remaining 10 least significant bits are allocated to Range 1.

Figure 4(b) shows how those 6 bits are placed at the highest order bit position in the 10-bit picture to be encoded, with zeros filled in the other bit positions, in order to maintain the highest accuracy after compression.

Figure 4(c) shows how the 10 least significant bits of the original content are allocated to Range 1.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Range 0 |  |  |  |  |  | Range 1 |  |  |  |  |  |  |  |  |  |

Figure 4(a). Example allocation of bits to ranges

| 0 | 1 | 2 | 3 | 4 | 5 | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 4(b). Coded bit positions in Range 0

| 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 4(c). Coded bit positions in Range 1

For this example, the two ranges were coded as pictures in separate layers, although the SEI message also supports compositing the two ranges within a single picture using subpictures or constituent rectangles.

Range 0, containing the 6 MSBs, was encoded using VVenc with QP=12, and had very few errors upon reconstructing the MSBs.  It is not straightforward to know exactly what QP must be used to guarantee lack of coding errors, but for use cases requiring no MSB errors, trans quant bypass may be used.

The QP for range 1, containing the LSBs, can be set to any value, according to the use case.

Table 1 shows coding results using QP=12 for range 0 and QP=27 for range 1, over the 300 frame sequence.  The PSNR represents the 16-bit PSNR calculation for the reconstructed 16-bit depth.

Table 2 shows PSNR data for the alternative with bit truncation from 16-bits to 10-bits, where the PSNR is calculated for the reconstructed 16-bit depth. Results are  shown for the uncompressed case, where the impact of bit truncation is measured, as well as for an example QP=12. The bit truncation operation itself significantly reduces PSNR vs. what can be achieved with the proposed SEI message, and places an upper limit on the compressed quality. Compression with QP=12 led to little reduction in PSNR.

Use of the proposed SEI message with VVenc encoding for Main profile at these QP values achieves 21.4 dB higher PSNR profile than the upper limit of what can be achieved when bit truncation is used.

| Range i | QP | Bitrate (kbps) | PSNR |
| --- | --- | --- | --- |
| 0 | 12 | 244.9728 |  |
| 1 | 27 | 407.6224 |  |
| Total | - | 652.5952 | 92.07 |

Table 1. VVenc coding results using the proposed SEI message

| Test condition | Bitrate (kbps) | PSNR |
| --- | --- | --- |
| Uncompressed |  | 70.67 |
| QP = 12 | 298.9328 | 70.52 |

Table 2. VVenc coding using alternative bit truncation

## Bit-truncation example

An example usage of the SEI message is provided for bit truncation of 10-bit input content coding to 4 bits, coded with an 8-bit profile.

Figure 5(a) shows the allocation of bits to ranges. 4 bits are allocated to Range 0, taken from the high order bits of the original content.

Figure 5(b) shows how those 4 bits are placed at the lowest order bit position in the 10-bit picture to be encoded.

The lowest order 6 bits of the original content are discarded.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Range 0 |  |  |  | Discard |  |  |  |  |  |

Figure 5(a). Example allocation of bits to range

| - | - | - | - | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 5(b). Coded bit positions in Range 0

# Proposal

## Proposed Features

The Bitdepth range information SEI message describes the contents of one or more bit-depth ranges, by signalling the following for each range:

- Number of bits allocated to the range

- Offset position for the significant bits in the coded picture

Syntax also identifies the location of where the ranges are coded in the picture(s) of the access unit. Each range may be coded as a complete picture, or may be coded in a region of a picture, using subpicture or constituent rectangles to identify the region. A flag is used to indicate that picture partitioning is used, and if that flag is set, a subpic or CR ID is signalled per range.

A flag is used to indicate whether multiple layers are used to code the ranges. If the flag is set, the NUH Layer ID values is signalled per range. If the flag is not set, the SEI message applies to the current layer.

The bit depth of the original source content is also signalled.

Semantics indicate the encoder pre-processing process and describe a decoder post-processing process to reconstruct a target picture of the same bit-depth as the encoder source picture.  Rounding offsets and clipping operations are used in the defined processes, in order to be robust to compression errors.

## VSEI Syntax

8.X.1Bitdepth range information SEI message syntax

| bitdepth_range_info( payloadSize ) { | Descriptor |
| --- | --- |
| bri_orig_bit_depth_minus1 | u(4) |
| bri_num_ranges_minus1 | u(2) |
| if ( bri_num_ranges_minus1 > 0 ) { |  |
| bri_nuh_layer_id_present_flag | u(1) |
| bri_pic_partition_flag | u(1) |
| if( bri_pic_partition_flag ) { |  |
| bri_partition_type_flag | u(1) |
| bri_partition_id_len_minus1 | u(4) |
| } |  |
| } |  |
| for( i = 0; i <= bri_num_ranges_minus1; i++ ) { |  |
| if ( bri_nuh_layer_id_present_flag ) |  |
| bri_nuh_layer_id[ i ] | u(6) |
| if ( bri_pic_partition_flag ) |  |
| bri_partition_id[ i ] | u(v) |
| bri_num_bits_in_range_minus1[ i ] | u(4) |
| bri_bit_offset[ i ] | u(4) |
| } |  |
| } |  |

## VSEI Semantics

8.X.2 Bitdepth range information SEI message semantics

The Bitdepth range information (BRI) SEI message enables reconstruction of a target picture of the same bit depth as an original source picture by inverting operations performed by the encoder to allocate bit ranges of the original source picture to one or more ranges coded in separate layer pictures, subpictures, or constituent rectangles.

Use of this SEI message requires the definition of the following variables, where i is the layer identifier of a layer that may be present in the current CVS:

–	A chroma format indicator, denoted herein by ChromaFormatIdc[ i ], as described in clause 7.3.

–	A bit depth for the samples, denoted herein by BitDepth[ i ].

–	An array of picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples[ i ] and PicHeightInLumaSamples[ i ], respectively.

–	An array of subpicture counts, denoted by NumSubpics[ i ].

–	Arrays of the width and height of the subpictures, denoted herein by SubPicWidth[ i ][ j ] and SubPicHeight[ i ][ j ] respectively, where j is the subpicture index in 0 .. NumSubpics[ i ] – 1.

bri_orig_bit_depth_minus1 plus 1 specifies the bit depth of the original picture.

The variable origBitDepth is set equal to bri_orig_bit_depth_minus1 + 1.

bri_num_ranges_minus1 + 1 specifies the number of ranges used for allocating bits of the original picture.

The variable briNumRanges is set equal to bri_num_ranges_minus1 + 1.

bri_nuh_layer_id_present_flag equal to 1 specifies that the bri_nuh_layer_id[ i ] syntax element is present in the SEI message. bri_nuh_layer_id_present_flag equal to 0 specifies that the bri_nuh_layer_id[ i ] syntax element is not present in the SEI message.

bri_pic_partition_flag equal to 1 specifies that the bri_partition_id[ i ] syntax element is present. bri_pic_partition_flag equal to 0 specifies that the bri_partition_id[ i ] syntax element is not present.

bri_partition_type_flag equal to 1 specifies that each range is coded as a constituent rectangle. bri_partition_type_flag equal to 0 specifies that each range is coded as a subpicture.

When bri_partition_type_flag equal to 1, it is a requirement of bitstream conformance that there is a constituent rectangles SEI message preceding the BRI SEI message in decoding order in the current PU.

bri_partition_id_len_minus1 + 1 specifies the length of the bri_rect_id[ i ] and bri_subpic_id[ i ] syntax elements.

bri_nuh_layer_id[ i ], when present, specifies the layer identifier of the texture component of the i-th display overlay. When not present, the value of bri_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the BDR SEI message.

If this SEI message is present in any layer in the current AU, it is a requirement of bitstream conformance that a BRI SEI message with the same payload is present in the layer with layer identifier bri_nuh_layer_id[ 0 ].

bri_partition_id[ i ], when present and bri_partition_type_flag equal to 1, specifies the cr_rect_id[ j ] of the texture component of the i-th display overlay. bri_partition_id[ i ], when present and bri_partition_type_flag equal to 0, specifies the subpicture index of the texture component of the i-th display overlay.  When not present, the value of bri_partition_id[ i ] is inferred to be equal to 0.

When bri_partition_type_flag equal to 1, bri_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ bri_nuh_layer_id[ i ] ] – 1.  When bri_partition_type_flag equal to 0, bri_partition_id[ i ] shall be in the range of 0 to NumSubpics[ bri_nuh_layer_id[ i ] ] – 1, inclusive.

bri_num_bits_in_range_minus1[ i ] + 1 specifies the number of bits in the i-th range. bri_num_bits_in_range_minus1[ i ] shall be in the range of 0 to BitDepth[ i ] – 1, inclusive.

The variable numBits[ i ] is set equal to bri_num_bits_in_range_minus1[ i ] + 1.

bri_bit_offset[ i ] specifies a bit shifting parameter used to derive the target picture. bri_bit_offset[ i ] shall be in the range of 0 to BitDepth[ i ] – numBits[ i ].

If bri_pic_partition_flag is equal to 0, the following applies:

–	CodedWidth[ i ] is set equal to PicWidthInLumaSamples[ bri_nuh_layer_id[ i ] ] ].

–	CodedHeight[ i ] is set equal to PicWidthInLumaSamples[ bri_nuh_layer_id[ i ] ].

–	CodedRange[ i ] is set equal to the cropped decoded picture from the layer with layer identifier bri_nuh_layer_id[ i ] in the AU

Otherwise, if bri_partition_type_flag is equal to 0, the following applies:

–	CodedWidth[ i ] is set equal to SubPicWidth[ bri_nuh_layer_id[ i ] ][ bri_partition_id[ i ] ].

–	CodedHeight[ i ] is set equal to SubPicHeight[ bri_nuh_layer_id[ i ] ][ bri_partition_id[ i ] ].

–	CodedRange[ i ] is set equal to the subpicture with subpicture index bri_partition_id[ i ] from the layer with layer identifier bri_nuh_layer_id[ i ].

Otherwise (bri_partition_type_flag is equal to 1), the following applies:

–	CodedWidth[ i ] is set equal to CrRectWidth[ bri_nuh_layer_id[ i ] ][ bri_rect_id[ i ] ].

–	CodedHeight[ i ] is set equal to CrRectHeight[ bri_nuh_layer_id[ i ] ][ bri_rect_id[ i ] ].

–	CodedRange[ i ] is set equal to the constituent rectangle with cr_rect_id[ j ] equal to bri_partition_id[ i ] from the layer with layer identifier bri_nuh_layer_id[ i ].

It is a requirement of bitstream conformance that CodedWidth[ i ] and CodedHeight[ i ], respectively, have equal values for all i in 0 .. bri_num_ranges – 1.

The samples SampleToCode[ i ] of CodedRange[ i ], for i in 0 .. briNumRanges, are indicated to have been pre-processed prior to encoding from an original source picture of resolution CodedWidth[ 0 ] x CodedHeight[ 0 ] with chroma format ChromaFormatIdc and bit depth origBitDepth, according to the process of subclause 8.X.2.1.

Each sample, TargetSample, of a target picture of resolution CodedWidth[ 0 ] x CodedHeight[ 0 ] with chroma format ChromaFormatIdc and bit depth origBitDepth, may be reconstructed using the Target sample reconstruction process described in subclause 8.X.2.2, for each sample, decodedSample[ i ] of CodedRange[ i ] for i in 0 .. briNumRanges.

8.X.2.1 Sample pre-process prior to encoding

SampleToCode[ i ] for i in 0 .. briNumRanges is derived as follows:

currOrigBitPos = 0

currSample = inputSample

for (i = 0; i < briNumRanges; i++) {

mask = ( 1 << numBits[ i ] ) – 1

k = origBitDepth – currOrigBitPos – numBits[ i ]

shiftedInput =  inputSample  >> k

SampleToCode[ i ] = ( shiftedInput & mask ) << bri_bit_offset[ i ]

currOrigBitPos += numBits[ i ]

}

8.X.2.2 Target sample reconstruction process

TargetSample is derived as follows:

currTargetBitPos = 0

TargetSample = 0

for (i = 0; i < briNumRanges; i++) {

k = origBitDepth – currTargetBitPos – numBits[ i ]

maxVal = (1 << ( numBits[ i ] + bri_bit_offset[ i ] ) ) − 1

if (bri_bit_offset[ i ] > 0)

decRoundingOffset = 1<< ( bri_bit_offset[ i ] − 1 )

else

decRoundingOffset = 0

adjustedDecodedSample =  Clip3( 0, maxVal, decodedSample[ i ] + decRoundingOffset)

shiftedDecodedSample = adjustedDecodedSample >> bri_bit_offset[ i ]

TargetSample +=  shiftedDecodedSample << k

currTargetBitPos += numBits[ i ]

}

## VVC Semantics

For purposes of interpretation of the constituent rectangles SEI message, the following variables are specified:

–	ChromaFormatIdc[ i ] is set equal to sps_chroma_format_idc of the picture with nuh_layer_id equal to i.

–	BitDepth[ i ] is set equal to BitDepth of the picture with nuh_layer_id equal to i.

–	PicWidthInLumaSamples[ i ] and PicHeightInLumaSamples[ i ] are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively, of the picture with nuh_layer_id equal to i.

–	NumSubpics[ i ] is set equal to sps_num_subpics_minus1 + 1 of the picture with nuh_layer_id equal to i.

–	SubPicWidth[ i ][ j ] is set equal to (sps_subpic_width_minus1[ j ] + 1 ) * CtbSizeY – 1 of the picture with nuh_layer_id equal to i.

–	SubPicHeight[ i ][ j ] is set equal to (sps_subpic_height_minus1[ j ] + 1 ) * CtbSizeY – 1 of the picture with nuh_layer_id equal to i.

# Mapping examples to syntax

Table 2 shows parameters for the example in section 1.1, in which 16-bit content is split into two ranges. Range 0 contains the 6 MSBs, which are placed in the high order bit position, e.g.. shifted left by 4 bits. Range 1 contains the 10 LSBs, which are not shifted.

| Range i | numBits[ i ] | bri_bit_offset[ i ] |
| --- | --- | --- |
| 0 | 6 | 4 |
| 1 | 10 | 0 |

Table 2. Range Parameters for example in section 1.1

Table 3 show parameters for the bit truncation example in section 1.2, where 4 bits of 10 bit content is include in range 0, and the remaining 6 bits are discarded.

| Range i | numBits[ i ] | bri_bit_offset[ i ] |
| --- | --- | --- |
| 0 | 4 | 0 |

Table 3. Range Parameters for example in section 1.2

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).