| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, Japan, 12–19 July 2024 | Document: JVET-AI0184-v5 |
| --- | --- |

| Title: | AHG9: Some comments and editorial changes on the GFV and GFVE SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Kaifa Yang2, Yue Li2, Yiling Xu2, Jie Chen3 1Bytedance Inc., US 2Shanghai Jiao Tong University, China 3Alibaba, China | Email: | yekui.wang@bytedance.com sekiroyyy@sjtu.edu.cn yueli983@sjtu.edu.cn yl.xu@sjtu.edu.cn jiechen.cj@alibaba-inc.com |
| Source: | Bytedance Inc., Shanghai Jiao Tong University, Alibaba |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the GFV and GFVE SEI messages:

- Some asserted purely editorial changes

- Specifying the missing value ranges for the following ue(v)-coded syntax elements: gfv_coordinate_precision_factor_minus1, gfv_coordinate_z_max_value_minus1, gfv_matrix_element_precision_factor_minus1, gfv_num_matrices_info[ i ], gfv_matrix_width_minus1[ i ], gfv_matrix_height_minus1[ i ], gfv_num_matrices_minus1[ i ], gfv_matrix_element_int[ i ][ j ][ k ][ m ], gfve_matrix_element_precision_factor, gfve_matrix_height_minus1[ i ], gfve_matrix_width_minus1[ i ], and gfve_matrix_element[ i ][ j ][ k ]

- Specifying the missing decoder handling of reserved values for the syntax element gfv_matrix_type_idx[ i ]

The above asserted purely editorial changes are included in the 1st attachment with "editorial" in the filename, and the other changes are included in the 2nd attachment with "technical" in the filename.

Furthermore, this contribution includes discussions on the following topics:

- The definition of the terms "base picture" and "driving picture"

- An "empty" GFV SEI message

In v2 to v4 of this contribution, a document template issue and some typos have been fixed, some more details for the second discussion have been added, and some more editorial changes have been added to the 1st attachment (for editorial changes) under the user IDs "Yue Li (yue0)" and "Ye-Kui Wang (v4)".

In v5 of this contribution, the following changes have been made to the 1st attachment with "editorial" in the filename:

- gfv_coordinate_z_max_value_minus1: this value should be aligned with picture width and height, and thus the proposed upper limit of 232 − 2 seems too large. Therefore it is suggested to change the upper limit to 216 − 1, which corresponds to 64K.

- gfve_matrix_height_minus1[ i ]/gfve_matrix_width_minus1[ i ]: change the proposed upper limit of 28 − 1 to 210 − 1, to align with gfv_matrix_width_minus1[ i ] and gfv_matrix_height_minus1[ i ].

- gfve_num_matrices_minus1: for this one the value range is already specified in the TuC with the upper limit being as 216 − 1, and it is suggested to change the upper limit to be 210 − 1, to align with the proposed value range for gfv_num_matrices_minus1[ i ].

# A discussion on the terms of "base picture" and "driving picture"

The terms of "base picture" and "driving picture" are used in the semantics without formal definitions. Readers could try to understand the terms from the syntax table and some NOTEs. For example, based on the comments in the syntax table: if gfv_base_pic_flag is equal to 1, the current picture is a base picture, otherwise the picture is a driving picture.

And we have the following NOTEs:

NOTE 1 – Facial parameters could be determined from source pictures prior to encoding. Such source pictures may be referred to as driving pictures.

NOTE 2 – Previously decoded output pictures input to GenerativeNN( ) may be a base picture (a decoded output picture that provides the reference texture from which the face pictures may be generated) and, optionally, a picture that can be fused by GenerativeNN( ) to improve background texture and facial details. When the current picture is not a base picture, the GFV SEI message may be used to generate a face picture based on the previously decoded base picture, the facial parameters conveyed by the GFV SEI message, and, optionally, the current decoded picture for fusion pupose.

Basically, the NOTEs say that base pictures are decoded output pictures that provides the reference texture from which the face pictures may be generated, and driving pictures are pictures from which the facial parameters are determined.

However, what is said in the NOTEs on base pictures also applies to driving pictures when used for the fusion purpose, and what is said in the NOTEs on driving pictures also applies to base pictures as base pictures are also used to determine the facial parameters, specifically for inputBaseKeyPoint and inputBaseMatrix, which are always included in the inputs to the GenerativeNN.

From our observation, it seems that the real distinction between base and driving pictures is whether the corresponding output is the cropped output picture or a generated picture.

We therefore suggest that 1) it would be good to formally define the terms of "base picture" and "driving picture" in the semantics of the GFV SEI message; and 2) it might be good to define them based on the distinction of whether the corresponding output is the cropped output picture or a generated picture.

# A discussion on an "empty" GFV SEI message

When a GFV SEI message is "empty", i.e., the elements of the key points and the elements of the matrices are all equal to 0, what does the GenerativeNN do?

One example of such a case occurs when both gfv_coordinate_present_flag and gfv_matrix_present_flag are equal to 0.

If nothing meaningful is done in this case, should such a case be disallowed?

If it is desirable to disallow such as case, then either the case could be prohibited by disallowing both flags to be equal to 0 at the same time, or through conditioning the presence of gfv_matrix_present_flag based on "if( gfv_coordinate_present_flag )", and infer the value of gfv_matrix_present_flag to be equal to 1 when gfv_coordinate_present_flag is equal to 0. Probably adding the syntax condition is better, because that saves one bit when gfv_coordinate_present_flag is equal to 0, and signalling overhead saving for this SEI message is particularly important as very low bit rate application is an important (if not the most important) target application for this SEI message.

# Patent rights declaration(s)

Bytedance Inc., Shanghai Jiao Tong University, and Alibaba may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).