| Joint Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0188-v3 |
| --- | --- |

| Title: | AhG11: Block based QP information in NN loop filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Franck Galpin, Tangi Poirier Thierry Dumas Philippe Bordes | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of the impact of using block based QP information in NNVC loop filter. In particular, the impact of using the AQ mode in NNVC is reported.

# Introduction

In NNVC, several loop filters and post-filters use the QP slice information. However, in real encoding, adaptive QP is often used to control the quality inside the slice and allow an increase of the subjective quality. In this contribution, we study the impact of having a QP per block instead of slice.

# Simulation

We tested the block based QP information vs the slice based QP information when using AQ (adaptive QP) encoding in NNVC.

In anchor, the AQ mode is activated in NNVC 9.1 default configuration. It leads to different QP per block.

In the test, the same AQ mode is used but the QP information fetch to the LOP model is based on a block-based information.

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM | EncT | DecT CPU |  |
| Class A1 | 0.74% |  | 0.52% |  | 0.03% |  | 0.96% |  | 0.32% |  | 0.20% | 101% | #NUM! |  |
| Class A2 | 0.36% |  | 0.17% |  | 0.17% |  | 0.54% |  | 0.23% |  | 0.10% | 105% | 91% |  |
| Class B | 0.26% |  | 0.35% |  | 0.18% |  | 0.40% |  | 0.32% |  | 0.31% | 96% | 92% |  |
| Class C | 0.15% |  | 0.29% |  | -0.05% |  | 0.15% |  | 0.31% |  | -0.10% | 98% | 92% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.35% |  | 0.33% |  | 0.09% |  | 0.47% |  | 0.30% |  | 0.14% | 99% | #NUM! |  |
| Class D | 0.14% |  | 0.46% |  | 0.24% |  | 0.07% |  | 0.21% |  | 0.31% | 100% | 91% |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM | EncT | DecT CPU |  |
| Class A1 | 0.05% |  | 0.07% |  | 0.07% |  | 0.01% |  | 0.02% |  | 0.01% | 97% | #DIV/0! |  |
| Class A2 | 0.02% |  | 0.02% |  | 0.05% |  | 0.01% |  | -0.02% |  | 0.03% | 99% | #DIV/0! |  |
| Class B | 0.03% |  | 0.03% |  | 0.07% |  | 0.02% |  | 0.01% |  | 0.01% | 99% | #DIV/0! |  |
| Class C | 0.06% |  | 0.05% |  | -0.01% |  | 0.02% |  | 0.00% |  | -0.02% | 100% | #DIV/0! |  |
| Class E | 0.06% |  | 0.15% |  | 0.13% |  | 0.03% |  | 0.03% |  | 0.03% | 100% | #DIV/0! |  |
| Overall | 0.05% |  | 0.06% |  | 0.06% |  | 0.02% |  | 0.01% |  | 0.01% | 99% | #DIV/0! |  |
| Class D | 0.09% |  | 0.08% |  | 0.15% |  | 0.01% |  | -0.02% |  | 0.00% | 99% | #DIV/0! |  |
| Class F | 0.02% |  | 0.02% |  | 0.03% |  | 0.00% |  | 0.03% |  | -0.01% | 98% | #DIV/0! |  |

Results show that the QP based information give loss in bdrate compared to the slice-based information. It is not surprising since during the training, the model never saw a non uniform QP channel.

It is proposed to test a training where AQ mode is activated and study the impact on the performance, both in CTC, when the QP is uniform, and in AQ mode, where the QP is non uniform. In practice, if the block based QP performance is improved, current CTC for training can remain unchanged since the slice QP and block QP would be the same. However, for a future model, block based QP training would be necessary to take into account real life encoding strategy.

# Conclusion

It is proposed to study the impact of block based QP in NNVC in next EE round, especially by adapting the training dataset of HOP and study the impact on the performance when using Adaptive QP encoding.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).