| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0209-v2 |
| --- | --- |

| Title: | EE2-6.2: Context offset for gtX flags for transform coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pavel Nikitin, Muhammed Coban, Marta Karczewicz ,Vadim Seregin, | Email: | pnikitin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of EE2-6.2. It proposes to change the calculation of offset for context modelling for gtX flags:

AI: -0.01% (Y), 0.02% (U), -0.05% (V), 100% (EncT), 100% (DecT), 100% (EncVmPeak), 100% (DecVmPeak)

# Introduction

In [3] by changing the binarization of the coefficient level for residual coding the modification of the offset calculation for context modelling was overlooked. In ECM13 the context modelling for abs_gtX_flags is done as follows:

ctxIdGtx = offset + (d == 0 ? (Luma ? 21 : 7) : Luma ? d < 3 ? 14 : (d < 10 ? 7 : 0) : 0)

offset = min( sumAbs – numSig, 6 ).

# Proposal

In this contribution it is proposed to fix the calculation of the offset used for context modelling for coding gtX flags as follows:

offset = (sumAbs > 16) ? 6 : (sumAbs > 8) ? 5 : (sumAbs > 4) ? 4 : sumAbs

# Results

Test 6.2a Context offset for gtX flags for transform coefficients

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.05% | 0.08% | 100.0% | 100.1% | 100.9% | 99.8% |
| Class A2 | -0.05% | 0.00% | -0.03% | 100.1% | 100.0% | 100.0% | 99.8% |
| Class B | -0.01% | -0.04% | -0.07% | 100.1% | 100.1% | 101.0% | 99.6% |
| Class C | 0.01% | 0.01% | -0.03% | 100.1% | 100.0% | 99.9% | 99.8% |
| Class E | 0.01% | 0.12% | -0.04% | 100.0% | 100.1% | 100.0% | 99.9% |
| Overall | -0.01% | 0.00% | -0.03% | 100.1% | 100.1% | 100.4% | 99.7% |
| Class D | 0.01% | 0.16% | -0.11% | 100.0% | 100.0% | 99.8% | 99.7% |
| Class F | -0.06% | -0.08% | -0.12% | 100.0% | 100.1% | 99.7% | 99.7% |

Test 6.2b Context offset for gtX flags for transform coefficients + Test 6.1a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | 0.04% | -0.03% | 100.2% | 100.0% | 100.5% | 99.8% |
| Class A2 | -0.05% | -0.07% | -0.04% | 100.2% | 99.9% | 101.0% | 99.9% |
| Class B | -0.01% | -0.11% | -0.19% | 100.2% | 99.9% | 99.3% | 99.6% |
| Class C | -0.02% | -0.01% | -0.06% | 100.2% | 99.9% | 100.0% | 99.8% |
| Class E | -0.04% | 0.03% | 0.16% | 100.2% | 100.0% | 100.0% | 99.9% |
| Overall | -0.03% | -0.03% | -0.05% | 100.2% | 99.9% | 100.1% | 99.8% |
| Class D | -0.02% | 0.28% | -0.09% | 100.2% | 99.9% | 99.8% | 99.8% |
| Class F | -0.04% | -0.02% | 0.10% | 100.1% | 99.9% | 99.7% | 99.7% |

# References

[1] ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

[3] 	P. Nikitin, M. Coban, M. Karczewicz, P. Garus, V. Seregin (Qualcomm), “EE2-3.2: Transform coefficient coding”, JVET-AG0100, Jan. 2024.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).