| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0250 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0197 (EE2-3.9: Affine candidates derived from temporal collocated pictures) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Na Zhang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | zhangna.cynthia@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results of JVET-AI0197 which is about affine candidates derived from temporal collocated pictures. It is confirmed that the available crosscheck results match with those provided in JVET-AI0197. The coding time is not accurate because the cluster is busy.

# Introduction.

In Test 3.9, it is proposed to add the affine candidates derived from temporal collocated pictures into current affine merge candidate list. The derived new affine candidates are inserted into the existing affine merge list and reordered together with the other affine merge candidates through the ARMC. The number of the final output affine merge candidates after the ARMC is kept unchanged.

# Experimental Results

JVET-AI0197 [1] was implemented on top of ECM-13.0 [2] and was evaluated following the common test conditions [3]. The crosscheck results are summarized in Table 1.

Table 1: Results of Test 3.9

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.08% | -0.05% | 104.7% | 102.9% | 102.4% | 102.7% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.11% | -0.12% | 101.6% | 101.9% | 101.5% | 101.6% |
| Class C | -0.05% | -0.08% | 0.05% | 100.5% | 102.3% | 100.9% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.04% | 0.02% | 0.04% | 101.9% | 102.3% | 100.4% | 100.4% |
| Class F | 0.08% | 0.17% | 0.12% | 98.3% | 102.0% | 101.1% | 101.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.31% | -0.30% | -0.34% | 101.8% | 106.3% | 100.5% | 100.7% |
| Class E | -0.29% | 0.06% | 0.04% | 101.6% | 105.9% | 100.7% | 101.3% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.42% | -0.69% | -1.33% | 100.6% | 106.7% | 100.4% | 100.4% |
| Class F |  |  |  |  |  |  |  |

# References

[1] C. Ma, X. Xiu, C.-W. Kuo, W. Chen, X. Wang, “EE2-3.9: Affine candidates derived from temporal collocated pictures,” JVET-AI0197, Jul. 2024.

[2] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-13.0

[3] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.