| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0255-v2 |
| --- | --- |

| Title: | Non-EE2: NSPT kernels for non-regular intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Gleb Verba, Bappaditya Ray, Patrick Garus, Muhammed Coban, Vadim Seregin | Email: | gverba@qti.qualcomm.com bray@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to introduce an additional set of NSPT kernels for DIMD, TIMD, SGPM, MIP and EIP  predicted blocks with width <=16, height <=16 and width*height < 256.

The simulation results with respect to ECM-13.0 are as follows:

AI: -0.17% (Y), -0.10% (U), -0.10% (V), 99.8% (EncT), 100.1% (DecT)

# Introduction

In ECM-13.0, NSPT/LFNST kernel set is chosen based on the transform block size and the intra mode. NSPT is used for block shapes 4x4, 4x8, 4x16, 4x32, 8x8, 8x16, 8x32 and corresponding transposed block shapes, and LFNST is used for the rest of the block shapes. NSPT/LFNST kernel is used for the majority of the intra prediction tools such as regular intra prediction, DIMD, TIMD, SGPM, MIP, EIP, IntraTMP etc.

# Proposal

Currently in ECM, NSPT kernels are used for a subset of block shapes and kernels are derived using corresponding intra prediction direction (for blocks which does not have an inherent direction, a virtual intra direction is derived using DIMD process or partition direction). This contribution proposes to introduce an additional set of NSPT kernels only for blocks with width <=16, height <=16 and width * height < 256 and where the intra coding modes are either DIMD, TIMD, SGPM, MIP or EIP. The usage of the additional set is implicit and no additional signaling is required.

# Simulation results

The simulations are performed according to the CTC condition on top of ECM-13.0. The results for the AI configuration are as follows:

|  |  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-13.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |  |
| Class A1 |  | -0.06% | 0.11% |  | 0.08% |  | 100.1% |  | 100.4% |  | 99.6% |  | 99.0% |  |  |
| Class A2 |  | -0.12% | -0.04% |  | 0.02% |  | 98.7% |  | 97.6% |  | 99.7% |  | 98.7% |  |  |
| Class B |  | -0.17% | -0.12% |  | -0.24% |  | 99.8% |  | 100.1% |  | 99.9% |  | 98.6% |  |  |
| Class C |  | -0.16% | -0.19% |  | -0.04% |  | 100.4% |  | 101.4% |  | 99.9% |  | 98.7% |  |  |
| Class E |  | -0.34% | -0.20% |  | -0.27% |  | 100.0% |  | 100.2% |  | 100.1% |  | 98.8% |  |  |
| Overall |  | -0.17% | -0.10% |  | -0.10% |  | 99.8% |  | 100.1% |  | 99.9% |  | 98.7% |  |  |
| Class D |  | -0.06% | 0.09% |  | -0.08% |  | 100.4% |  | 101.4% |  | 100.3% |  | 98.7% |  |  |
| Class F |  | -0.16% | -0.08% |  | -0.06% |  | 97.8% |  | 97.2% |  | 99.8% |  | 99.1% |  |  |

# References

[1] ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).