| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0272-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0272 (EE2-1.2b : restricting BT CUs to apply QT-like partitioning structure) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | T. Dumas | Email: | thierry.dumas@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

The contribution reports the results of the cross-check of EE2-1.2, Test 1.2b, designed by Ericsson. In EE2-1.2, some restrictions are applied on CUs using BT splits to suppress QT-like partitioning structure when QT split is allowed for such CUs. EE2-1.2 derives from the contribution JVET-AH0155 presented during the 34th JVET meeting, Rennes, FR. On top of JVET-AH0155, EE2-1.2 adds one additional CABAC context to improve the coding efficiency of the proposed restriction. In EE2-1.2, Test 1.2b, the proposed restriction applies to 128x128, 64x64,32x32 and 16x16 CU sizes exclusively.

The code of EE2-1.2b has been run under the AHG12 CTC, and it is reported that the obtained BD-rates match the BD-rates provided by the proponents.

# Introduction

In ECM-13.0, the CUs using BT splits can feature QT-like partitioning structure even though the QT split is allowed for such CUs. This redundant partitioning structure exists in two ways. Firstly, when a square CU is partitioned by BTH split and its each subblock is further partitioned by BTV splitting as presented in Figure 1 (a). Secondly, when a square CU is partitioned by BTV split and its each subblock is further partitioned by BTH splitting as illustrated in Figure 1 (b).

CU with size m×m

BTH

BTV

BTV

a) BTH split is applied on a square block followed by BTV splits on its both subblocks.

CU with size m×m

BTH

BTH

BTV

b) BTV split is applied on a square block followed by BTH splits on its both subblocks.

Figure 1: the restriction introduced by EE2-1.2 on BT splitting to disallow QT like structure.

A restriction is proposed on CUs using BT splits to avoid applying QT-like structure, see Figure 1. When the BT split is applied on a luma channel CU, and the first subblock selects a different binary split compared to the binary split selected by its parent, then the same BT split as chosen by the first subblock is disallowed for the second subblock [2]. A new CABAC context is added to signal “split_cu_flag”. The context is used when the binary split is applied on a luma channel CU, and the first subblock selects a different binary split compared to the binary split selected by its parent CU.

In Test 1.2b, the proposed restriction applies to 128x128, 64x64,32x32 and 16x16 CU sizes only.

# Experiments

The source code of EE2-1.2, Test 1.2b, picked from the EE2 branch “TEST_1.2”, has been run under the AHG12 CTC [1].

In AI configuration, the results are reported in Table 1.

Table 1: EE2-1.2b versus ECM-13.0 in AI

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.08% | 0.00% | 99.4% | 100.0% | 101.5% | 100.0% |
| Class A2 | 0.01% | 0.06% | 0.04% | 99.2% | 99.3% | 102.0% | 100.0% |
| Class B | 0.03% | -0.01% | -0.16% | 97.3% | 100.4% | 99.8% | 100.0% |
| Class C | 0.01% | 0.12% | 0.06% | 97.8% | 100.3% | 100.0% | 100.0% |
| Class E | -0.02% | 0.19% | 0.13% | 98.2% | 100.7% | 102.0% | 100.1% |
| Overall | 0.01% | 0.08% | 0.00% | 98.2% | 100.1% | 100.9% | 100.0% |
| Class D | 0.03% | 0.10% | -0.12% | 99.9% | 100.2% | 99.9% | 100.0% |
| Class F | -0.03% | 0.13% | -0.13% | 99.4% | 100.0% | 101.2% | 99.9% |

In RA configuration, the results are presented in Table 2.

Table 2: EE2-1.2b versus ECM-13.0 in RA.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.20% | -0.10% | 99.0% | 100.1% | 100.0% | 99.9% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.01% | 0.03% | 98.9% | 99.9% | 100.0% | 100.3% |
| Class C | -0.01% | 0.06% | 0.19% | 99.1% | 99.6% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.04% | -0.11% | 0.24% | 99.8% | 100.0% | 99.9% | 100.0% |
| Class F | 0.03% | -0.24% | -0.12% | 100.5% | 102.0% | 100.1% | 100.4% |

In LDB configuration, the results are shown in Table 3.

Table 3: EE2-1.2B versus ECM-13.0 in LDB.

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.02% | 0.28% | -0.24% | 99.4% | 100.1% | 100.0% | 100.0% |
| Class E | 0.06% | -0.85% | -0.54% | 99.3% | 99.8% | 101.0% | 100.3% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | 0.78% | -0.33% | 99.7% | 100.4% | 99.9% | 100.0% |
| Class F |  |  |  |  |  |  |  |

The BD-rates produced by the cross-check match the BD-rates from the proponents of EE2-1.2b.

# References

[1] M. Karczewicz, Y. Ye. "Common Test Conditions and evaluation procedures for enhanced compression tool testing". JVET-AF2017, 32nd JVET meeting, Hannover, DE, 13-20 October 2023.

[2] W. Ahmad, P. Wennersten, K. Andersson. “EE2-1.2 : restricting BT CUs to apply QT-like partitioning structure”. JVET-AI0087, 35th JVET meeting, Sapporo, JP, 12–19 July 2024.

# Patent rights declaration(s)

Interdigital does not have any current or pending patent rights relating to the technology described in this contribution.