| Joint Video Experts Team (JVET) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29 35th Meeting: Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0319-v3 Document: WG05 M069205 |
| --- | --- |

| Title: | A historic analysis of Multi-Layer VVC (MLVVC) Verification Test (VT) Plan. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Test plan document |  |  |
| Author(s) or Contact(s): | Stefano Battista | Tel: Email: | bautz66@gmail.com |
| Source: | UNIVPM, IT |  |  |

# Abstract

This document collects a summary of the available JVET/WG05 contributions related to the Multi-Layer Versatile Video Coding (ML VVC) Verification Test (VT) planning and preliminary test results.

The period considered is between 2022/01 (JVET meeting 25, MPEG meeting 137, WG05 meeting 6) and 2024/04 (JVET meeting 34, MPEG meeting 146, WG05 meeting 15).

In summary, the “Verification Test plan for VVC Multi-Layer coding” has been published in 6 successive versions, and the “Results on Spatial Scalability category of the VVC Multi-Layer Verification Test” have been published in 4 separate reports.

In all versions of the preliminary tests, the range of bitrates used for conducting the subjective tests or the expert viewing tests is between about 0.5 Mbps and about 10.0 Mbps.

In the large majority of tests, with different sequences and a different number of test points (2, 3, 4), the comparison, in terms of MOS versus BR curves, of VVC Single-Layer at full resolution UHD (3840 * 2160) with VVC Dual-Layer at base resolution HD (1920 * 1080) and enhancement resolution UHD (3840 * 2160) shows a small gain (of the order of 1.0 MOS points) for DL at lower bitrates, a crossover at intermediate bitrates, a small gain (of the order of 1.0 MOS points) for SL at higher bitrates.

Approximately, the DL over SL gain is at around 1.0 Mbps, the crossover at around 2.0 Mbps, the SL over DL gain is at around 4.0 Mbps and above.

# List of documents analyzed.

| 01 | Y0047 (M058568) meeting 25 – subjective test of Spatial Scalability “AHG4: first report of spatial scalability verification tests” (01) Philippe de Lagrange, Inter Digital, FR. [01] |
| --- | --- |
| 02 | AB0123 (M060888) meeting 28 – ML VVC Verification Test plan “Proposed multilayer VVC verification test draft plan” (01) Philippe de Lagrange, Inter Digital, FR. [02] |
| 03 | AB2021 (M061503) meeting 28 – ML VVC Verification Test plan “Draft verification test plan for VVC multilayer coding” (02) Mathias Wien, RWTH, DE. [03] |
| 04 | AC2021 (M062438) meeting 29 – ML VVC Verification Test plan “Draft verification test plan for VVC multilayer coding” (03) Mathias Wien, RWTH, DE. [04] |
| 05 | AD0399 (M063562) meeting 30 – expert viewing of Spatial Scalability “Results of expert viewing for the spatial scalability category of the VVC multilayer VT” (02) Mathias Wien, RWTH, DE. [05] |
| 06 | AD2021 (M063600) meeting 30 – ML VVC Verification Test plan “Verification test plan for VVC multilayer coding” (04) Mathias Wien, RWTH, DE. [06] |
| 07 | AE0288 (M064522) meeting 31 – expert viewing of Spatial Scalability “Results of expert viewing for the spatial scalability category of the VVC multilayer VT” (03) Mathias Wien, RWTH, DE. [07] |
| 08 | AE2021 (M064601) meeting 31 – ML VVC Verification Test plan “Verification test plan for VVC multilayer coding” (05) Mathias Wien, RWTH, DE. [08] |
| 09 | AF0311 (M065735) meeting 32 – expert viewing of Spatial Scalability “Results of expert viewing for the spatial scalability category of the VVC multilayer VT” (04) Mathias Wien, RWTH, DE. [09] |
| 10 | AF2021 (M065816) meeting 32 – ML VVC Verification Test plan “Verification test plan for VVC multilayer coding” (06) Mathias Wien, RWTH, DE. [10] |
| 11 | AH2021 (M068143) meeting 34 – ML VVC Verification Test plan “Verification test plan for VVC multilayer coding (update 3)” (07) Mathias Wien, RWTH, DE. [11] |

# Meeting 2022/01

## Y0047 (M058568) meeting 25 (subjective test) [01]

ML VVC (spatial scalability)

Philippe de Lagrange, Inter Digital, FR.

Range of Base Layer QP from 23 to 45.

Range of Enhancement Layer QP from 28 to 47.

Range of Bitrates between 0.4 and 13.4 Mbps.

With equal bitrate for BL HD and EL UHD layers,

PSNR-Y based performance (BD Rate) is –24% vs simulcast,

MOS-based performance (BD Rate) is –15% vs simulcast.

| SEQUENCE | BL QP | BL BR | EL QP | EL BR | TOTAL |
| --- | --- | --- | --- | --- | --- |
| Procession | 44 | 471 | 46 | 393 | 864 |
|  | 39 | 958 | 40 | 1123 | 2081 |
|  | 33 | 2111 | 34 | 2455 | 4566 |
|  | 26 | 5630 | 28 | 5216 | 10846 |
| RiverBoat | 45 |  | 47 |  | 696 |
|  | 39 |  | 41 |  | 1662 |
|  | 33 |  | 35 |  | 3814 |
|  | 27 |  | 29 |  | 9072 |
| Marathon2 | 43 |  | 45 |  | 1221 |
|  | 39 |  | 41 |  | 2244 |
|  | 36 |  | 38 |  | 3459 |
|  | 31 |  | 33 |  | 7070 |
|  | 27 |  | 29 |  | 13415 |
| MountainBay2 | 42 |  | 43 |  | 401 |
|  | 37 |  | 39 |  | 790 |
|  | 34 |  | 36 |  | 1345 |
|  | 31 |  | 33 |  | 2264 |
|  | 28 |  | 30 |  | 3889 |
| TallBuildings2 | 42 |  | 44 |  | 696 |
|  | 38 |  | 40 |  | 1253 |
|  | 33 |  | 36 |  | 2203 |
|  | 28 |  | 32 |  | 3928 |
|  | 23 |  | 28 |  | 7149 |

# Meeting 2022/10

## AB0123 (M060888) meeting 28 (test plan) [02]

ML VVC Verification Test Plan

Philippe de Lagrange, Inter Digital, FR.

Mathias Wien, RWTH, DE.

Summary:

QP(BL) = QP(EL) – 6

Dry run.

### Spatial Scalability

Five bitrate points per sequences covering the whole MOS range.

1920x1080 HD base layer coded with UHD enhancement layer QP – 6.

3840x2160 UHD enhancement layer coded with QP.

Dry-run experiments are needed for the selection of sequences and QP settings.

### Quality Ladder.

| Resolution | BR (kbps) | % of total | % of total |
| --- | --- | --- | --- |
| 416 x 234 | 130 | 0.4 |  |
| 480 x 270 | 260 | 0.7 |  |
| 640 x 360 | 530 | 1.5 |  |
| 768 x 432 | 790 | 2.2 |  |
| 960 x 540 | 1300 | 3.6 |  |
| 1280 x 720 | 1800 | 5.0 |  |
| 1280 x 720 | 2400 | 6.6 |  |
| 1920 x 1080 | 3100 | 8.5 |  |
| 1920 x 1080 | 4000 | 11.0 | 39.5 |
| 2560 x 1440 | 5000 | 14.0 |  |
| 3840 x 2160 | 7000 | 19.0 |  |
| 3840 x 2160 | 10000 | 28.0 | 61.0 |

## AB2021 (M061503) meeting 28 (test plan) [03]

ML VVC Verification Test Plan

Philippe de Lagrange, Inter Digital, FR.

Mathias Wien, RWTH, DE.

Summary:

QP(BL) = QP(EL) – 6

Dry run.

### Spatial Scalability

Five bitrate points per sequences covering the whole MOS range.

1920x1080 HD base layer coded with UHD enhancement layer QP – 6.

3840x2160 UHD enhancement layer coded with QP.

# Meeting 2023/01

## AC2021 (M062438) meeting 29 (test plan) [04]

ML VVC Verification Test Plan

Philippe de Lagrange, Inter Digital, FR.

Mathias Wien, RWTH, DE.

Summary:

QP(BL) = QP(EL) – 6

Dry run.

Report with dry run.

### Spatial Scalability

Five bitrate points per sequences covering the whole MOS range.

1920x1080 HD base layer coded with UHD enhancement layer QP – 6.

3840x2160 UHD enhancement layer coded with QP.

Dry-run experiments are needed for the selection of sequences and QP settings.

Report on activity by input document to the 30th (AD) JVET meeting:

dry runs and assessment of suitability of rate points and sequences.

### Quality Ladder

| Resolution | HEVC (HDR) | VVC | % of HEVC HDR | % of previous | % of total | % of total |
| --- | --- | --- | --- | --- | --- | --- |
| 416×234 | 160 | 130 | 81 | - | 0.4 |  |
| 480×270 | 360 | 260 | 72 | 200 | 0.7 |  |
| 640×360 | 730 | 530 | 73 | 204 | 1.5 |  |
| 768×432 | 1090 | 790 | 72 | 149 | 2.2 |  |
| 960×540 | 1930 | 1300 | 67 | 165 | 3.6 |  |
| 1280×720 | 2900 | 1800 | 62 | 138 | 4.9 |  |
| 1280×720 | 4080 | 2400 | 59 | 133 | 6.6 |  |
| 1920×1080 | 5400 | 3100 | 57 | 129 | 8.5 |  |
| 1920×1080 | 7000 | 4000 | 57 | 129 | 11.0 | 39.4 |
| 2560×1440 | 9700 | 5200 | 54 | 130 | 14.2 |  |
| 3840×2160 | 13900 | 7000 | 50 | 135 | 19.2 |  |
| 3840×2160 | 20000 | 10000 | 50 | 143 | 27.4 | 60.8 |

# Meeting 2023/04

## AD0399 (M063562) meeting 30 (expert viewing) [05]

Expert Viewing ML VVC Spatial Scalability

Mathias Wien, RWTH, DE.

### Spatial Scalability

Range of Base Layer QP from 26 to 40.

Range of Enhancement Layer QP from 35 to 49.

Range of Bitrates between 0.4 and 7.4 Mbps.

Range of MOS:

- CorsaireJoy from 2 to 8

- DrivingPOV3 from 2 to 8

- Procession from 3 to 7

- Marathon2 from 2 to 8

| SEQUENCE | SL QP | BL QP | EL QP | TOTAL BR |
| --- | --- | --- | --- | --- |
| CorsaireJoy | 35, 38, 42 | 29, 32, 36 | 36, 40, 44 | 1032, 690, 415 |
| DrivingPOV3 | 34, 39, 46 | 28, 33, 40 | 35, 40, 47 | 4046, 1808, 669 |
| Procession | 38, 42, 46 | 32, 36, 40 | 40, 44, 47 | 2421, 1421, 842 |
| Marathon2 | 32, 38, 46 | 26, 32, 40 | 35, 41, 49 | 7364, 2944, 896 |
| MountainBay2 |  |  |  |  |
| TallBuildings2 |  |  |  |  |
| Fortnite |  |  |  |  |

MOS differences between +1.0, 0.0, –1.0 points (DL over SL).

## AD2021 (M063600) meeting 30 (test plan) [06]

ML VVC Verification Test Plan

Philippe de Lagrange, Inter Digital, FR.

Mathias Wien, RWTH, DE.

Summary:

QP(BL) = QP(SL) – 6

QP(EL) = QP(SL) + 1

Dry run.

Report with dry run.

### Spatial Scalability

Five bitrate points per sequences covering the whole MOS range.

1920x1080 HD base layer coded with UHD single layer QP – 6.

3840x2160 UHD enhancement layer coded with UHD single layer QP + 1.

Report on activity by input document to the 31st (AE) JVET meeting including dry runs:

- Assessment of suitability of rate points and sequences.

- Assessment of appropriate setting for the DMVREncMvSelect parameter.

# Meeting 2023/07

## AE0288 (M064522) meeting 31 (expert viewing) [07]

Expert Viewing ML VVC Spatial Scalability

Mathias Wien, RWTH, DE.

### Spatial Scalability

Range of Base Layer QP from 24 to 40.

Range of Enhancement Layer QP from 31 to 47.

Range of Bitrates between 0.5 and 7.5 Mbps.

Range of MOS:

- CorsaireJoy from 4 to 8

- DrivingPOV3 from 4 to 8

- Procession from 4 to 8

- Marathon2 from 3 to 7

- MountainBay2 from 3 to 9

- TallBuildings2 from 2 to 9

| SEQUENCE | SL QP | BL QP | EL QP | TOTAL BR |
| --- | --- | --- | --- | --- |
| CorsaireJoy | 33, 40 | 27, 34 | 34, 41 |  |
| DrivingPOV3 | 34, 42 | 28, 36 | 35, 43 |  |
| Procession | 35, 44 | 29, 38 | 36, 45 |  |
| Marathon2 | 32, 41 | 26, 35 | 33, 42 |  |
| MountainBay2 | 30, 38, 42 | 24, 32, 36 | 31, 39, 43 |  |
| TallBuildings2 | 34, 42, 46 | 28, 36, 40 | 35, 43, 47 |  |
| Fortnite |  |  |  |  |

MOS differences between +1.0, 0.0, –1.0 points (DL over SL).

- DL slight gain compared to UP for DrivingPOV3

- DL very minor gain compared to UP for MountainBay2.

- DL slight loss compared to UP for Procession.

- DL more loss compared to UP for CorsaireJoy, Marathon2, TallBuilding2.

## AE2021 (M064601) meeting 31 (test plan) [08]

ML VVC Verification Test Plan

Philippe de Lagrange, Inter Digital, FR.

Mathias Wien, RWTH, DE.

Summary:

QP(BL) = ???? REMOVED

QP(EL) = ???? REMOVED

Dry run REMOVED.

Report with dry run REMOVED.

### Spatial Scalability

Four bitrate points per sequences covering the whole MOS range.

The QP relation between single layer, as well as base and enhancement layer, is TO BE DEFINED.

Report on activity by input document to the 32nd (AF) JVET meeting [including dry runs]:

- Assessment of suitable parameters for encoder control.

- Assessment of suitability of rate points and sequences.

| SEQUENCE | SL QP |  |  |
| --- | --- | --- | --- |
| CorsaireJoy | 33, 37, 40, 43 |  |  |
| DrivingPOV3 | 34, 38, 42, 46 |  |  |
| Procession | 35, 40, 44, 47 |  |  |
| Marathon2 | 32, 36, 41, 46 |  |  |
| MountainBay2 | 30, 34, 38, 42 |  |  |
| TallBuildings2 | 34, 38, 42, 46 |  |  |
| Fortnite | 40, 43, 46, 49 |  |  |

# Meeting 2023/10

## AF0311 (M065735) meeting 32 (expert viewing) [09]

Expert Viewing ML VVC Spatial Scalability

Mathias Wien, RWTH, DE.

### Spatial Scalability

Range of Base Layer QP from 27 to 37.

Range of Enhancement Layer QP from 34 to 44.

Range of Bitrates between 0.5 and 5.0 Mbps.

Range of MOS:

- CorsaireJoy from 3 to 8

- DrivingPOV3 from 3 to 8

- TallBuildings2 from 3 to 8

- BoxeLogo from 2 to 8

| SEQUENCE | SL QP | BL QP | EL QP | TOTAL BR |
| --- | --- | --- | --- | --- |
| CorsaireJoy | 33, 40 | 27, 34 | 34, 41 |  |
| DrivingPOV3 | 32, 38, 42 | 27, 33, 37 | 34, 40, 44 |  |
| Procession |  |  |  |  |
| Marathon2 |  |  |  |  |
| MountainBay2 |  |  |  |  |
| TallBuildings2 | 34, 42 | 28, 36 | 35, 43 |  |
| Fortnite |  |  |  |  |
| CatRobot1 |  |  |  |  |
| LupoPuppet |  |  |  |  |
| DrivingPOVLogo |  |  |  |  |
| BoxeLogo | 34, 38, 42 | 30, 32, 37 | 37, 39, 44 |  |
| TrafficHDLogo |  |  |  |  |
| StarCraft |  |  |  |  |

MOS differences between +1.0, 0.0, –1.0 points (DL over SL).

## AF2021 (M065816) meeting 32 (test plan) [10]

ML VVC Verification Test Plan

Philippe de Lagrange, Inter Digital, FR.

Mathias Wien, RWTH, DE.

Summary:

QP(BL) = ???? REMOVED

QP(EL) = ???? REMOVED

Dry run REMOVED.

Report with dry run REMOVED.

Test sequences added (from the LCEVC VT).

### Spatial Scalability

Four bitrate points per sequence, covering the whole MOS range.

The QP relation between single layer, as well as base and enhancement layer are TO BE DEFINED.

Report on activity by input document to the 34th (AH) JVET meeting:

- Assessment of suitable parameters for encoder control.

- Assessment of suitability of rate points and sequences.

| SEQUENCE | SL QP |  |  |
| --- | --- | --- | --- |
| CorsaireJoy | 33, 37, 40, 43 |  |  |
| DrivingPOV3 | 34, 38, 42, 46 |  |  |
| Procession | 35, 40, 44, 47 |  |  |
| Marathon2 | 32, 36, 41, 46 |  |  |
| MountainBay2 | 30, 34, 38, 42 |  |  |
| TallBuildings2 | 34, 38, 42, 46 |  |  |
| Fortnite | 40, 43, 46, 49 |  |  |
| CatRobot1 | 28, 33, 38, 43 |  |  |
| LupoPuppet | 31, 35, 39, 43 |  |  |
| DrivingPOVLogo | 32, 37, 42, 46 |  |  |
| BoxeLogo | 30, 34, 38, 42 |  |  |
| TrafficHDLogo | 26, 30, 34, 38 |  |  |
| StarCraft | 26, 30, 34, 38 |  |  |

# Meeting 2024/01

## AG2021 (Mxxxxxx) meeting 33 (test plan)

ML VVC Verification Test Plan

SKIPPED.

# Meeting 2024/04

## AH2021 (M068143) meeting 34 (test plan) [11]

ML VVC Verification Test Plan

Registered on 2024/04/24 at 15:32:45

SCHEDULED for 2024/07/05.

# BD Rates of MLVVC vs VVC from Expert Viewing

The RD curves of BR versus MOS for the 7 sequences tested with at least 3 test points in the expert viewings reported in AD0399, AE0288, AF0311 ([05], [07], [09]) show a similar trend, when comparing VVC at Full resolution UHD with VVC at Quarter resolution HD plus MLVVC enhancement to Full resolution UHD.

## Legenda

Azure curve: BR (logarithmic) versus MOS for VVC Full resolution (UHD) from MLVVC VT expert viewing (up to 2024/01).

Orange curve: BR (logarithmic) versus MOS for VVC Quarter resolution (HD) + MLVVC Full resolution (UHD) from MLVVC VT expert viewing (up to 2024/01).

## Comparison

Below a certain value of MOS, the VVC curve is above the MLVVC curve, showing that there is an advantage for MLVVC at lower MOS and BR.

At a certain value of MOS, there is a crossover between the two RD curves.

Above a certain value of MOS, the MLVVC curve is above the VVC curve, showing that there is an advantage for VVC at higher MOS and BR.

The crossover values of MOS and BR are reported below for each sequence.

The crossover takes place for MOS values between 3.5 (Tall Buildings) and 6.8 (Boxe Logo).

The crossover takes place for BR values between 0.6 Mbps (Tall Buildings) and 3.2 Mbps (Marathon).

Marathon:

Crossover at MOS = 5.3, log(BR) = 3.50, BR = 3.2 Mbps.

Mountain Bay:

Crossover at MOS = 5.5, log(BR) = 3.00, BR = 1.0 Mbps.

Driving Logo:

Crossover at MOS = 5.0, log(BR) = 3.26, BR = 1.8 Mbps.

Boxe Logo:

Crossover at MOS = 6.8, log(BR) = 3.06, BR = 1.1 Mbps.

Corsaire:

Crossover at MOS = 3.8, log(BR) = 2.84, BR = 0.7 Mbps.

Procession:

Crossover at MOS = 4.8, log(BR) = 2.91, BR = 0.8 Mbps.

Tall Buildings:

Crossover at MOS = 3.5, log(BR) = 2.81, BR = 0.6 Mbps.

## BD Rates (MLVCC)

The table below reports the BD Rates computed from the JVET documents with BR and MOS data for 7 sequences, for which at least three test points are available.

The last two columns report the same DB Rates, but using as MOS range only values below 5.0 and only values above 5.0, respectively. The goal is to have an estimate of the BD Rates performance in the “low MOS range” (below 5.0) and in the “high MOS range” (above 5.0).

| SEQUENCE | BD Rate | MOS < 5 | MOS > 5 |
| --- | --- | --- | --- |
| Marathon [05] | -5,99 | -16,79 | 11,38 |
| Mountain Bay [07] | 4,56 | -11,63 | 10,48 |
| Driving Logo [09] | 9,77 | -7,66 | 15,78 |
| Boxe Logo [09] | -14,72 | -19,63 | -10,54 |
| AVERAGE4 | -1,60 | -13,93 | 6,78 |
| Corsaire [05] | 2,57 | 0,96 | 4,35 |
| Procession [05] | -2,46 | -5,62 | 0,10 |
| Tall Buildings [07] | 12,66 | 2,81 | 19,77 |
| AVERAGE7 | 0,91 | -8,22 | 7,33 |

## BD Rates (LCEVC)

The table below reports the BD Rates from the WG04 document with the LCEVC Verification Test report, N0076 [12].

| SEQUENCE | BD Rate |
| --- | --- |
| Marathon [12] | -16,82 |
| Mountain Bay [12] | -16,92 |
| Driving Logo [12] | -10,27 |
| Boxe Logo [12] | -18,64 |
| AVERAGE4 | -15,66 |

## RD Curves: Marathon, Mountain Bay, Driving Logo, Boxe Logo

## RD Curves: Corsaire, Procession, Tall Buildings

# BD Rates of LCEVC Verification Test and MLVVC Expert Viewing

The RD curves of BR versus MOS for the 4 sequences used in LCEVC Verification Test ([12]) and used in MLVVC Expert Viewing with at least 3 test points reported in AD0399, AE0288, AF0311 ([05], [07], [09]) are reported in the same RD graph per sequence.

## Legenda

Blue curve: BR (logarithmic) versus MOS for VVC Full resolution (UHD) from LCEVC VT (2021/04) [12].

Red curve: BR (logarithmic) versus MOS for VVC Quarter resolution (HD) + LCEVC Full resolution (UHD) from LCEVC VT (2021/04) [12].

Azure curve: BR (logarithmic) versus MOS for VVC Full resolution (UHD) from MLVVC VT expert viewing (up to 2024/01) [05], [07], [09].

Orange curve: BR (logarithmic) versus MOS for VVC Quarter resolution (HD) + MLVVC Full resolution (UHD) from MLVVC VT expert viewing (up to 2024/01) [05], [07], [09].

## Comparison

Comparing VVC Full resolution in 2021/04 (blue) and VVC Full resolution in 2024/04 (azure):

- VVC 2024 tends to perform better with respect to VVC 2021 at higher MOS (from about MOS = 6.0 up).

Comparing LCEVC in 2021/04 (red) and MLVVC in 2024/04 (orange):

- MLVVC tends to perform better with respect to LCEVC at higher MOS (from about MOS = 6.0 up).

## RD Curves: Marathon, Mountain Bay

## RD Curves: Driving Logo, Boxe Logo

## RD Curves: VVC21, VVC24, LCEVC, MLVVC

Quadratic model for each sequence:

MOS=k1*BR2+k2*BR+k3

Average of quadratic parameters (ki) over 4 common sequences: Marathon, Mountain Bay, Driving Logo, Boxe Logo:

ki, avg=1nj=1nki, j

Comparison of the average quadratic models. Methodology described in [13].

VVC24 crossover with VVC21:

MOS = 5.6; log(BR) = 3.21; BR = 1.6 Mbps.

MLVVC crossover with LCEVC:

MOS = 9.1; log(BR) = 3.82; BR = 6.6 Mbps.

MLVVC saturation  MOS = 10.0; log(BR) = 3,99; BR = 9.8 Mbps.

LCEVC saturation  MOS = 10.0; log(BR) = 4.02; BR = 10.5 Mbps.

# Conclusions

This document collects and analyzes the available JVET/WG05 contributions related to the Multi-Layer Versatile Video Coding (ML VVC) Verification Test (VT) planning and preliminary test results, for the period since 2022/01 (JVET meeting 25, MPEG meeting 137, WG05 meeting 6).

The available information provided to the JVET/WG05 community shows that:

- In all versions of the preliminary tests, the range of bitrates used for conducting the subjective tests or the expert viewing tests is between about 0.5 Mbps and about 10.0 Mbps.

- In the large majority of tests, with different sequences and a different number of test points (2, 3, 4), the comparison, in terms of MOS versus BR curves, of VVC Single-Layer at full resolution UHD (3840 * 2160) with VVC Dual-Layer at base resolution HD (1920 * 1080) and enhancement resolution UHD (3840 * 2160) shows a small gain (of the order of 1.0 MOS points) for DL at lower bitrates, a crossover at intermediate bitrates, a small gain (of the order of 1.0 MOS points) for SL at higher bitrates.

- Approximately, the DL over SL gain is at around 1.0 Mbps, the crossover at around 2.0 Mbps, the SL over DL gain is at around 4.0 Mbps and above.

- There is a saturation of the subjective quality in terms of MOS reaching the value 10 at about 10 Mbps for both MLVVC (form the preliminary tests) and LCEVC (from the verification tests).

The execution of the MLVVC verification tests can be performed without further delay, applying the methodology already described and preliminarily tested in documents:

- AD0399 of 2023/04

- AE0288 of 2023/07

- AF0311 of 2023/10

These documents already identify the bitrate ranges and QP ranges for the MLVVC verification tests (which are also in the same range as the LCEVC verification tests performed in 2021/04).

Unless, of course, there are implementation issues with the Reference Software for MLVCC.

# References

- Y0047 (M058568) meeting 25 (2022/01) – “AHG4: first report of spatial scalability verification tests” – Philippe de Lagrange, Inter Digital, FR.

- AB0123 (M060888) meeting 28 (2022/10) – “Proposed multilayer VVC verification test draft plan” – Philippe de Lagrange, Inter Digital, FR.

- AB2021 (M061503) meeting 28 (2022/10) – “Draft verification test plan for VVC multilayer coding” – Mathias Wien, RWTH, DE.

- AC2021 (M062438) meeting 29 (2023/01) – “Draft verification test plan for VVC multilayer coding” – Mathias Wien, RWTH, DE.

- AD0399 (M063562) meeting 30 (2023/04) – “Results of expert viewing for the spatial scalability category of the VVC multilayer VT” – Mathias Wien, RWTH, DE.

- AD2021 (M063600) meeting 30 (2023/04) – “Verification test plan for VVC multilayer coding” – Mathias Wien, RWTH, DE.

- AE0288 (M064522) meeting 31 (2023/07) – “Results of expert viewing for the spatial scalability category of the VVC multilayer VT” – Mathias Wien, RWTH, DE.

- AE2021 (M064601) meeting 31 (2023/07) – “Verification test plan for VVC multilayer coding” – Mathias Wien, RWTH, DE.

- AF0311 (M065735) meeting 32 (2023/10) – “Results of expert viewing for the spatial scalability category of the VVC multilayer VT” – Mathias Wien, RWTH, DE.

- AF2021 (M065816) meeting 32 (2023/10) – “Verification test plan for VVC multilayer coding” – Mathias Wien, RWTH, DE.

- AH2021 (M068143) meeting 34 (2024/04) – “Verification test plan for VVC multilayer coding (update 3)” – Mathias Wien, RWTH, DE.

- WG04/N0076 (W020173) (2021/04) – “Verification Test Report on the Compression Performance of Low Complexity Enhancement Video Coding” – Lu Yu, Zhejiang University, CN.

- “Methodology for Modeling and Comparing Video Codecs: HEVC, EVC, and VVC” – MDPI Electronics (2020/09) – Stefano Battista, Massimo Conti, Simone Orcioni.