| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li R. Chernyak K. Naser Y. Yu | Tel: Email: | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@google.com chernyak@global.tencent.com karam.naser@interdigital.com yue.yu@oppo.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 35th meeting, Sapporo, Japan, 12–19 July 2024, and the 36th meeting, Kemer, Turkey, 1–8 November 2024.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM13 algorithm description JVET-AI2025.

- Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Support AHG6 in generating anchors according to the test conditions in JVET-AI2017.

- Analyse the results of exploration experiments described in JVET-AI2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The Common Test Conditions were updated (JVET-AI2017). A new classF_lowdelay configuration file was added, which will be used in low-delay coding of screen content sequences (classes F and TGM). The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AI2024). The combined improvements of the ECM-14.0 over VTM-11.0ecm14.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -13.22% | -14.28% | -25.56% | 1027.4% | 499.0% |
| Class A2 | -20.23% | -22.93% | -27.49% | 1017.1% | 516.3% |
| Class B | -13.68% | -21.41% | -19.24% | 946.6% | 526.5% |
| Class C | -13.74% | -10.65% | -11.72% | 936.0% | 466.6% |
| Class E | -17.61% | -20.82% | -19.15% | 907.0% | 530.6% |
| Overall | -15.36% | -17.98% | -19.98% | 961.9% | 507.0% |
| Class D | -11.59% | -7.67% | -8.17% | 924.6% | 483.1% |
| Class F | -29.28% | -32.84% | -33.09% | 606.3% | 558.8% |
| Class TGM | -42.24% | -47.92% | -47.17% | 508.4% | 604.3% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -26.35% | -22.60% | -34.85% | 1050.1% | 994.0% |
| Class A2 | -29.45% | -32.76% | -38.16% | 992.2% | 1183.8% |
| Class B | -23.85% | -30.85% | -28.16% | 849.2% | 1008.1% |
| Class C | -25.61% | -21.03% | -21.95% | 876.9% | 1068.8% |
| Class E |  |  |  |  |  |
| Overall | -25.94% | -26.96% | -29.84% | 921.9% | 1054.4% |
| Class D | -26.48% | -21.78% | -22.78% | 844.5% | 1165.9% |
| Class F | -32.10% | -34.18% | -35.05% | 712.1% | 677.3% |
| Class TGM | -41.56% | -47.16% | -46.90% | 661.3% | 604.4% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -18.82% | -44.06% | -41.03% | 753.6% | 812.9% |
| Class C | -21.38% | -33.77% | -34.25% | 686.8% | 858.2% |
| Class E | -19.03% | -33.98% | -34.83% | 700.4% | 601.9% |
| Overall | -19.73% | -38.11% | -37.22% | 717.4% | 767.8% |
| Class D | -23.76% | -35.64% | -36.67% | 715.2% | 1029.2% |
| Class F | -27.42% | -42.56% | -42.93% | 613.6% | 631.9% |
| Class TGM | -37.61% | -51.53% | -50.84% | 623.0% | 564.0% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-13.0’s {-25.51%, -25.80%, -28.72%} to ECM-14.0’s {-25.94%, -26.96%, -29.84%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-13.0’s {-40.63%, -45.97%, -46.06%} to ECM-14.0’s { -41.56%, -47.16%, -46.90%}.

# Contributions

In addition to 46 EE2 contributions, 50 (comparing to 46 last meeting) EE2-related and AHG12-related contributions were received. The EE2-related and AHG12-related contributions can be subdivided as follows:

## Intra (22)

JVET-AJ0087, "Non-EE2: Block vector guided DIMD", L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AJ0088, "Non-EE2: On MPM with matrix-based position dependent intra prediction", Z. Sun, Y. Yu, H. Yu, L. Xu, D. Wang (OPPO)

JVET-AJ0098, "Non-EE2: OBIC with improved sampling strategy and scaling weights", Y. Liu, Y. Huo (Transsion), G. Li, J. Chen, L. Luo, H. Guo, C. Zhu (UESTC)

JVET-AJ0102, "Non-EE2: Intra mode coding based on HoG of neighboring templates", M. Hong, J. Choi, J. Lim, S. Kim (LGE)

JVET-AJ0113, "Non-EE2: Intra OBMC", X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-AJ0116, "Non-EE2: On interpolation filter selection for TIMD", Z. Zhang, J. Chen, X. Li, R.-L. Liao, Y. Ye (Alibaba)

JVET-AJ0118, "Non-EE2: Virtual intra prediction mode (VIPM) for chroma intra coding", J. Choi, M. Hong, J. Lim, S. Kim (LGE)

JVET-AJ0152, "AhG12: An alternative TIMD mode from common reference samples", H. Qin, K. Ding, J. Konieczny, A. Filippov, V. Rufitskiy, T. Dong (TCL)

JVET-AJ0155, "AhG12: SGPM Improvements", K. Ding, H. Qin, J. Konieczny, A. Filippov, V. Rufitskiy, T. Dong (TCL)

JVET-AJ0156, "Non-EE2 Adaptive interpolation filter for reference sample in angular mode", T. Dong, A. Filippov, J. Konieczny, H. Qin, K. Ding (TCL)

JVET-AJ0157, "Non-EE2: Template matching based jointly reordering for GPM split modes and partition indexes", C. Ma, X. Xiu, X. Wang (Kwai)

JVET-AJ0169, "Non-EE2: Improvement on non-MPM", G. Wang, C. Zhou, Z. Lv (vivo)

JVET-AJ0171, "Non-EE2: Weighted OBIC", P. Andrivon, M. Blestel, N. Zouidi (Ofinno)

JVET-AJ0194, "AHG12: Extended IntraTMP merge candidate list with an improved fusion mode", D. Ruiz Coll, J.-K. Lee (Ofinno)

JVET-AJ0216, "Non-EE2: On interpolation filter for intra template prediction", C. Zhou, Z. Lv, G. Wang (vivo)

JVET-AJ0217, "Non-EE2: Template-based intra interpolation filter", C. Zhou, Z. Lv, G. Wang (vivo)

JVET-AJ0218, "Non-EE2: Multiple filter taps for EIP", C. Zhou, Z. Lv, G. Wang (vivo)

JVET-AJ0223, "Non-EE2: Bilateral Filtering for Intra Prediction", W. Yin, K. Zhang, Y. Wang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)

JVET-AJ0229, "EE2-related: Non-adjacent DIMD fusion modes for TMRL intra mode candidate list construction", V. Rufitskiy, A. Filippov, J. Konieczny, H. Qin, T. Dong, K. Ding (TCL)

JVET-AJ0230, "Non-EE2: Non-EE2: On reference sample processing for angular intra-prediction modes", A. Filippov, V. Rufitskiy, T. Dong, J. Konieczny, H. Qin (TCL)

JVET-AJ0269, "Non-EE2: OBIC extension with PDP and advanced sampling strategy", J. Fu, Z. Li, Y. Zhao, J. Zhang, C. Jia, S. Ma (PKU)

JVET-AJ0299, "AHG12: Intra TMP improvements", M. Radosavljević, F. Le Léannec, K. Naser, T. Dumas (InterDigital)

## Inter (5)

JVET-AJ0043, "Template Matching Picture Boundary Padding in ECM 14", N. Neumann, M. Wien (RWTH Aachen)

JVET-AJ0134, "Non-EE2: Enhanced derivation of affine merge candidates", L. Zhao, K. Zhang, Z. Deng, N. Zhang, Y. Wang, W. Yin, M. Salehifar, L. Zhang (Bytedance)

JVET-AJ0089, "Non-EE2: GPM extension", H. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AJ0117, "Non-EE2: Regression-based GPM with intra and inter prediction", K. Jia, J. Chen, X. Li, R.-L. Liao, Y. Ye (Alibaba)

JVET-AJ0261, "Non-EE2: TMVP Candidate Selection", S. Hong, L. Wang, K. Panusopone (Nokia)

## In Loop Filters (6)

JVET-AJ0090, "AHG12: temporal ALF", L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AJ0173, "AHG12: ALF-CCCM", P. Astola, I. Jumakulyyev, D. Bugdayci Sansli, J. Lainema (Nokia)

JVET-AJ0174, "Non-EE2: Laplacian Information for Adaptive Loop Filter", I. Jumakulyyev, D. Bugdayci Sansli, J. Funnel, J. Lainema (Nokia)

JVET-AJ0210, "AhG12: On In-Loop Filtering in ECM", D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)

JVET-AJ0224, "Non-EE2: Boundary-aware Offset Refinement for In-Loop-Filters", W. Yin, K. Zhang, Y. Wang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)

JVET-AJ0276, "Non-EE2: Improvement to ALF scaling factor", M. Karczewicz, N. Hu, H. Wang, Y. Shao, V. Seregin (Qualcomm)

## Transform (4)

JVET-AJ0086, "Non-EE2: LFNST/NSPT set derivation for CCP coded block", H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AJ0164, "AhG12: On advanced SBT with position and direction inference", G. Laroche, M. Rousselot, P. Onno (Canon)

JVET-AJ0255, "AHG12: Multiple transform set selection for intra LFNST/NSPT with subsampled DIMD for transform set selection", M. Coban, M. Karczewicz, P. Nikitin, P. Garus, B. Ray, V. Seregin (Qualcomm), F. Wang, Y. Yu, H. Yu, D. Wang (OPPO), C. Bonnineau, K. Naser, S. Puri, F. Le Léannec (InterDigital)

JVET-AJ0259, "EE2-related: On implicit MTS", M. Karczewicz, P. Garus, B. Ray, P. Nikitin (Qualcomm)

## Entropy Coding (2)

JVET-AJ0198, "AHG12: on residual coding in ECM", F. Le LÃ©annec, M. Balcilar, C. Salmon-Legagneur, F. Galpin (InterDigital)

JVET-AJ0222, "Non-EE2: Additional contexts for CABAC", K. Zhang, Z. Deng, M. Salehifar, Y. Wang, W. Yin, N. Zhang, L. Zhao, L. Zhang (Bytedance)

## Inter and Intra (4)

JVET-AJ0078, "EE2-related: Extended overlapped block blending for MV/BV based prediction", Y. Kidani, H. Kato, K. Kawamura (KDDI)

JVET-AJ0196, "AHG12: Range-Exact signaling for adaptive luma clipping", D. Ruiz Coll, J.-K. Lee (Ofinno)

JVET-AJ0237, "AHG12: 12-bit internal bit depth for ECM", R. Yu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)

JVET-AJ0238, "AHG12: OBMC modifications", R. Yu, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm)

## Encoder Optimization, CTC and Requirements (6)

JVET-AJ0226, "AHG12/AHG10: Using CABAC cost for MTT split modes selection", W. Ahmad, P. Wennersten, K. Andersson (Ericsson)

JVET-AJ0236, "Proposed additional requirements for the next generation video coding standard", S. Wenger (Tencent)

JVET-AJ0250, "AHG4/AHG12: New test sequences for ECM exploration", R.-L. Liao, Y. Ye (Alibaba)

JVET-AJ0251, "Proposed timeline and requirements for the next generation video coding standard", Y. Ye (Alibaba), M. Karczewicz (Qualcomm), M.-L. Champel (Xiaomi), P. Onno (Canon), L. Zhang (Bytedance), X. Wang (Kwai), Y. Yu (OPPO), J. Yuan (vivo), Y. Huo (Transsion), A. Luthra (Picsel Labs), E. François (InterDigital), H.-B. Teo (Panasonic), Y. Kidani (KDDI), S.-C. Lim (ETRI), D. Ruiz (Ofinno), F. Chen (Hikvision), W. Husak (Dolby), P. Wetzel (VITEC), J. Shingala (Ittiam), J.-H. Son (WILUS)

JVET-AJ0253, "Comments on setting a timeline for a next generation video coding standard", S. Wenger (Tencent), E. Alshina (Huawei), J. Lemotheux (Orange)

JVET-AJ0305, "AHG4/AHG12: Test Sequences Featuring User-Generated Contents", Z. Deng, Y. Wang, K. Zhang, L. Zhang (Bytedance)

JVET-AJ0321," AHG12: ECM complexity trade-offs", Y.-J. Chang, C.-C. Chen, M. Coban, K. Cui, P. Garus, N. Hu, H. Huang, M. Karczewicz, J.-L. Lin, P.-H. Lin, P. Nikitin, B. Ray, V. Seregin, G. Verba, H. Wang, E. Ye, R. Yu, Y. Zhang, Z. Zhang (Qualcomm)

# Recommendations

The AHG recommends to:

- To review all the related contributions.