| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0041-v1 |
| --- | --- |

| Title: | [AHG11] [AHG14] Teleconference on NNVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, F. Galpin | Email: | elena.alshina@huawei.com franck.galpin@interdigital.com |
| Source: | AhG11 & AhG14 |  |  |

_____________________________

# Abstract

This document summarizes discussions during teleconferences on Neural Network-based Video Coding (NNVC) topic conducted jointly by AhG11 and AhG14 between the JVET-AI and JVET-AJ meetings.

# Introduction

The teleconferences are conducted in order to discuss the NNVC reference software integration and EE1 tests setup.

First teleconference was conducted on August 06, 2024. In total 43 experts attended the discussion.

Agenda of the first teleconference:

- NN-Intra training cross-check report,

- NNVC10.0 software integration status and anchor performance,

- EE1 description finalization.

# Summary of discussion

## NN-Intra training cross-check report

According to 35th meeting of JVET decision JVET-AI0130  (combination of the neural network-based intra prediction mode and ISP) was tentatively adopted. Decision is pending upon availability of training cross-check report.

Only NN-Intra application rules are changed

for the combination of the NN-based intra prediction mode and ISP:

- The luma CB can only be split into 4 equal-sized sub-partitions for both prediction and {transform coding, entropy coding of quantized transform coefficients}. For instance, a 8x8 luma CB predicted via the NN-based intra prediction mode cannot be split vertically into 2 4x8 sub-partitions for prediction and 4 2x8 sub-partitions for {transform coding, entropy coding of quantized transform coefficients}.

- A sub-partition of the luma CB cannot have a dimension strictly smaller than 4. For instance, a 4x16 luma CB predicted via the NN-based intra prediction mode cannot be split vertically via ISP into 4 1x16 sub-partitions.

- At least one of the two dimensions of the luma CB must be equal to 4.

- The luma CB must belong to a CU in separate tree.

Loss-function

Lx, y; θ=SATDy, fh, wx; θh,w+SADy, fh, wx; θh, w

Figure 1 Wide activation in NNVC-9.0

| architecture |  | description |
| --- | --- | --- |
| Number of hidden layers |  | For 16x16, 3. For the other six NNs, 2. |
| Number of neurons per hidden layer |  | For 4x4, 8x4, and 16x16, 1216 for the last hidden layer, and 640 for the other hidden layers. For the other four NNs, 1216. |
| Percentage of non-zero weights | hidden layers | 5% |
|  | layer returning repIdx | For 4x4, 10%. For the other six NNs, 50% |
|  | layer returning the predicted block | 50% |
|  | layer returning grpIdx1 and grpIdx2 | 100% |

⊕

⊕

Figure 1.  NN-Intra description

Cross-checker reported results. Both float point model and quantized results matched perfectly in PSNR and very minor in MS-SSIM. Results are available in up-dated JVET-AI0273.

New NN-Intra 4.8 kMAC/pxl (the worst case), 1.3 M and 0.1% BD-rate gain in both all-intra and random-access configuration.

It is recommended to replace NN-Intra in NNVC-10 with one JVET-AI0130 (adopted). No need to maintain old version of NN-Intra in NNVC SW.

EE1 participants are encouraged to re-generate anchor for accurate time measurement.

## NNVC10.0 SW integration status and anchor performance

All adopted contributions have been integrated. Several new SADL functionalities needed for EE1 tests are under integration. If some of those functionalities are not ready by August, 9 then will be included in NNVC-10.1.

SW release NNVC-10.0 – August 9. Anchor performance results – August 16 (all-intra and random access, low-delay will be added later).

Expectation on NNVC-10.0:

- decoder speed of more than 50% (SADL implementation improvements)

- 0.1% performance improvement due to new NN-Intra.

- NNVC code base will become closer to ECM, in particular GOP structure has been aligned.

JVET-AH0120 proposes new training data set. Partial results of HOP3 retraining have been reported: 0.4% gain in class F (most of new sequences are gaming), minor drop in other classes (HOP3 is best out of three rounds of training, with new training set only single round of training has been performed).

Details of SW integration can be found in attached presentation.

## EE1 description up-date

Several cross-checkers have been added.

# SW location

https://vcgit.hhi.fraunhofer.de/jvet-ai-ee1/VVCSoftware_VTM/-/tree/EE1-X.X