| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0042-v5 |
| --- | --- |

| Title: | AHG10: Teleconference on encoder optimization for multi-layer coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | P. de Lagrange | Email: | philippe.delagrange@interdigital.com |
| Source: | AHG10 |  |  |

_____________________________

# Abstract

This document summarizes discussions during teleconference on encoder optimization for multi-layer coding conducted by JVET AHG10 between the 35th meeting (Sapporo, JP, 12–19 July 2024) and the 36th meeting (Kemer, TR, 1–8 November 2024).

Version 1 was a report of the first teleconference

Version 2 updated software details, with anchor data updated in the attached spreadsheet

Version 3 added reporting of the second teleconference, with cross-checked anchor data in the attached spreadsheet

Version 4 added reporting of the third teleconference, with updated software version (fixing a minor bug) and candidate configuration changes for the enhancement layer

Version 5 added reporting of the fourth teleconference, with results of RDO decision changes mostly solving the degradation issue, and considerations for next steps (preparation of expert viewing)

# Agenda

This teleconference is a follow-up of the one that took place during the previous meeting cycle; this section is copied from the JVET-AI0042 report.

The agenda is related to the AHG mandate related to multi-layer coding:

- Study optimized encoding for reference picture resampling and scalability modes in VTM, and coordinate with AHG4 on improving encoders and test settings for multi-layer verification testing.

The work is related to an issue found during visual tests of spatial scalability in Geneva (31st meeting), where the VTM encoder could create an enhancement layer that degrades visual quality (see JVET-AE0288), introducing coding artefacts that outweigh the benefits of detail enhancement. The situation was improved at the following meeting (see JVET-AF0186 and JVET-AF0311) but still not satisfying.

The goal of the teleconference is to organize encodings and preliminary checks (both objective metrics and informal visual reports) with different configuration and/or new ideas and spread the load, so that meaningful visual tests can be conducted with AHG4/AG5.

The focus is on the first two test categories of the JVET-AH2021 test plan:

- “Spatial scalability”: how does dual-layer compare with adaptive resolution & single-layer (with enhancement layer typically 10% of total bitrate)

- “Quality ladder”: how much can be saved with dual-layer compared to simulcast, when enhancement layer brings significant quality improvement over base layer (typically 50% of total bitrate)

# Discussion

The first teleconference was held on August 20th, 2024, at 15:30h UTC.

The second teleconference was held on September 5th, 2024, at 15:30h UTC.

The third teleconference was held on September 26th, 2024, at 15:30h UTC

The fourth teleconference was held on October 14th, 2024, at 15:30h UTC

## Description of visual quality issues under study

The dual-layer encoding performance issues to be investigated were described in JVET-AI0042: in a nutshell, sometimes, the enhancement layer has lower visual quality than the inter-layer prediction because that prediction is not selected.

Also the R/D performance of the enhancement layer may be significantly lower (visually) than the base layer: for the same total bitrate, the bitrate taken by the enhancement layer means a higher QP for the base layer than a standalone reduced-resolution encoding, with stronger visual artefacts that are not corrected by the enhancement layer (loss of detail in low-contrast areas, pumping effect, etc), resulting in a lower visual quality rating despite of effective detail improvement.

A typical example is shown in Figure 1, where details are better on high-contrast areas (e.g. cars), but low-contrast areas suffer (e.g. trees, road surface in the shade).

Figure 1 – Comparison of QP=26 upscaled with QP=28+35 dual-layer on TallBuildings2 (similar bitrate). Enhancement layer is effective on high-contrast details, but the higher base QP washes out low-contrast detail.

## Comments

Comments raised in the previous teleconference were related to QP adaptation, LMCS, pumping effect, scaling ratio, comparison with SHVC, quantization matrices, MCTF, and deblocking strength.

August 20th: one participant commented that for adaptive QP, we should not go for large local changes (contrary to real-life encoder), to stay close to the flat QP configuration we are used to.

September 5th: visual degradation caused by the enhancement layer compared to upscaled base layer is confirmed to be present with the test conditions described in 2.4, on the CorsairJoy2 example with QP 34+41. Degradation gets worse with increased enhancement layer QP (44), whereas the goal is to converge to a simple upscale (equivalent to doing nothing).

October 14th: considering the progress, next steps could be some offline activity before the meeting to prepare candidates for expert viewing, using RDO fixes and adjusted EL configuration. The new file repository is on https://vqa.lfb.rwth-aachen.de (with different logins for upload and download).

## Test sequences and rate points

For the time being, the focus can be on the sequences and rate points listed in Table 1, especially the two highlighted ones. Other sequences listed in JVET-AH2021 and the attached spreadsheet can be optionally tested.

Depending on experiments, it can be useful to test EL QPs ranging from BL QP+6 to BL QP+10, in addition to the QPs indicated in Table 1 or in JVET-AH2021. For rate alignment purpose, it may also be useful to generate reduced resolution encodings with QP -1 or -2 compared to the base layer of dual-layer encodings (as done for the expert viewing reported in JVET-AE0288).

Table 1 – Selected test sequences and QP settings for viewing reported in JVET-AF0311

| Sequence | QP SL | QP UP | QP BL QP EL |
| --- | --- | --- | --- |
| CorsairJoy | 33 | 27 | 27 34 |
| CorsairJoy | 40 | 34 | 34 41 |
| TallBuildings2 | 34 | 28 | 28 35 |
| TallBuildings2 | 42 | 36 | 36 43 |
| DrivingPOV3Logo | 32 | 27 | 27 34 |
| DrivingPOV3Logo | 38 | 33 | 33 40 |
| DrivingPOV3Logo | 42 | 37 | 37 44 |
| BoxeLogo | 34 | 30 | 30 37 |
| BoxeLogo | 38 | 32 | 32 39 |
| BoxeLogo | 42 | 37 | 37 44 |

The location and md5sum of source videos are found in the attached spreadsheet.

September 5th: it has been noticed during the first teleconference that CorsairJoy contains repeated frames (two occurrences). A new version has then been generated without repeated frames and is called CorsairJoy2: it starts 1 frame earlier and contains one more frame at the end. Since there are more different frames to code, the results are different from CorsairJoy: slightly higher bits and slightly lower PSNR, which is expected. The new reference coding results are provided in the attached spreadsheet. The updated source video is provided as an .mp4 file generated using lossless HEVC compression: the decoded file matches the original YUV md5sum.

## Encoding configuration, tools

Details about running the VTM in scalable configuration are given in the annex of JVET-AH2021 (some of which are repeated here). It is suggested to focus on random-access configuration and 2x scaling ratio for the time being.

The VTM version to be used for new experiments is commit 029f7b12, corresponding to VTM-23.4 + merge request 2747 (that adds a configuration syntax for explicit inter-layer references: see next section) and merge request 2760 (fixing a bug with global variables causing interference of layer 1 into layer 0).

The first step is to downscale source videos for the base layer using TAppDownConvert (see details in JVET-AH2021, section 4.1.1). An example is given below:

TAppDownConvertStatic 3840 2160 TallBuildings2_3840x2160_30fps_10bit_420pf.yuv \
  1920 1080 TallBuildings2_1920x1080_30fps_10bit_420pf.yuv 10 10

Then dual-layer encodings can be run using configuration files found in the cfg/multi-layer folder of the VTM (two_layers_scalable.cfg and layer1_gop_randomaccess.cfg), and sequence-specific configuration files as usual (to specify e.g. source video resolution and frame rate); a “resHD_30.cfg” example is given below:

InputBitDepth      : 10         # Input bitdepth

InputChromaFormat  : 420        # Ratio of luminance to chrominance samples

FrameRate          : 30         # Frame Rate per second

FrameSkip          : 0          # Number of frames to be skipped in input

SourceWidth        : 1920       # Input  frame width

SourceHeight       : 1080       # Input  frame height

FramesToBeEncoded  : 300        # Number of frames to be coded

Level              : 4.1

A UHD configuration file would adjust width and height, and set Level to 5.1.

Example dual-layer VTM encoder command line:

EncoderApp -c encoder_randomaccess_vtm.cfg -c two_layers_scalable.cfg \
  -l0 -q 28 -l0 -c resHD_30.cfg -l0 -i TallBuildings2_1920x1080_30fps_10bit_420pf.yuv \
  -l1 -q 34 -l1 -c resUHD_30.cfg -l1 -i TallBuildings2_3840x2160_30fps_10bit_420pf.yuv \
  -l1 -c layer1_gop_randomaccess.cfg -l1 --Level=6.1 \
  -ip 32 -fs 0 -b TallBuildings2_3840x2160_30fps_10bit_420pf_2834.bin \
  --InternalBitDepth=10 --OutputBitDepth=10 --SEIDecodedPictureHash=1 \
  --DMVREncMvSelect=1 --Verbosity=6 --PrintHexPSNR=1 --PrintMSSSIM=1 -o /dev/null \
  >> TallBuildings2_3840x2160_30fps_10bit_420pf_2834.log

Decoding is performed as usual, but adding “-p 0” to get base output, or “-p 1” to get enhanced output. See notes in section 2.4.2 below to upscale the base layer on the fly.

### GOP-based parameters of enhancement layer (e.g. reference picture list)

Previously, the patch from JVET-AF0186 was applied to put inter-layer reference pictures in the first position in the reference picture lists.

The expected method is now to use explicit configuration as discussed in JVET-AI0248 and provided by MR !2747. This MR includes example configuration files, like cfg/layer1_gop_randomaccess.cfg. The MR is now merged into the VTM master (commit e80e6078).

Beyond controlling inter-layer references, it is also possible to configure specific QP offsets and deblocking strength within the GOP for the enhancement layer (see the FrameX parameters, and section 3.1 “GOP structure table” in the VTM software manual).

### Upscaling the output of the decoder

In order to compare e.g. upscaled base layer with full-resolution anchor or enhancement layer, upscale within the decoder is available in VTM commit 657c28b5 where MR !2686 has been merged.

To upscale the base layer, the decoder parameters to be used are:

DecoderApp -b <input.bin> -o <output.yuv> -p 0 --UpscaledOutput=2
--UpscaledOutputWidth=3840 --UpscaledOutputHeight=2160	
--UpscaleFilterForDisplay=0

### Where -p 0 selects the base layer and --UpscaleFilterForDisplay=0 forces use of the normative (RPR) filter coefficients.

### Option: internal downscale, to avoid generating downscaled source videos

It is possible to perform the downscale on the fly for the base layer instead of providing an externally downscaled source, using the --SourceScalingRatioHor and --SourceScalingRatioVer parameters.

Note: this was not compatible with MCTF on the base layer, which has been fixed in VTM commit cd4ca456, following merge request !2758.

### Option: “DebugBitstream” to avoid recoding the base layer

The --DebugBitstream option is working in recent VTM versions and is very helpful to test different enhancement layer configurations using the same base layer:

- A first encoding is performed, either in dual-layer, or just the base layer in single-layer configuration (leaving room for future EL ALF APS ids using --MaxNumALFAPS=4)

- Then, dual-layer encodings can use -l0 --DebugBitstream=<base-layer.bin> to avoid re-encoding the base layer. This is much faster since the base layer usually takes most of the encoding time.

Example command line:

EncoderApp -c encoder_randomaccess_vtm.cfg -c two_layers_scalable.cfg \
  -l0 --DebugBitstream=TallBuildings2_3840x2160_30fps_10bit_420pf_2834.bin
  -l0 -q 28 -l0 -c resHD_30.cfg -l0 -i TallBuildings2_1920x1080_30fps_10bit_420pf.yuv \
  -l1 -q 35 -l1 -c resUHD_30.cfg -l1 -i TallBuildings2_3840x2160_30fps_10bit_420pf.yuv \
  -l1 -c layer1_gop_randomaccess.cfg -l1 --Level=6.1 \
  -ip 32 -fs 0 -b TallBuildings2_3840x2160_30fps_10bit_420pf_2835.bin \
  --InternalBitDepth=10 --OutputBitDepth=10 --SEIDecodedPictureHash=1 \
  --DMVREncMvSelect=1 --Verbosity=6 --PrintHexPSNR=1 --PrintMSSSIM=1 -o /dev/null \
  >> TallBuildings2_3840x2160_30fps_10bit_420pf_2835.log

### Expected tool improvements

More improvements are expected on a few practical issues:

- There is no tool to compute the MS-SSIM metric from YUV files, so this metric is not available to compare upscaled anchor to video source. A workaround is to use the ssim/ms-ssim metric from the VMAF tool (for all cases). Extracting the MS-SSIM computation from the VTM to make it available independently of the encoder would be helpful.

- Similarly,

  - Upscale from YUV is not officially available as a separate tool, but a patch exists.

  - Downscale of the source (to encode base layer) is performed using the TAppDownConvert tool from the SHM.

These features could be grouped into one rescale tool, made available in the VTM.

# Experiments

## List of experiments

| Test | Volunteer |
| --- | --- |
| Adaptive QP - Both layers - Enhancement layer only (optional) | F. Bossen to try the functionality available in the VTM and report if it brings visual benefits |
| LMCS - Disabled, on both anchor and test - Both layers | O. Chubach |
| Pumping analysis - Check per-picture rate and distortion for anomalies (compared to base layer or single layer) | P. de Lagrange |
| RDO related to inter-layer prediction - Optimize to avoid degrading base layer quality | InterDigital |
| BL/EL QP offset - Consider 10% EL rate instead of +7QP | The EL ratio is available in the spreadsheet. |
| MCTF - Disable on base layer | O. Chubach |
| Deblocking - Increase strength on enhancement layer | K. Andersson |
| DMVR - Disable, either on both layers or just enhancement layer. - Objective metrics and visual outcome compared to --DMVREncMvSelect=1 |  |
| Position of inter-layer reference - Play with the position depending on Tid | K. Andersson |
| QP offsets within GOP (for enhancement layer) | K. Andersson |
| Quantization matrices - Select some from JVET contributions | InterDigital |

A spreadsheet is attached to this document and is to be used to report tests performed and objective metrics. It contains details about test configuration and test sequences (including md5sums for source, downscales and upscales), and example reporting tabs.

Results of experiments are collected in successive versions of this document (or attachments).

## Results

### Cross-check of anchors

September 5th: encodings of the sequences described in 2.3 with the configuration described in 2.4 are available and metrics are reported in the tabs “VTM 23.4 (Full-res)” and “VTM 23.4 (Dual-layer, 2x)” of the attached spreadsheet. Cross-checks have been done between InterDigital and Ericsson, and results matched after fixing some configuration mistakes on both sides.

September 26th: a bug causing minor base layer RDO instability with Marathon2 has been fixed in MR 2760 and merged in commit 029f7b12. The anchors will be checked again, with very little impact expected.

### October 14th: new anchors with commit 029f7b12 are available in the spreadsheet, and now the base layer is always the same for the same QP. A bitrate difference of 0.1 to 1% has been observed on sequences that are using LMCS (DrivingPOV3 0.5%, Fortnite_Part1 1%, Marathon2 0.1%, Procession 0.3%, CatRobot1 0.2%, LupoPuppet 0.4%, StarCraft 1%); BD-rate impact has not been checked.

### Local QP adaptation

F. Bossen verbally reported that the --qpa parameter works in dual-layer configuration and seems to help visually. The legacy method has not been tested yet.

### LMCS off

Bitstreams are available on the FTP site but were not analyzed yet.

### RDO related to inter-layer prediction

September 5th:

The first question is if visual defects correlate with local distortion metrics, meaning that in those cases the coding mode chosen for the enhancement layer actually increases distortion compared to the upscaled base layer. The answer is yes, as shown by local PSNR maps on the CorsairJoy2 example; Figure 2 illustrates the degradation, that gets worse with increased EL QP.

Figure 2 – Local PSNR maps of CorsairJoy2 at POC22.
Top-left = luma image, Top-right = local PSNR of upscaled base layer (QP 34).
Bottom = local PSNR difference of EL vs BL (left: QP 41, right: QP 44).

September 26th:

Progress on coding mode selection analysis is reported, but with no fix yet.

October 14th:

A fix is being tested that selects inter-layer modes when tested against a non-inter-layer mode and distortion is lower (regardless of cost). In addition, at least one inter-layer mode is kept in the first SATD-based pruning pass for merge/skip candidates. Analysis of inter-layer mode selection and PSNR maps (see Figure 3) tend to indicate that the degradation issue is solved or mostly solved.

Figure 3 – RDO fix: local PSNR maps of CorsairJoy2 at POC22.
Local PSNR difference of EL vs QP-34 BL (left: QP 41, right: QP 44).

Full simulation results are available in the attached spreadsheet: PSNR and MS-SSIM is generally higher, with a moderate increase of bitrate. BD-rates have not been compared. Visual checks on a few pictures are encouraging, but are yet to be done on a broader scale.

### EL bitrate ratio

September 5th: a column (BitRatio1) has been added to the reporting spreadsheet, computing the EL/(BL+EL) ratio.

### MCTF off

September 5th: encodings are almost ready (missing CatRobot and HD), but with VTM-23.3+MR!2747. They will be regenerated or cross-checked using VTM-24.3+MR!2747+MR!2760 (commit 029f7b12).

### GOP structure for EL

September 26th:

Lowering the intra picture QP (-4 instead of -3) for enhancement layer leads to visual and BD-rate improvement (better details, around -0.4%).

Removing inter-layer picture from list 1 (but still 1st position in list 0) gives both visual and BD-rate improvements (also around -0.4% and better preserving details).

Deblocking offset: tried removing the bias for enhancement layer (0 instead of -2 = more deblocking), which seems to bring visual improvement and very minor BD-Rate impact.

October 14th:

BD-rate using a 7 QP offset between base and enhancement layer are reported in attached spreadsheets, for those 3 configuration changes, individually and in combination.

### Quantization matrices for EL

October 14th: a few candidate matrices have been identified, but could not be tested yet because of VTM bugs in dual-layer context. Bugs are now fixed (no merge request yet, pending visual checks).

# Closing notes

The first teleconference closed at 16:40 UTC after planning a second one on September 5th at 15:30 UTC.

The second teleconference closed at 16:00 UTC after planning a third one on September 26th at 15:30 UTC.

The third teleconference closed at 16:10 UTC after planning a fourth one on October 14th at 15:30 UTC.

The fourth teleconference closed at 16:10 UTC.

# Participants

|  | Aug. 20th | Sept. 5th | Sept. 26th | Oct. 14th |
| --- | --- | --- | --- | --- |
| Kenneth Andersson (Ericsson - SE) |  | x | x | x |
| Olena Chubach (MediaTek - US) | x | x | x |  |
| Lorenzo Ciccarelli (V-Nova - UK) | x |  | x | x |
| Alberto Duenas (Warner Bros. Discovery - US) |  | x |  |  |
| Philippe de Lagrange (InterDigital - FR) | x | x | x | x |
| Jens-Rainer Ohm (RWTH - DE) | x | x | x |  |
| Charles Salmon-Legagneur (InterDigital - FR) |  | x |  |  |
| Fabrice Urban (InterDigital - FR) | x | x |  | x |
| Mathias Wien (RWTH - DE) | x | x | x | x |
| Ping Wu (ZTE - UK) | x |  |  | x |
| Yin Zhao (Huawei - CN) |  | x |  | x |
| Hilmi E. Egilmez (Apple - US) |  |  | x |  |
| Mikael Le Pendu (InterDigital - FR) |  |  | x | x |
| Johannes Sauer (Huawei - DE) |  |  |  | x |
| Gary Sullivan (SC29 chair) |  |  |  | x |