| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0043-v2 |
| --- | --- |

| Title: | Template Matching Picture Boundary Padding in ECM 14 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nicolas Neumann (RWTH) Mathias Wien (RWTH) | Tel: Email: | Nicolas.Neumann@lfb.rwth-aachen.de Mathias.Wien@lfb.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This contribution proposes template matching padding for reference picture boundary handling. Template matching structure prediction is used to generate the outside samples of the reference picture from content inside the reference picture. For this purpose, the outside area is partitioned into so-called target blocks. Then a template matching search is performed for each target block and the search results are used to fill the target block. The method can be used as a replacement for motion compensated padding (JVET-AA0096) or can be combined with this method. The presented combination first applies motion compensated padding and then fills the remaining area with template matching boundary padding instead of repetitive padding. This document provides results with ECM 14 as the anchor:

TM-Padding

RA: {0.08%, 0.01%, -0.06%; 100.0%, 99.9%};

LDB: {-0.09%, 0.07% -0.68%; 100.4%, 100.2%};

LDP: {-0.08%, 0.19% -0.27%; 99.2%, 100.2%}.

MC-TM-Padding:

RA: {-0.02%, -0.04%, -0.01%; 99.2%, 100.4%};

LDB: {0.01%, 0.03%, -0.29%; 99.4%, 100.6%};

LDP: {-0.05%, -0.01%, 0.07%; 98.6%, 100.2%}.

It is proposed to study the methods in an EE.

# Introduction

In VTM repetitive padding is used for reference picture padding. For ECM motion compensated padding [1] was proposed which yields improved compression efficiency. Motion compensated padding (MC-Padding, JVET-AA0096) uses content from other reference pictures to pad the current reference picture for applicable areas. The remaining outside area is still padded with repetitive padding.

The proposed method describes a reference picture padding algorithm, which is applicable for the whole outside area of the reference picture and can be combined with motion compensated padding to substitute the repetitive padding.

In the following results will be provided for:

- Template Matching Padding (TM-Padding: Template Matching instead of TM-Padding)

- Combination Padding (MC-TM-Padding: Combination of MC and TM-Padding)

# Method

The method is described in JVET-AH0168 in detail. In the simulations reported in this document, the stopping criterion was changed compared to the previous proposal as follows: If one of the candidates in the search area has an SAD value of 128 or lower, the search is terminated at the current search step. The previous value was 24. This is done to reduce the decoder runtime increase further. ECM 14 is used as the anchor for all the simulations.

# Results

The following tables show the results for two methods. Due to problems with the simulation cluster only partial results can be delivered at this point in time. The data of the still open simulations are filled the values from the Crosscheck JVET-AJ0044 and marked in blue. Runtime and peak memory data is completely taken from the crosscheck to have homogeneous data for the average values.

## TM-Padding

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.08% | -0.14% | -0.07% | 100.0% | 100.0% | 99.7% | 100.8% |
| Class A2 | 0.10% | 0.00% | 0.02% | 100.1% | 99.9% | 99.9% | 100.5% |
| Class B | 0.06% | -0.02% | 0.02% | 99.7% | 100.1% | 99.6% | 99.5% |
| Class C | 0.08% | 0.17% | 0.22% | 100.3% | 99.6% | 98.9% | 98.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.08% | 0.01% | 0.06% | 100.0% | 99.9% | 99.5% | 99.6% |
| Class D | 0.21% | -0.02% | 0.34% | 100.1% | 99.3% | 99.2% | 96.2% |
| Class F | 0.05% | 0.03% | 0.03% | 100.0% | 99.5% | 99.4% | 98.5% |
| Class TGM | -0.02% | 0.05% | -0.04% | 100.3% | 100.2% | 100.2% | 99.3% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.03% | -0.42% | 100.5% | 100.2% | 99.7% | 99.7% |
| Class C | -0.09% | 0.21% | -0.46% | 100.2% | 99.9% | 99.7% | 97.3% |
| Class E | -0.13% | 0.06% | -1.39% | 100.5% | 100.8% | 100.5% | 98.2% |
| Overall | -0.09% | 0.07% | -0.68% | 100.4% | 100.2% | 99.9% | 98.5% |
| Class D | -0.06% | 0.53% | 0.25% | 100.6% | 99.7% | 99.1% | 96.2% |
| Class F | -0.38% | -0.63% | 0.08% | 110.2% | 101.4% | 98.9% | 98.6% |
| Class TGM | 0.05% | 0.03% | 0.21% | #ZAHL! | #ZAHL! | #ZAHL! | #ZAHL! |

|  | Low delay P Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,08% | 0,22% | -0,04% | 99,6% | 100,0% | 99,8% | 97,1% |
| Class C | -0,11% | 0,20% | -0,63% | 98,5% | 100,4% | 100,9% | 96,2% |
| Class E | -0,04% | 0,14% | -0,18% | 99,7% | 100,4% | 98,7% | 96,7% |
| Overall | -0,08% | 0,19% | -0,27% | 99,2% | 100,2% | 99,9% | 96,7% |
| Class D | -0,27% | -1,02% | -0,04% | 99,2% | 99,0% | 100,1% | 96,1% |
| Class F | 0,12% | -0,05% | 0,14% | 99,0% | 100,2% | 99,5% | 97,9% |
| Class TGM | 0,00% | -0,08% | 0,01% | 98,7% | 100,2% | 101,4% | 100,5% |

## MC-TM-Padding

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | -0.14% | -0.20% | 99.0% | 100.8% | 99.9% | 99.8% |
| Class A2 | -0.01% | 0.05% | -0.11% | 99.0% | 100.3% | 99.8% | 99.6% |
| Class B | -0.02% | -0.01% | 0.08% | 99.2% | 100.4% | 100.0% | 99.7% |
| Class C | -0.04% | -0.05% | 0.10% | 99.4% | 100.1% | 99.7% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.01% | 99.2% | 100.4% | 99.9% | 99.7% |
| Class D | -0.02% | -0.14% | 0.29% | 99.2% | 100.2% | 100.0% | 100.0% |
| Class F | -0.01% | -0.03% | 0.17% | 99.2% | 101.0% | 100.3% | 99.9% |
| Class TGM | -0.03% | 0.07% | -0.02% | 100.0% | 100.4% | 100.4% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.09% | 0.00% | 99.1% | 100.1% | 100.0% | 99.8% |
| Class C | -0.01% | 0.03% | -0.48% | 99.7% | 100.5% | 99.6% | 100.0% |
| Class E | 0.00% | -0.07% | -0.53% | 99.6% | 101.6% | 100.8% | 100.3% |
| Overall | 0.01% | 0.03% | -0.29% | 99.4% | 100.6% | 100.1% | 100.0% |
| Class D | -0.21% | 0.17% | 0.00% | 100.8% | 100.0% | 100.1% | 100.0% |
| Class F | -0.48% | -0.94% | 0.43% | 109.4% | 102.1% | 99.6% | 100.0% |
| Class TGM | -0.02% | 0.03% | 0.13% | #ZAHL! | #ZAHL! | #ZAHL! | #ZAHL! |

|  | Low delay P Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,05% | 0,39% | -0,06% | 99,1% | 100,2% | 100,0% | 99,9% |
| Class C | -0,03% | 0,24% | -0,25% | 97,8% | 100,0% | 100,7% | 99,9% |
| Class E | -0,06% | -1,02% | 0,70% | 98,7% | 100,4% | 99,0% | 100,1% |
| Overall | -0,05% | -0,01% | 0,07% | 98,6% | 100,2% | 100,0% | 99,9% |
| Class D | -0,14% | -1,13% | -0,86% | 98,8% | 101,2% | 100,6% | 100,0% |
| Class F | 0,28% | -0,18% | 1,46% | 98,5% | 100,2% | 101,6% | 100,0% |
| Class TGM | -0,02% | 0,03% | -0,10% | 98,3% | 98,7% | 100,4% | 99,9% |

# Conclusion

It can be seen, that the TM-Padding while the method does not improve compression efficiency for the Random Access case, an average luma gain of -0.09% BD-rate can be observed for the Low Delay B configuration and -0.08% BD-rate for Low Delay P with a decoder runtime increase of 100.2% on average for both configurations. The gains are consistent across classes for Low Delay B except for TGM, where a slight loss is observed. For Low Delay P only a loss for class F is observed. We therefore propose to study the method in an Exploration Experiment as an alternative method to MC-padding for low delay applications.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).