| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0044-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0043 (Template Matching Picture Boundary Padding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Sebastian Schwarz | Email: | sebastian.schwarz@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution reports the results of crosscheck of JVET-AJ0043 proposed by RWTH Aachen. The Method is described in JVET-AH0168 in detail. The following crosschecks were performed:

- MC-TM-Padding: Combination of MC- and TM-Padding.

- TM-Padding: Template Matching instead of MC-Padding (JVET_AA0096_MC_BOUNDARY_PADDING=0).

# Verification

Source code with changes on the top of ECM-14.0 was provided by email on 09.09.2024 (authored by RWTH Aachen). As JVET-AJ0043 does not provide full results as of now, we present our internal simulation results below. We will update this document as soon as JVET-AJ0043 provides full results.

To estimate EncT/DecT in similar conditions, the anchor and the test are simulated on a homogenous cluster at full load. Only RA and LB configurations are reported.

MC-TM-Padding

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | -0.14% | -0.20% | 99.0% | 100.8% | 99.9% | 99.8% |
| Class A2 | -0.01% | 0.05% | -0.11% | 99.0% | 100.3% | 99.8% | 99.6% |
| Class B | -0.02% | -0.01% | 0.08% | 99.2% | 100.4% | 100.0% | 99.7% |
| Class C | -0.04% | -0.05% | 0.10% | 99.4% | 100.1% | 99.7% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.01% | 99.2% | 100.4% | 99.9% | 99.7% |
| Class D | -0.02% | -0.14% | 0.29% | 99.2% | 100.2% | 100.0% | 100.0% |
| Class F | -0.01% | -0.03% | 0.17% | 99.2% | 101.0% | 100.3% | 99.9% |
| Class TGM | -0.03% | 0.07% | -0.02% | 100.0% | 100.4% | 100.4% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.09% | 0.00% | 99.1% | 100.1% | 100.0% | 99.8% |
| Class C | -0.01% | 0.03% | -0.48% | 99.7% | 100.5% | 99.6% | 100.0% |
| Class E | 0.00% | -0.07% | -0.53% | 99.6% | 101.6% | 100.8% | 100.3% |
| Overall | 0.01% | 0.03% | -0.29% | 99.4% | 100.6% | 100.1% | 100.0% |
| Class D | -0.21% | 0.17% | 0.00% | 100.8% | 100.0% | 100.1% | 100.0% |
| Class F | -0.48% | -0.94% | 0.43% | 109.4% | 102.1% | 99.6% | 100.0% |
| Class TGM | -0.02% | 0.03% | 0.13% | #NUM! | #NUM! | #NUM! | #NUM! |

TM-Padding

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.08% | -0.14% | -0.07% | 100.0% | 100.0% | 99.7% | 100.8% |
| Class A2 | 0.10% | 0.00% | 0.02% | 100.1% | 99.9% | 99.9% | 100.5% |
| Class B | 0.06% | -0.02% | 0.02% | 99.7% | 100.1% | 99.6% | 99.5% |
| Class C | 0.08% | 0.17% | 0.22% | 100.3% | 99.6% | 98.9% | 98.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.08% | 0.01% | 0.06% | 100.0% | 99.9% | 99.5% | 99.6% |
| Class D | 0.21% | -0.02% | 0.34% | 100.1% | 99.3% | 99.2% | 96.2% |
| Class F | 0.05% | 0.03% | 0.03% | 100.0% | 99.5% | 99.4% | 98.5% |
| Class TGM | -0.02% | 0.05% | -0.04% | 100.3% | 100.2% | 100.2% | 99.3% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.03% | -0.42% | 100.5% | 100.2% | 99.7% | 99.7% |
| Class C | -0.09% | 0.21% | -0.46% | 100.2% | 99.9% | 99.7% | 97.3% |
| Class E | -0.13% | 0.06% | -1.39% | 100.5% | 100.8% | 100.5% | 98.2% |
| Overall | -0.09% | 0.07% | -0.68% | 100.4% | 100.2% | 99.9% | 98.5% |
| Class D | -0.06% | 0.53% | 0.25% | 100.6% | 99.7% | 99.1% | 96.2% |
| Class F | -0.38% | -0.63% | 0.08% | 110.2% | 101.4% | 98.9% | 98.6% |
| Class TGM | 0.05% | 0.03% | 0.21% | #NUM! | #NUM! | #NUM! | #NUM! |

# Patent rights declaration(s)

Nokia does not have any current or pending patent rights relating to the technology described in this contribution.