| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0047-v1 |
| --- | --- |

| Title: | AHG9: Merging NNPF filtering process into processing chain handling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Jizheng Xu1, Li Zhang1, Yue Li2, Kaifa Yang2, Yiling Xu2 1Bytedance Inc., US 2Shanghai Jiao Tong University, China | Tel: Email: | yekui.wang@bytedance.com xujizheng@bytedance.com lizhang.idm@bytedance.com yueli983@sjtu.edu.cn sekiroyyy@sjtu.edu.cn yl.xu@sjtu.edu.cn |
| Source: | Bytedance Inc., Shanghai Jiao Tong University |  |  |

_____________________________

# Abstract

Subclause 8.28.1 specifies the general post-processing filtering process using NNPFs. However, with the introduction of the process for handling of a processing chain in subclause 8.30.3, which is a superset of the former in functionality, the two processes are asserted largely overlapping with each other with redundancies.

This contribution proposed to merge the general post-processing filtering process using NNPFs in subclause 8.28.1 into the process for handling of a processing chain in subclause 8.30.3.

The detailed proposed text changes, marked relative to the proposed text in JVET-AJ0046-v1, are included in an attachment to this contribution.

# An explanation of subclause 8.28.1 coverage in subclause 8.30.3

An explanation of the coverage of subclause 8.28.1 in subclause 8.30.3 (with the proposal in this contribution) is provided below.

| Subclause 8.28.1 | Subclause 8.30.3 (with the proposal in this contribution) |
| --- | --- |
| ### 8.28.1 General post-processing filtering process using NNPFs #### 8.28.1.1 General Input to this process is a bitstream BitstreamToFilter. Output of this process is a list of NNPF output pictures ListNnpfOutputPics. First, BitstreamToFilter is decoded, and the list CroppedDecodedPictures is set to be the list of the cropped decoded pictures in output order resulted from decoding BitstreamToFilter. Second, the filtering process for one picture, as specified in clause 8.28.1.2, is repeatedly invoked, in output order, for each cropped decoded picture that is in CroppedDecodedPictures and for which one or more NNPFs are activated. The order of the pictures in ListNnpfOutputPics is in output order. Within ListNnpfOutputPics there shall be no more than one picture pertaining to any particular output time instance. When for any particular picture in CroppedDecodedPictures there are multiple NNPFs activated and only one of the NNPFs is allowed to be chosen to be applied although any of the NNPFs may be chosen, the above constraint shall apply regardless of which NNPF is chosen to be applied to the particular picture. For any particular pair of pictures inputPicA and inputPicB consecutive in output order in CroppedDecodedPictures, when there are one or more pictures intermediatePicSetA in ListNnpfOutputPics between inputPicA and inputPicB in output order, one and only one of the following shall apply: – The pictures in intermediatePicSetA shall be among the pictures that were output by applying a particular NNPF nnpfA with PictureRateUpsamplingFlag equal to 1 when a particular picture currPicA in CroppedDecodedPictures was the current picture. – The pictures in intermediatePicSetA shall be among the pictures that were output by applying a particular NNPF nnpfA with TemporalExtrapolationFlag equal to 1 when a particular picture currPicA in CroppedDecodedPictures was the current picture. The application of any other NNPF that was used in the filtering process for one picture when currPicA was the current picture or the application of any NNPF (including nnpfA) that was used in the filtering process for one picture when any other picture currPicB in CroppedDecodedPictures was the current picture shall not output any picture between the inputPicA and inputPicB in output order. NOTE – The intent of the constraints expressed in the above paragraph is to disallow generating NNPF output pictures between any particular pair of consecutive input pictures more than once. #### 8.28.1.2 Filtering process for one picture using an NNPF The filtering process specified in this clause applies to each cropped decoded picture, referred to as the current picture, that is in CroppedDecodedPictures and for which one or more NNPFs are activated. When applying an NNPF to the current picture, the following applies: – The filtered and/or interpolated pictures are generated by the NNPF by applying the NNPF process specified in the semantics of the NNPFC SEI message, in a patch-wise manner, to the current picture. – The order of the pictures generated by the NNPF by applying the NNPF process being stored into the output tensor of the NNPF is in output order. When the applied NNPF is the last NNPF that is applied to the current picture, the pictures generated by the NNPF and output by the NNPF process are included into ListNnpfOutputPics, in the same order as when the pictures are stored into the output tensor of the NNPF. | - Handling of a processing chain - General Processing chains are alternatives to each other, i.e., at most one processing chain can be chosen to be applied by a decoding system at one time. A special NNPF cascading case is defined as the case when such two NNPFs are both activated for a picture: one of the two NNPFs has nnpfc_purpose equal to 4 and the other has multiple input pictures, and neither of the two NNPFs is associated with an SPO SEI message. In this case, the two NNPFs are implicitly considered as belonging to one processing chain and the processing chain only contains these two NNPFs. Except for the special NNPF cascading case, each processing chain containing multiple SEI message types is indicated by an SPO SEI message with a particular value of po_id. Except for the special NNPF cascading case, any SEI message for which the payloadType is present in SpoProcessSeiList but is not indicated by an SPO SEI message is in its own processing chain. In the special NNPF cascading case, po_breadth_first_flag is inferred to be equal to 0, PoNumProcStgs is set equal to 2, PoNumSeiMsgs is set equal to 2, and the following applies: - If both of the NNPFs have nnpfc_purpose equal to 4 and multiple input pictures, either of the two is chosen to be applied first, and for the one chosen to be applied first, PoProcStgIdx[ i ] is set equal to 0, and PoSeiList[ 0 ] is set to be the NNPFA SEI message activating the NNPF, and for the other, PoProcStgIdx[ i ] is set equal to 1, and PoSeiList[ 1 ] is set to be the NNPFA SEI message activating the NNPF. - Otherwise, PoProcStgIdx[ i ] corresponding to the NNPF with nnpfc_purpose equal to 4 is set equal to 0, PoProcStgIdx[ i ] corresponding to the NNPF with multiple input pictures is set equal to 1, PoSeiList[ 0 ] is set to be the NNPFA SEI message activating the NNPF with nnpfc_purpose equal to 4, and PoSeiList[ 1 ] is set to be the NNPFA SEI message activing the NNPF with multiple input pictures. In case of a single SEI message in its own processing chain, po_breadth_first_flag is inferred to be equal to 0, PoNumProcStgs is set equal to 1, PoNumSeiMsgs is set equal to 1, PoProcStgIdx[ 0 ] is set equal to 0, and PoSeiList[ 0 ] is set to be the single SEI message. The PoSeiList for a corresponding picture of picA or for an associated inserted picture of picA is derived to be the same as the PoSeiList derived for picA. A decoding system may choose and apply a processing chain as follows: - The bitstream is decoded, a processing chain is chosen, and the following applies: – The list PoCdoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream. – When the SEI message type indicated by PoSeiTypeIdx[ 0 ] of the chosen processing chain corresponds to an FGC SEI message, the list PoDecPicList is set to be the list of the decoded pictures in output order resulted from decoding the bitstream. - If the SEI message type indicated by PoSeiTypeIdx[ 0 ] of the chosen processing chain does not correspond to an FGC SEI message, the list CandInputPicList[ 0 ] is set to be identical to PoCdoPicList. Otherwise, the list CandInputPicList[ 0 ] is set to be identical to PoDecPicList. - For each i in the range of 1 to PoNumProcStgs, inclusive, the list CandInputPicList[ i ] is set to be identical to PoCdoPicList. NOTE 1 – The lists CandInputPicList[ i ] for i in the range of 1 to PoNumProcStgs, inclusive, may be updated during the next step. After step 4 below, the list CandInputPicList[ PoNumProcStgs ] stores the final output of the chosen processing chain. - If the chosen processing chain is indicated by an SPO SEI message and po_breadth_first_flag in the SPO SEI message is equal to 1, the breadth-first handling of a processing chain specified in clause 8.30.3.2 is invoked. Otherwise, either the depth-first handling of a processing chain specified in clause 8.30.3.3 or the breadth-first handling of a processing chain specified in clause 8.30.3.2 is invoked. – When applying an NNPF to a picture during the invocation of a process for handling of a processing chain, the following applies: – The filtered and/or interpolated pictures are generated by the NNPF by applying the NNPF process specified in the semantics of the NNPFC SEI message, in a patch-wise manner, to the current picture. – The order of the pictures generated by the NNPF by applying the NNPF process being stored into the output tensor of the NNPF is in output order. - The list PoOutputPicList is set to be identical to CandInputPicList[ PoNumProcStgs ]. The pictures in PoOutputPicList shall be in increasing output order. NOTE 2 – This covers that within PoOutputPicList there are no two pictures having the same output order. For each processing stage with processing stage index i in the range of 0 to PoNumProcStgs − 1, inclusive, the following applies: – For any particular pair of pictures inputPicA and inputPicB consecutive in output order in CandInputPicList[ i ], which is the list of candidate input pictures for the processing stage, when there are one or more pictures intermediatePicSetA between inputPicA and inputPicB in output order added to CandInputPicList[ i + 1 ] when applying the process implied by a particular SEI message of the processing stage, one and only one of the following shall apply: – The pictures in intermediatePicSetA shall be among the pictures that were output by applying a particular NNPF nnpfA with PictureRateUpsamplingFlag equal to 1 of the processing stage when a particular picture currPicA in CandInputPicList[ i ] was the current picture. – The pictures in intermediatePicSetA shall be among the pictures that were output by applying a particular NNPF nnpfA with TemporalExtrapolationFlag equal to 1 of the processing stage when a particular picture currPicA in CandInputPicList[ i ] was the current picture. – The application of the process implied by the particular SEI message when another picture other than currPicA was the current picture or the application of the process implied by of another SEI message of the same processing stage when any picture (including currPicA) was the current picture shall not output any picture between the inputPicA and inputPicB in output order. NOTE 2 – The intent of the constraints expressed above is to disallow generating output pictures between any particular pair of consecutive input pictures more than once within a processing stage. ... - Depth-first handling of a processing chain The following is repeatedly applied, in output order, for each picture picA in CandInputPicList[ 0 ]: – The following applies for each SEI message with SEI message index seiIdx in PoSeiList of picA in an increasing order of list indexes for PoSeiList: – When PoProcStgIdx[ PoSeiTypeList[ seiIdx ] ] is greater than 0, the following applies for the interpretation of the SEI message: – The interface variables for purposes of interpretation of the SEI message are derived from the pictures in CandInputPicList[ PoProcStgIdx[ PoSeiTypeList[ seiIdx ] ] ]. – The semantics of the SEI message, or of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in CandInputPicList[ PoProcStgIdx[ PoSeiTypeList[ seiIdx ] ] ]. – When the SEI payloadType value of the SEI message is present in SpoProcessSeiList, the process implied by the SEI message is invoked repeatedly, in output order, for each of the pictures in CandInputPicList[ PoProcStgIdx[ PoSeiTypeList[ seiIdx ] ] ] that is the corresponding picture of picA or an associated inserted picture of picA. After each invocation of the process, each list CandInputPicList[ PoProcStgIdx[ PoSeiTypeList[ seiIdx ] ] + j ] with j in the range of 1 to PoNumProcStgs − PoProcStgIdx[ PoSeiTypeList[ seiIdx ] ], inclusive, is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into CandInputPicList[ PoProcStgIdx[ PoSeiTypeList[ seiIdx ] ] + j ] so that the output order is obeyed. When the SEI message is an FGC SEI message, PoProcStgIdx[ PoSeiTypeList[ seiIdx ] ] is equal to 0, and j is equal to 1, during the replacement of a picture in CandInputPicList[ 1 ] with the corresponding processed picture, the corresponding processed picture is first cropped, in the same manner as generating a cropped decoded output picture from the corresponding decoded picture, and the cropped picture is used for the replacement. |

# Patent rights declaration(s)

Bytedance Inc. and Shanghai Jiao Tong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).