| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0048-v1 |
| --- | --- |

| Title: | AHG9: On SeiProcessingOrderSeiList and SpoProcessSeiList |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Jizheng Xu1, Li Zhang1, Yue Li2, Kaifa Yang2, Yiling Xu2 1Bytedance Inc., US 2Shanghai Jiao Tong University, China | Tel: Email: | yekui.wang@bytedance.com xujizheng@bytedance.com lizhang.idm@bytedance.com yueli983@sjtu.edu.cn sekiroyyy@sjtu.edu.cn yl.xu@sjtu.edu.cn |
| Source: | Bytedance Inc., Shanghai Jiao Tong University |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on SeiProcessingOrderSeiList and SpoProcessSeiList:

- Remove the SEI payloadType value 210 from SeiProcessingOrderSeiList and SpoProcessSeiList, i.e., disallow an SPO SEI message to include an SEI message type corresponding to an NNPFC SEI message.

- Add the payloadType value 45, the SEI payloadType value of the frame packing arrangement (FPA) SEI message, to SeiProcessingOrderSeiList (but not to SpoProcessSeiList).

- Add the payloadType value 217, the SEI payloadType value of the object mask information (OMI) SEI message, to SeiProcessingOrderSeiList (but not to SpoProcessSeiList).

- Add the payloadType value 218, the SEI payloadType value of the modality information (MI) SEI message, to SeiProcessingOrderSeiList (but not to SpoProcessSeiList).

- Revise the note about the EOI SEI message to avoid mentioning inclusion of an EOI SEI message in a processing chain.

# Problem statement

## General

The list SeiProcessingOrderSeiList includes the SEI payloadType values of those SEI messages that are allowed to be indicated in an SPO SEI message. The list SpoProcessSeiList includes the SEI payloadType values of those SEI messages that are allowed to be indicated in an SPO SEI message and implying a post-filtering process. Currently, these two lists are specified in JVET-AI2005 as follows:

–	The list SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 137, 142, 144, 147, 148, 149, 150, 153, 155, 165, 177, 210, and 211.

–	The list SpoProcessSeiList is set to consist of the payloadType values 19, 142, 155, 210, and 211.

## On the SEI payloadType value of the NNPFC SEI message

The lists SeiProcessingOrderSeiList and SpoProcessSeiList both include the SEI payloadType value 210, which is the SEI payloadType value of the NNPFC SEI message. However, an NNPF can only be activated by an NNPFA SEI message, therefore in an SPO SEI message it would never be useful to indicate an SEI message type corresponding to an NNPFC SEI message. Therefore, the SEI payloadType value of the NNPFC SEI message should not be included in these lists.

## On the SEI payloadType value of the FPA SEI message

The lists SeiProcessingOrderSeiList and SpoProcessSeiList both do not include the SEI payloadType value 45, the SEI payloadType value of the FPA SEI message. However, the FPA SEI message can actually indicate that a de-packing process, in a similar nature as the region-wise packing (RWP) SEI message, for which the SEI payloadType value 155 is included in both SeiProcessingOrderSeiList and SpoProcessSeiList. Furthermore, in omnidirectional video bitstreams, FPA SEI messages and RWP are often used together. However, on the other hand, the depacking process of the FPA SEI message is to generate pictrues of different views, which in a sense is closer to the projection process of the equirectangular projection (ERP) SEI message or the generalized cubemap projection (GCMP) SEI message, which is to generate a sphere signal from the 2D projected picture. Since the the payload type values, 150 and 153, for the ERP and GCMP SEI messages, respectively, are both included in SeiProcessingOrderSeiList but not in SpoProcessSeiList, it seems to make sense to do the same for the FPA SEI message.

## On the SEI payloadType values of the OMI, MI, and EOI SEI messages

Both the OMI and MI SEI messages provide properties of a picture, which may be changed after a post processing stage, e.g. an NNPF may be trained to convert modility from one to another for a picture. Thus, it makes sense to add the SEI payloadType values of both the OMI and MI SEI messages to SeiProcessingOrderSeiList to allow them to be inidcated by an SPO SEI message.

One related question is about the encoder optimization information (EOI) SEI message. There is a NOTE in page 30 of JVET-AI2006, as follows:

"NOTE 3 – The values of po_for_human_viewing_idc and po_for_machine_analysis_idc are in force for the output of a processing chain instead of the respective syntax elements in an encoder optimization information (EOI) SEI message (eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc) or a neural-network post-filter characteristics (NNPFC) SEI message (nnpfc_for_human_viewing_idc and nnpfc_for_machine_analysis_idc), when the EOI or NNPFC SEI message is associated with the processing chain."

From the NOTE, it seems that the EOI SEI can be associated with a processing chain. However, it is also noticed that there is the following description:

"The encoder optimization information SEI message is used to indicate if the video has been optimized for human viewing or machine analysis and which types of optimization have been applied in pre-processing or encoding."

From the description, the EOI SEI is about the information of pre-processing and/or encoding. Thus, it does not seem to be suitable to be associated with a post-processing chain.

# Proposal

The proposal for resolving the above problems is summarized in the abstract above.

# Patent rights declaration(s)

Bytedance Inc. and Shanghai Jiao Tong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).