| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0051-v1 |
| --- | --- |

| Title: | AHG9: On the GFV SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Yue Li2, Kaifa Yang2, Yiling Xu2 1Bytedance Inc., US 2Shanghai Jiao Tong University, China | Tel: Email: | yekui.wang@bytedance.com yueli983@sjtu.edu.cn sekiroyyy@sjtu.edu.cn yl.xu@sjtu.edu.cn |
| Source: | Bytedance Inc., Shanghai Jiao Tong University |  |  |

_____________________________

# Abstract

This contribution proposes to add value ranges for nine ue(v)-coded syntax elements in the GFV SEI message as follows:

- The value of gfv_chroma_key_thr_value[ i ] shall be in the range of 0 to 255, inclusive.

- The value of gfv_coordinate_x_abs[ i ] shall be in the range of 0 to 216 − 1, inclusive.

- The value of gfv_coordinate_y_abs[ i ] shall be in the range of 0 to 216 − 1, inclusive.

- The value of gfv_coordinate_z_abs[ i ] shall be in the range of 0 to 216 − 1, inclusive.

- The value of gfv_coordinate_dx_abs[ i ] shall be in the range of 0 to 216 − 1, inclusive.

- The value of gfv_coordinate_dy_abs[ i ] shall be in the range of 0 to 216 − 1, inclusive.

- The value of gfv_coordinate_dz_abs[ i ] shall be in the range of 0 to 216 − 1, inclusive.

- The value of gfv_matrix_delta_element_int[ i ][ j ][ k ][ m ] shall be in the range of 0 to 232 − 2, inclusive.

- The value of gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] shall be in the range of 0 to 232 − 2, inclusive.

The detailed proposed text changes, including a typo fix in addition to the above changes, marked relative to JVET-AI2032-v3, are included in an attachment to this contribution.

# Patent rights declaration(s)

Bytedance Inc. and Shanghai Jiao Tong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).