| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0052-v1 |
| --- | --- |

| Title: | AHG 16: Multi-resolution Models for Generative Face Video Compression |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shanzhi Yin, Shiqi Wang (CityU) Bolin Chen, Yan Ye (Alibaba) Goluck Konuko, Giuseppe Valenzise (CNRS/CentraleSupelec/Université Paris-Saclay) | Email: | shanzhyin3-c@my.cityu.edu.hk shiqwang@cityu.edu.hk chenbolin.chenboli@alibaba-inc.com yan.ye@alibaba-inc.com giuseppe.valenzise@l2s.centralesupelec.fr |
| Source: | City University of Hong Kong, Alibaba, CNRS/CentraleSupelec/Université Paris-Saclay |  |  |

_____________________________

- Abstract

At the 35th JVET meeting, higher-resolution test sequences for generative face video coding (GFVC) were proposed for JVET AHG16 experimentation. One of the GFVC models (i.e., CFTE) was retrained with higher-resolution training data and experimental results showed that the retrained CFTE model can also achieve promising RD performance for higher resolution sequences for perceptual-level metrics following the GFVC test conditions. However, other GFVC algorithms were not implemented for higher resolution, and the compatibility between models trained for different resolutions was not studied. In this contribution, we propose a multi-resolution GFVC pipeline that can support multiple resolutions in a single model. We train multi-resolution models for four GFVC algorithms in the current AHG16 GFVC software. Experimental results show that the proposed multi-resolution models can maintain the RD performance of current single resolution models for 256×256 sequences and achieve promising RD performance on higher resolution 512×512 sequences following the GFVC test conditions.

Introduction

The current AHG16 GFVC software can achieve promising rate-distortion performance mainly at the resolution of 256×256. Several efforts have been made to extent GFVC to code higher resolution face video sequences. At the 34th JVET meeting, JVET-AH0113 [1] was proposed to use adaptive interpolation on low resolution models without re-training, and at the 35th JVET meeting, JVET-AI0047 [2] was proposed to retrain model for one of the GFVC algorithms (i.e., CFTE) with higher-resolution training data. However, the previous attempts only included CFTE algorithm and did not study the multi-resolution support for GFVC algorithms. This type of approach also creates the question on interoperability between models trained on different resolution training data.

To advance more GFVC algorithms to higher resolution scenario and study their resolution compatibility, in this contribution, we first build multi-resolution GFVC pipeline with proposed multi-resolution generator and then train multi-resolution models for AHG16 GFVC software. We evaluate multi-resolution models following the test conditions of GFVC [10] but on expanded set of test sequences that include not only 256×256 sequences already in the GFVC test conditions, but also additional sequences at 512×512 resolution. We compare rate-distortion performance and subjective quality with both VVC anchor and the single-resolution models currently supported in the AHG16 GFVC software.

Proposed Algorithm

Multi-resolution GFVC Pipeline

The structure of proposed multi-resolution GFVC pipeline is shown in Figure 1. The modifications from current structure of AHG16 GFVC software are indicated in red blocks. First, two down-sample blocks are modified from fixed-ratio down-sample to fixed-resolution down-sample. Previous GFVC algorithms adopt fixed down-sample ratio (e.g., 1 in FV2V [3] or 0.25 in CFTE [4]) for feature extraction from inter frames or reconstructed key frames. Such an operation will produce inconsistent motion flow sizes for inputs of different resolutions, which could cause compatibility issue during feature-warping-based generation. In the proposed multi-resolution GFVC pipeline, the down-sample operation is set as fixed-resolution down-sample to 64×64, so that the motion flow size will be fixed at 64×64 for different resolutions. Additionally, the generator in GFVC decoder is replaced by a multi-resolution generator that can implement feature-warping at fixed 64×64 features for all input resolutions to match the motion flow size. Herein, the proposed multi-resolution GFVC pipeline is implemented for four GFVC algorithms in AHG 16 software [7], i.e., FOMM [5], CFTE [4], FV2V [3], DAC [6]. For each algorithm, two resolutions, i.e., 256×256 and 512×512, are supported.

Figure 1 - Multi-resolution GFVC pipeline

Multi-resolution GFVC Generator

Figure 2 - Detailed structure of the proposed multi-resolution generator

The detailed structure of multi-resolution GFVC generator is shown in Figure 2, where “↓2” and “↑2” denote down-sampling and up-sampling by a factor of 2. “w” denotes feature-warping by optical flow and “→” denotes the block that maintains the feature size. All blue blocks are convolutional layers and can be optimized during model training. It can be seen that, to handle the inputs of different resolutions, the first layer of the multi-resolution generator has two branches. For 512×512 inputs, they are spatially down-sampled by a factor of 2, and for 256×256 inputs, their spatial resolution will be maintained by the first layer.

Then, the features will be down-sampled two times to form 64×64 features, which will be warped by 64×64 optical flows. Finally, the features will be up-sampled two times to recover 256×256 features. The last layer of multi-resolution generator also has two branches, for 256×256 output, the features will be directly transformed to images, while for 512×512 output, the features will be up-sampled before transforming to images.

By such a two-branch design in the first and last layer of the multi-resolution generator, the module can take inputs of two different resolutions while it can still maintain feature-warping at fixed size of 64×64.

Experimental results

Training Settings

We train multi-resolution GFVC models on a mixed training set of VoxCeleb [8] and CelebV [9]. The resolution of training sequences are 512×512, where VoxCeleb sequences are interpolated from original resolution of 256×256. During training, we set a probability of 0.5 to down-sample the training batch to 256×256 resolution, so that the models can be equally trained on two resolutions.

Testing Sequences

Herein, we follow the test conditions and software reference configurations for GFVC testing specified in JVET-AG2035 [10]. For 256×256 resolution, we use 33 test sequences specified in JVET-AG2035.

For 512×512 resolution, we use 33 test sequences specified in JVET-AJ0209 [13]. They are selected and pre-processed from the CFVQA dataset [12].

Testing Conditions

The test settings for VVC and for FOMM, CFTE, FV2V, DAC are set according to JVET-AG2035, specifically, as follows:

- For VVC, VTM 22.2 is used and the test is conducted under low-delay B (LDB) configuration with quantization parameters (QP) set to 37, 42, 47 and 52.

- For GFVC algorithms, its base picture is compressed as an I picture using the VTM 22.2, and other pictures are further represented with a series of facial parameters and compressed via a context-adaptive arithmetic coder. The QPs of the base picture are set to 22, 32, 42 and 52.

Rate Distortion Performance of the Proposed Multi-resolution Models

From Figure 3 to Figure 6, the RD performances of proposed multi-resolution GFVC models are compared with the performance of single-resolution models in the current AHG16 software as well as the VVC anchor in terms of Rate-DISTS and Rate-LPIPS. It can be seen that, for each algorithm, multi-resolution models can maintain the RD performance of single-resolution models (i.e., 256×256 models for CFTE, FOMM, FV2V and DAC). In addition, the multi-resolution models are able to extend CFTE, FOMM, FV2V and DAC to higher resolution of 512×512 with promising RD performance against VVC. As such, for each algorithm, two resolutions are successfully supported in a single model, providing better flexibility and resolution compatibility to GFVC pipeline.

Figure 3 - RD Performance of CFTE models in terms of rate-DISTS and rate-LPIPS

Figure 4 - RD Performance of FOMM models in terms of rate-DISTS and rate-LPIPS

Figure 5 - RD Performance of FV2V models in terms of rate-DISTS and rate-LPIPS

Figure 6 - RD Performance of DAC models in terms of rate-DISTS and rate-LPIPS

BD-Rate Savings

The BD-Rate savings of multi-resolution GFVC models against VVC on 256×256 sequences is shown in Table 1. It can be seen that CFTE can achieve -56.03% and -55.59% BD-Rate saving, FOMM can achieve -38.75% and -40.17% BD-Rate saving, FV2V can achieve -52.36% and -51.14% BD-Rate, DAC can achieve -69.56% and -69.97% BD-Rate saving in terms of Rate-DISTS and Rate-LPIPS. For reference, the BD-rate savings of the respective GFVC algorithms in the current AHG16 software are shown in the bottom row of each table in Table 1. It can be seen that, most of the time the proposed multi-resolution models out-perform or have similar performance when compared to the current GFVC models for 256x256 test sequences.

The BD-Rate saving of multi-resolution GFVC models against VVC on 512px scenario is shown in Table 2. It can be seen that CFTE can achieve -66.43% and -52.81% BD-Rate saving, FOMM can achieve -54.04% and -48.11% BD-Rate saving, FV2V can achieve -65.38% and -54.59% BD-Rate, DAC can achieve -73.39% and -69.67% BD-Rate saving in terms of Rate-DISTS and Rate-LPIPS. For Class B sequences, which have the same content and are only different in resolution (512x512 instead 256x256), the BD-rate savings in terms of DISTS and LPIPS in Table 2 are mostly higher than the corresponding BD-rate savings in Table 1. This shows not only the robustness of the proposed multi-resolution models, but also GFVC’s performance potential for higher resolution video.

The results also illustrate that the proposed multi-resolution GFVC models can achieve superior RD performance against VVC on both resolutions.

Table 1 - Average bit-rate savings of 256×256 face video sequences for AHG16 software algorithms compared with the traditional VVC codec and single resolution models evaluated in the RGB 444 and YUV 420 domain

- CFTE

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -60.49% | -59.91% | -14.87% | -37.88% | -1.03% | -31.51% |
| Class B | -51.57% | -51.28% | -10.54% | -35.28% | -4.35% | -29.86% |
| Overall | -56.03% | -55.59% | -12.70% | -36.58% | -2.69% | -30.69% |
| Overall of current CFTE model | -58.02% | -58.49% | -6.36% | -36.14% | -4.90% | -32.52% |

- FOMM

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -41.94% | -44.39% | -0.85% | -18.13% | 2.83% | -13.86% |
| Class B | -35.57% | -35.95% | -0.21% | 4.16% | 32.19% | -14.53% |
| Overall | -38.75% | -40.17% | -0.53% | -6.98% | 17.51% | -14.19% |
| Overall of current FOMM model | -32.76% | -32.67% | 11.33% | -2.92% | 9.79% | #VALUE! |

- FV2V

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -57.55% | -56.73% | -8.00% | -29.29% | -2.22% | -25.01% |
| Class B | -47.17% | -45.54% | -4.73% | -19.78% | -2.52% | -18.77% |
| Overall | -52.36% | -51.14% | -6.37% | -24.54% | -2.37% | -21.89% |
| Overall of current FV2V model | -47.52% | -45.38% | -0.48% | 49.35% | -0.15% | -14.81% |

- DAC

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -74.35% | -75.71% | -11.48% | -53.05% | -4.10% | -59.33% |
| Class B | -64.77% | -64.23% | 2.72% | -50.54% | -2.32% | -45.07% |
| Overall | -69.56% | -69.97% | -4.38% | -51.80% | -3.21% | -52.20% |
| Overall of current DAC model | -67.35% | -68.10% | -22.72% | -51.79% | -14.48% | -46.39% |

Table 2 - Average bit-rate savings of 512×512 face video sequences for AHG16 software algorithms compared with the traditional VVC codec evaluated in the RGB 444 and YUV 420 domain

- CFTE

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -71.30% | -56.21% | -3.67% | -7.88% | 0.00% | -4.21% |
| Class B | -61.57% | -49.41% | 8.02% | -13.60% | 0.00% | -13.45% |
| Overall | -66.43% | -52.81% | 2.18% | -10.74% | 0.00% | -8.83% |

- FOMM

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -58.74% | -52.26% | -1.47% | -11.48% | 0.00% | -6.73% |
| Class B | -49.34% | -43.97% | 9.48% | -15.44% | 60.10% | -4.61% |
| Overall | -54.04% | -48.11% | 4.00% | -13.46% | 30.05% | -5.67% |

- FV2V

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -71.22% | -57.78% | 0.00% | -7.82% | 0.00% | -3.85% |
| Class B | -59.53% | -51.39% | 2.82% | -11.23% | 13.50% | -2.05% |
| Overall | -65.38% | -54.59% | 1.41% | -9.53% | 6.75% | -2.95% |

- DAC

| Sequences | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -78.19% | -73.74% | -4.42% | -13.93% | 0.00% | -5.01% |
| Class B | -68.58% | -65.59% | 1.17% | -24.71% | 0.00% | -21.94% |
| Overall | -73.39% | -69.67% | -1.63% | -19.32% | 0.00% | -13.47% |

Subjective Quality Comparisons

Reconstructed face images are shown in Figure 7 and Figure 8 to provide comparison of reconstruction quality using GFVC single-resolution models, GFVC multi-resolution models and VVC at ultra-low bitrates on both 256×256 and 512×512 sequences. It can be seen that at ultra-low bitrates, VVC reconstructed face images are unrecognizable, whereas proposed multi-resolution models can still reconstruct vivid faces for both resolutions. In addition, multi-resolution models can produce similar subjective quality compared to single resolution models of corresponding algorithms, illustrating the robustness of multi-resolution models across different resolutions.

(a) ClassA Seq 001

(b) ClassB Seq 005

Figure 7 - Comparison of reconstruction quality using single-resolution models, multi-resolution models and VVC at 256×256 resolution.

- ClassA Seq 001                               (b) ClassB Seq001

Figure 8 - Comparison of reconstruction quality using single-resolution models, multi-resolution models and VVC at 512×512 resolution.

Conclusion

This contribution proposes multi-resolution models for AHG16 GFVC software. By fixing feature size of feature-warping process for different resolutions, the proposed multi-resolution GFVC pipeline can support two resolutions in a single model. We train multi-resolution models for four GFVC algorithms in current AHG16 software and the experiment results show that the proposed multi-resolution models can maintain the performances of single resolution models as well as advance promising rate-distortion performances of GFVC to higher resolution. It is suggested to adopt multi-resolution models as the new version of AHG16 software models and replace the previous single-resolution models.

References

- Zou R, Chen B, et al. “AHG16: Lightweight CFTE with Multi-Resolution support,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0113, April 2024.

- Chen B, Yin S, et al. “AhG16: Higher-resolution Test Sequences and Test Results for Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AI0047, July 2024.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021: 10039-10049.

- Chen B, Wang Z, Li B, et al. Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression[C]//2022 Data Compression Conference (DCC). IEEE, 2022: 13-22.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Konuko G, Valenzise G, Lathuilière S. Ultra-low bitrate video conferencing using deep image animation[C]//ICASSP 2021-2021 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2021: 4210-4214.

- Chen B, Ye Y, et al. “AHG 16: Updated Common Software Tools for Generative Face Video Compression”, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0114, April 2024.

- Nagrani A, Chung J S, Zisserman A. Voxceleb: a large-scale speaker identification dataset[J]. arXiv preprint arXiv:1706.08612, 2017.

- Zhu H, Wu W, Zhu W, et al. CelebV-HQ: A large-scale video facial attributes dataset[C]//European conference on computer vision. Cham: Springer Nature Switzerland, 2022: 650-667.

- McCarthy S, Chen B et al. “Test conditions and evaluation procedures for generative face video coding ,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG2035, January 2024.

- Chen B, Chen J, et al. “AHG16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0042, January 2024.

- Li Y, Chen B, Chen B, et al. Perceptual Quality Assessment of Face Video Compression: A Benchmark and An Effective Method[J]. IEEE Transactions on Multimedia, 2024.

- Chen B, Ye Y, et al. “AHG9/AHG16: Performance results of generative face video SEI message on high-resolution sequences”, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AJ0209, November 2024.

Patent rights declaration(s)

City University of Hong Kong, Alibaba and CentraleSupelec may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).