| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0056-v1 |
| --- | --- |

| Title: | EE1-4.1 – Wavelet transform for super-resolution loss function |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiedong Ye, Qiong Liu Zhuoyi Lv (vivo) | Email: | ye_jd@hust.edu.cn q.liu@hust.edu.cn zhuoyi.lv@vivo.com |
| Source: | Huazhong University of Science and Technology, vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, NNSR JVET-AI0074 is re-trained with the DWT loss proposed in JVET-AI0105[1].

This contribution reports the results of EE1-4.1 on top of NNVC10.0. Two sub-tests were conducted as follows:

- EE1-4.1.1 train NNSR w/o wavelets in loss

- EE1-4.1.2 train NNSR with wavelets in loss

Test EE1-4.1.1:

The overall BD-rate savings are shown: AI{-0.81%, 2.86%, 3.05%}, RA{-0.81%, 3.12%, 2.59%}.

The overall M-SSIM savings are shown: AI{ -2.30%, 2.23%, 2.25%}, RA{ -2.07%, 1.48%, 1.52%}.

Test EE1-4.1.2:

The overall BD-rate savings are shown: AI{ -0.82%, 2.86%, 2.76%}, RA{ -0.79%, 2.19%, 2.36%}.

The overall M-SSIM savings are shown: AI{ -2.29%, 2.41%, 2.35%}, RA{-2.07%, 1.69%, 1.65%}.

Introduction

Most of the recent super resolution contributions calculate L1 or L2 scores between ground truth and SR image as a loss function during training. In this contribution, the DWT transform of the ground truth image and the SR image output from the network is conducted before calculating the loss. This does not require any changes to the network structure or training data.

Proposed method

Loss with DWT

By using the DWT transform, the loss is computed directly in the low and high frequency domains to improve the focus of the neural network on the high frequency part.

The network architecture is based on JVET-AI0074[2].

Figure 1 The DWT is performed before calculating loss

Figure 2 Network architecture

Training stage information

Table 1 Network Information

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 60 |
|  | Batch size: | 128 |
|  | Training time: | ~7 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn on NNLF-LOP2 and NNIntra |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128 |
|  | Learning rate: | 4e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Inference stage information

Table 3 Inference information

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 19.6 k |
|  | Total Parameter Number | 19.6 k |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (kB) | 39.2 kB |
|  | Multiply Accumulate (MAC) | 4.7kMAC/pixel |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | 256 |

Experiment results

The proposed method is tested on top of NNVC 10.0, according to the common test conditions [3]. The tested QP values are {22, 27, 32, 37, 42}.

Table 4 Results of EE1-4.1.1 in RA configurations

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10_nnvc |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.04% | 7.70% | 6.15% | -6.05% | -0.72% | 0.52% | 112% | 113% |
| Class A2 | -0.99% | 7.89% | 6.82% | -4.29% | 8.14% | 7.05% | 90% | 107% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.81% | 3.12% | 2.59% | -2.07% | 1.48% | 1.52% | 100% | 104% |

Table 5 Results of EE1-4.1.1 in AI configurations

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10_nnvc |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.65% | 6.88% | 7.68% | -4.62% | 4.87% | 4.84% | 125% | 113% |
| Class A2 | -3.21% | 10.29% | 10.64% | -9.16% | 8.53% | 8.64% | 98% | 113% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.81% | 2.86% | 3.05% | -2.30% | 2.23% | 2.25% | 103% | 104% |

Table 6 Results of EE1-4.1.2 in RA configurations

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10_nnvc |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.18% | 2.77% | 4.03% | -6.09% | -0.37% | 0.70% | 90% | 119% |
| Class A2 | -0.79% | 8.18% | 7.78% | -4.25% | 8.85% | 7.55% | 100% | 106% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.79% | 2.19% | 2.36% | -2.07% | 1.69% | 1.65% | 98% | 105% |

Table 7 Results of EE1-4.1.2 in AI configurations

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10_nnvc |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.79% | 5.81% | 5.39% | -4.66% | 4.84% | 4.90% | 83% | 100% |
| Class A2 | -3.15% | 11.38% | 11.20% | -9.09% | 9.62% | 9.17% | 68% | 105% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.82% | 2.86% | 2.76% | -2.29% | 2.41% | 2.35% | 91% | 101% |

Conclusion

This contribution propose to add DWT loss into the training stage of NNSR, to increase the coding performance without any changes of network structure or training data. This method does not require modification of the network structure and can be used in combination to any network structure for high compatibility.

It is shown that objective qualities can be improved by the proposal. So it is recommended to adopt it into NNVC.

Reference

[1] Jiedong Ye, Kejun Wu, Qiong Liu “Wavelet transform for super-resolution loss function”, JVET-AI0105

[2] Zhuoyi Lv, Chuan Zhou “EE1-4.3: RPR new input for super-resolution”, JVET-AI0074

[3] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016.

Patent rights declaration(s)

Huazhong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).