| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0057-v1 |
| --- | --- |

| Title: | EE2-2.14: High-level control of intra prediction methods |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyungyong (Kyle) Kim John Juhyung Son Jin Sam Kwak | Email: | kyungyong.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-2.14, which proposes to disable several methods in intra prediction for screen content, including reference sample filtering, PDPC (and gradient PDPC), and intra prediction fusion.

The simulation results, based on ECM-14.0 [1], are reported as follows:

AI: { -0.14% Y, -0.02% U, -0.02% V, 99.8% EncT, 99.0% DecT } for class F

{ -0.28% Y, -0.27% U, -0.31% V, 99.4% EncT, 98.8% DecT} for class TGM

RA: { -0.05% Y, 0.06% U, 0.11% V, 99.7% EncT, 100.2% DecT } for class F

{ -0.16% Y, 0.01% U, -0.02% V, 99.8% EncT, 100.9% DecT } for class TGM

Introduction

Reference sample filtering helps reduce artifacts caused by edges in the reference samples. However, for screen content, edges can be important for generating the prediction block. Position-dependent prediction combination (PDPC) reduces prediction errors along block boundaries. Intra prediction fusion method derives predicted samples by using a weighted combination of multiple predictors generated from different reference lines. Although these methods provide gains in natural contents, reference sample filtering, PDPC and intra prediction fusion may be less effective when predicting screen content.

Proposal

This contribution proposes disabling several methods in intra prediction for screen content, including reference sample filtering, PDPC (and gradient PDPC), and intra prediction fusion.

The application of these methods for screen content is controlled by individual SPS flags, as illustrated in Fig. 1. Each SPS flag is set based on the hash block hit percentage at the encoder, a metric already available in ECM and used in other tools. If the percentage exceeds a certain threshold, the video sequence is interpreted as screen content, and the specified methods (reference sample filtering, PDPC (and gradient PDPC), and intra prediction fusion) are not applied to that sequence.

Figure 1. SPS flags for controlling reference sample filtering, PDPC and intra prediction fusion

For testing, we added several configurations (DisableRefFilter, DisablePdpc and DisableIntraFusion) in class F configuration file.

Test Result

The proposed methods were implemented on top of ECM-14.0 [1]. Simulations for anchor were performed by following the common test conditions [2]. More detailed results can be found in the attached Excel sheets. The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.14% | -0.02% | -0.02% | 99.8% | 99.0% | 100.4% | 100.0% |
| Class TGM | -0.28% | -0.27% | -0.31% | 99.4% | 98.8% | 99.3% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.05% | 0.06% | 0.11% | 99.7% | 100.2% | 99.9% | 100.0% |
| Class TGM | -0.16% | 0.01% | -0.02% | 99.8% | 100.9% | 100.2% | 100.0% |

Conclusion

This contribution proposes disabling several methods in intra prediction for screen content, specifically reference sample filtering, PDPC (and gradient PDPC), and intra prediction fusion. It is recommended that these proposed changes be incorporated into the next ECM.

Reference

[1] M. Coban, and et al., “Algorithm description of Enhanced Compression Model 14 (ECM 14),” JVET-AI2025, July 2024.

[2] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, July 2024.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).