| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0058-v2 |
| --- | --- |

| Title: | AHG9: Generalizing text prompt usage in the NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Claire-Hélène Demarty, Didier Doyen, Philippe de Lagrange | Tel: Email: | 1stname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution relates to the usage of a text prompt to guide the inference of an NN model, as described in NNPFC SEI message with spatial extrapolation purpose, currently specified in the VSEI version 4 specification.

The contribution proposes to extend the concept of text prompt to other purposes than the spatial extrapolation, and to add an indication whether the text prompt information is signaled in the SEI message or provided by external means.

# Introduction

In contribution JVET-AI0061, it was proposed to optionally add in the NNPFC syntax a text prompt for the purpose of image generation through spatial extrapolation. The text prompt is passed as an auxiliary information to the NNPF and is used as input of the NNPF input tensor for generating the contents of the spatially extrapolated image area. The concept was adopted in the VSEI version 4 specification (JVET_AI2006).

An excerpt of the related syntax and semantics is provided below.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| … |  |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( SpatialExtrapolationFlag ) { |  |
| … |  |
| nnpfc_spatial_extrapolation_right_offset | se(v) |
| nnpfc_spatial_extrapolation_prompt_present_flag | u(1) |
| if (nnpfc_spatial_extrapolation_prompt_present_flag) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_c | u(1) |
| nnpfc_prompt | st(v) |
| } |  |
| } |  |
| … |  |
| } |  |

nnpfc_spatial_extrapolation_prompt_present_flag equal to 1 specifies that nnpfc_prompt syntax element is present and nnpfc_alignment_zero_bit_c syntax element may be present. nnpfc_spatial_extrapolation_prompt_present_flag equal to 0 specifies that nnpfc_prompt syntax element and nnpfc_alignment_zero_bit_c syntax element are not present.

nnpfc_alignment_zero_bit_c shall be equal to 0.

nnpfc_prompt specifies the text string prompt used for generating the contents of the spatial extrapolation image area. When nnpfc_spatial_extrapolation_prompt_present_flag is equal to 1, nnpfc_prompt shall not be a null string.

The current proposal suggests extending this concept with the two following points:

- Generalizing the text prompt usage to other purposes than the spatial extrapolation, namely, temporal extrapolation, general visual quality improvement, colourization.

- Indicating whether the text prompt information is signaled in the SEI message or provided by external means.

# Proposals

Proposal 1: Generalizing the text prompt usage to other purposes

We consider that the usage of a text prompt to guide the NNPF inference may be useful for other purposes than the spatial extrapolation. A few examples are mentioned below.

- Temporal extrapolation in interactive applications for managing latency issues:

  - For example, in interactive immersive viewing using a head-mounted display, a text prompt may be used to indicate a user action, such as the motion of the head of the user. For instance, text prompts such as “head does not move”, “head turns to the right”, “head turns to the left”, “head turns to the top”, “head turns to the bottom” may be signaled.

  - Another example is cloud gaming, to anticipate the next step of the game, depending on the expected action(s) of the gamer(s). The end-user gaming device may detect the actual action of the gamer, translate it as a text prompt, and give this information as input of the NNPF.

- Colourization: the text prompt may provide indication of style (e.g., in the context of style transfer) in the colourization process, for example, “colourization with the style of Van Gogh paintings”, “colourization with cartoon style”, “colourization with old movie style”.

- General visual quality improvement: examples of improvements may be “increase sharpness”, “increase contrast”, “increase brightness”, “reduce blocking artefacts”, “reduce grain”, etc.

The following syntax and semantics changes are proposed in yellow:

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| … |  |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | se(v) |
| nnpfc_spatial_extrapolation_right_offset | se(v) |
| nnpfc_spatial_extrapolation_top_offset | se(v) |
| nnpfc_spatial_extrapolation_bottom_offset | se(v) |
| nnpfc_spatial_extrapolation_prompt_present_flag | u(1) |
| if (nnpfc_spatial_extrapolation_prompt_present_flag) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_c | u(1) |
| nnpfc_prompt | st(v) |
| } |  |
| } |  |
| if( QualityImprovementFlag ¦ ¦ ColourizationFlag ¦ ¦ TemporalExtrapolationFlag ¦ ¦ SpatialExtrapolationFlag ¦ ¦ ) { |  |
| nnpfc_prompt_present_flag | u(1) |
| if (nnpfc_prompt_present_flag) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_c | u(1) |
| nnpfc_prompt | st(v) |
| } |  |
| … |  |
| } |  |

The variables QualityImprovementFlag, ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, ColourizationFlag, and TemporalExtrapolationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include quality improvement, chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, colourization, and temporal extrapolation, respectively, are derived as follows:

QualityImprovementFlag = ( ( nnpfc_purpose & 0x01 ) > 0 ) ? 1 : 0
ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0
SpatialExtrapolationFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : 0

…

nnpfc_prompt_present_flag nnpfc_spatial_extrapolation_prompt_present_flag equal to 1 specifies that nnpfc_prompt syntax element is present and nnpfc_alignment_zero_bit_c syntax element may be present. nnpfc_prompt_present_flag nnpfc_spatial_extrapolation_prompt_present_flag equal to 0 specifies that nnpfc_prompt syntax element and nnpfc_alignment_zero_bit_c syntax element are not present.

nnpfc_prompt specifies the text string prompt used as input for the NNPF, for example as input for a generative NNPF for generating the contents of the filtered image area. When nnpfc_spatial_extrapolation_prompt_present_flag is equal to 1 nnpfc_prompt is present, it shall not be a null string.

…

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor. nnpfc_auxiliary_inp_idc equal to 1, 2 or 3 specifies that auxiliary input data is derived as specified in Equation 95.

When nnpfc_auxiliary_inp_idc is equal to 2 or 3, nnpfc_spatial_extrapolation_prompt_present_flag nnpfc_prompt_present_flag shall be equal to 1.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 255, inclusive. Values of 2 to 255, inclusive, for nnpfc_auxiliary_inp_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_auxiliary_inp_idc in the range of 4 to 255, inclusive.

When nnpfc_auxiliary_inp_idc is equal to 1 the auxiliary input data consists of strengthControlScaledVal[ i ].

When nnpfc_auxiliary_inp_idc is equal to 2 the auxiliary input data consists of nnpfc_prompt character values.

When nnpfc_auxiliary_inp_idc is equal to 3, the auxiliary input data consists of strengthControlScaledVal[ i ] and nnpfc_prompt character values.

…

Proposal 2: Enabling the text prompt to be provided by external means

In the second proposal, it is proposed to enable the text prompt to be provided by external means. This option is relevant when the NN is able to handle various hypotheses.

A first control flag (nnpfc_prompt_control_flag) is signaled to indicate whether a text prompt is used or not as input of the NNPF.

Then, when the control flag is true, two options are considered:

- 2a: signal a present flag indicating whether the text prompt is signaled or comes from external means

- 2b: signal several possible text prompts, the one to be used being selected by external means (except when the number of text prompts is equal to 1)

Proposal 2a: signal a present flag indicating whether the text prompt is signaled or comes from external means

In proposal 2a, it is proposed to use two different flags related to the text prompt, namely a first flag nnpfc_prompt_control_flag indicating whether a text prompt is used as the NNPF input tensor, and another flag nnpfc_prompt_present_flag only signaled when nnpfc_prompt_control_flag is true, indicating whether the text prompt is signaled in the SEI message or is provided from external means.

The following syntax changes are described below.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| … |  |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | se(v) |
| nnpfc_spatial_extrapolation_right_offset | se(v) |
| nnpfc_spatial_extrapolation_top_offset | se(v) |
| nnpfc_spatial_extrapolation_bottom_offset | se(v) |
| nnpfc_spatial_extrapolation_prompt_present_flag | u(1) |
| if (nnpfc_spatial_extrapolation_prompt_present_flag) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_c | u(1) |
| nnpfc_prompt | st(v) |
| } |  |
| } |  |
| if( QualityImprovementFlag ¦ ¦ ColourizationFlag ¦ ¦ TemporalExtrapolationFlag ¦ ¦ SpatialExtrapolationFlag ) { |  |
| nnpfc_prompt_control_flag | u(1) |
| if (nnpfc_prompt_control_flag) { |  |
| nnpfc_prompt_present_flag | u(1) |
| if (nnpfc_prompt_present_flag) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_c | u(1) |
| nnpfc_prompt | st(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

nnpfc_prompt_control_flag equal to 1 specifies that a text string prompt shall be used as auxiliary input of the NNPF input tensor. nnpfc_prompt_control_flag equal to 0 specifies that no text string prompt is used as auxiliary input of the NNPF for generating the temporal extrapolation.

nnpfc_prompt_present_flag equal to 1 specifies that nnpfc_prompt is present and nnpfc_alignment_zero_bit_c syntax element may be present in the SEI message. nnpfc_prompt_present_flag equal to 0 specifies that nnpfc_prompt and nnpfc_alignment_zero_bit_c syntax elements are not present and that nnpfc_prompt is obtained from external means, not specified in this document.

- When nnpfc_prompt_control_flag is equal to 1, the parameter textPrompt, defined as a string of characters, is derived as follows:

  - When nnpfc_prompt_present_flag is equal to 1, textPrompt is set equal to nnpfc_prompt

  - Otherwise (nnpfc_prompt_present_flag equal to 0), textPrompt is provided by external means not specified in this document.

Note: when nnpfc_prompt_control_flag is equal to 1 and nnpfc_prompt_present_flag is equal to 0, the text string prompt is provided by external means to the NNPF, not specified in this document, for example by the end-user, or the end-user machine.

nnpfc_prompt specifies the text string prompt used as input for the NNPF, for example as input for a generative NNPF for generating the contents of the filtered image area. When nnpfc_spatial_extrapolation_prompt_present_flag is equal to 1 nnpfc_prompt is present, it shall not be a null string.

…

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor. nnpfc_auxiliary_inp_idc equal to 1, 2 or 3 specifies that auxiliary input data is derived as specified in Equation 95.

When nnpfc_auxiliary_inp_idc is equal to 2 or 3, nnpfc_spatial_extrapolation_prompt_present_flag nnpfc_prompt_control_flag shall be equal to 1.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 255, inclusive. Values of 2 to 255, inclusive, for nnpfc_auxiliary_inp_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_auxiliary_inp_idc in the range of 4 to 255, inclusive.

When nnpfc_auxiliary_inp_idc is equal to 1 the auxiliary input data consists of strengthControlScaledVal[ i ].

When nnpfc_auxiliary_inp_idc is equal to 2 the auxiliary input data consists of nnpfc_prompt textPrompt character values.

When nnpfc_auxiliary_inp_idc is equal to 3, the auxiliary input data consists of strengthControlScaledVal[ i ] and nnpfc_prompt character values.

…

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				promptCharVal = utf8ToUInt(nnpfc_prompt textPrompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag )
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
				else
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc − 1 ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ nnpfc_auxiliary_inp_idc − 1 ] = promptCharVal
			}
	else if( nnpfc_inp_order_idc  = =  1 )				(96)
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				promptCharVal = utf8ToUInt(nnpfc_prompt textPrompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][  nnpfc_auxiliary_inp_idc ] = promptCharVal
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yY = cTop + yP
				xY = cLeft + xP
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				promptCharVal = utf8ToUInt(nnpfc_prompt textPrompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc + 1 ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ nnpfc_auxiliary_inp_idc + 1 ] = promptCharVal
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yTL = cTop + yP * 2
				xTL = cLeft + xP * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yP
				xC = cLeft / 2 + xP
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ], 2 ) )
				promptCharVal = utf8ToUInt(nnpfc_prompt textPrompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
					inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
					inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc + 4 ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ nnpfc_auxiliary_inp_idc + 4 ] = promptCharVal
			}
}

Proposal 2b: signal several possible text prompts, the one to be used being selected by external means (except when the number of text prompts is equal to 1)

In proposal 2b, several text prompts may be signaled in the SEI message when the control flag is true. The number of signaled text prompts is signaled. When this number is 1, the single signaled text prompt is used by the NNPF. When this number greater than 1, the text prompt to be used by the NNPF should be selected by external means among the set of signaled text prompts.

The following syntax and semantics changes in yellow are described below.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| … |  |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | se(v) |
| nnpfc_spatial_extrapolation_right_offset | se(v) |
| nnpfc_spatial_extrapolation_top_offset | se(v) |
| nnpfc_spatial_extrapolation_bottom_offset | se(v) |
| nnpfc_spatial_extrapolation_prompt_present_flag | u(1) |
| if (nnpfc_spatial_extrapolation_prompt_present_flag) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_c | u(1) |
| nnpfc_prompt | st(v) |
| } |  |
| } |  |
| if( QualityImprovementFlag ¦ ¦ ColourizationFlag ¦ ¦ TemporalExtrapolationFlag ¦ ¦ SpatialExtrapolationFlag ) { |  |
| nnpfc_prompt_control_flag | u(1) |
| if (nnpfc_prompt_control_flag) { |  |
| nnpfc_num_prompts_minus1 | u(8) |
| for( i = 0; i < nnpfc_num_prompts_minus1+1; i++) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_c | u(1) |
| nnpfc_prompt[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

nnpfc_prompt_control_flag equal to 1 specifies that a text string prompt shall be used as auxiliary input of the NNPF input tensor. nnpfc_prompt_control_flag equal to 0 specifies that no text string prompt is used as auxiliary input of the NNPF for generating the temporal extrapolation.

nnpfc_num_prompts_minus1 plus 1 specifies that nnpfc_prompt[ i ] is present, with i in 0.. nnpfc_num_prompts_minus1.

nnpfc_prompt[ i ] specifies the ith text string prompt used for generating the contents of the filtered image area. When present, nnpfc_prompt[ i ] shall not be a null string.

- When nnpfc_prompt_control_flag is equal to 1, the parameter textPrompt, defined as a string of characters, and the parameter promptIdx, are derived as follows:

  - When nnpfc_num_prompts_minus1 is equal to 0, promptIdx is set equal to 0

  - Otherwise (nnpfc_num_prompts_minus1 not equal to 0), promptIdx is provided by external means not specified in this document. promptIdx shall be a value belonging to the interval 0..nnpfc_num_prompts_minus1.

  - textPrompt is set equal to nnpfc_text_prompt[ promptIdx ].

Note: when nnpfc_num_prompts_minus1 is greater than 0, the text prompt to be used in the input tensor must be selected by external means not specified in this document.

…

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor. nnpfc_auxiliary_inp_idc equal to 1, 2 or 3 specifies that auxiliary input data is derived as specified in Equation 95.

When nnpfc_auxiliary_inp_idc is equal to 2 or 3, nnpfc_spatial_extrapolation_prompt_present_flag nnpfc_prompt_control_flag shall be greater than 0.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 255, inclusive. Values of 2 to 255, inclusive, for nnpfc_auxiliary_inp_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_auxiliary_inp_idc in the range of 4 to 255, inclusive.

When nnpfc_auxiliary_inp_idc is equal to 1 the auxiliary input data consists of strengthControlScaledVal[ i ].

When nnpfc_auxiliary_inp_idc is equal to 2 the auxiliary input data consists of nnpfc_prompt textPrompt character values.

When nnpfc_auxiliary_inp_idc is equal to 3, the auxiliary input data consists of strengthControlScaledVal[ i ] and nnpfc_prompt character values.

…

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				promptCharVal = utf8ToUInt(nnpfc_prompt textPrompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag )
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
				else
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc − 1 ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ nnpfc_auxiliary_inp_idc − 1 ] = promptCharVal
			}
	else if( nnpfc_inp_order_idc  = =  1 )				(96)
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				promptCharVal = utf8ToUInt(nnpfc_prompt textPrompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][  nnpfc_auxiliary_inp_idc ] = promptCharVal
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yY = cTop + yP
				xY = cLeft + xP
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				promptCharVal = utf8ToUInt(nnpfc_prompt textPrompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc + 1 ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ nnpfc_auxiliary_inp_idc + 1 ] = promptCharVal
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yTL = cTop + yP * 2
				xTL = cLeft + xP * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yP
				xC = cLeft / 2 + xP
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ], 2 ) )
				promptCharVal = utf8ToUInt(nnpfc_prompt textPrompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
					inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
					inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc + 4 ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ nnpfc_auxiliary_inp_idc + 4 ] = promptCharVal
			}
}

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).