| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0060-v1 |
| --- | --- |

| Title: | AHG9: Packed regions information SEI message with multi-layer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Didier Doyen, Fabrice Urban, Edouard François, Philippe de Lagrange | Tel: Email: | 1stname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes an extension of the Packed regions information SEI message to consider the case where the different regions of interest are conveyed in different layers. Corresponding syntax and semantics changes are proposed.

# Packed regions Information SEI

Packed regions information (PRI) SEI message was proposed in JVET-AH0161 and adopted in the VSEI Technologies under consideration (TuC) (JVET-AI2032), to use packing of rectangular regions of interest (ROIs) from a larger picture into a smaller resolution coded picture. In the example of figure 1 the background is coded at lower resolution, while the two foreground faces are coded at full resolution. The packed picture is of lower resolution than the native picture. The reconstruction of a target picture is done from the specified regions of the decoded picture.

The packed regions information SEI message described above is only designed for a mono-layer coding solution. However, multi-layer coding may also be used to provide the functionality of the PRI SEI message. For example, each ROI can use an independent layer, or the background can be transmitted in a separate layer instead of being packed within the same picture as the face ROIs.

Figure 1. Mono-layer version – Top-left: source image with 2 ROIs and background. Top-right: code packed picture with downsampled background packed with the 2 ROIs. Bottom-right: reconstructed target image.

# Proposal

The PRI SEI message uses packing of rectangular regions of interest (ROIs) from a larger picture into a smaller resolution to code and reconstruct the picture.

The proposal consists of enhancing the PRI SEI message by introducing a multi-layer variant.

Coding and transmitting regions in different layers allow using different encoder and quality settings. Moreover, layers can be easily managed, at the system level. This allows adding more functionalities to the current PRI SEI message ones, such as changing the background (by coding different backgrounds in different layers), dynamically changing a region resolution/quality without redoing the complete frame packing and encoding.

Using different layers additionally eases the packing: for example, trying to pack faces with background may introduce holes in the packed frame (visible as grey areas in figure 1), whereas face ROIs may have same size, and may be easy to pack together.

## Multi-layer use case

In the multi-layer use case, multi-layer encoding can be used to transmit each region in a separate layer, or in combination with region-packing such as depicted in 2. The main syntax changes consist of the following. An overall flag indicates whether multi-layer coding is used or not. When multi-layer is used, an information is added in the PRI SEI message to indicate for each region which layer it belongs to.

Figure 2. Multi-layer version – Top-left: source image with 2 ROIs and background. Top-right: background coded in layer0, 2 ROIs packed and coded in layer1. Bottom-right: reconstructed target image.

## Option1: indication of the id of the layer used by each ROI

As highlighted in the table below, a flag is added to signal whether the bitstream is a multi-layer one, and if true, layer information is added for each region.

| packed_regions_info( payloadSize) { | Descriptor |
| --- | --- |
| pri_cancel_flag | u(1) |
| if ( !pri_cancel_flag ) { |  |
| pri_persistence_flag | u(1) |
| pri_num_regions_minus1 | ue(v) |
| pri_multilayer_flag | u(1) |
| pri_use_max_dimensions_flag | u(1) |
| pri_log2_unit_size | u(4) |
| pri_region_size_len_minus1 | u(4) |
| pri_region_id_present_flag | u(1) |
| pri_target_pic_params_present_flag | u(1) |
| if( pri_target_pic_params_present_flag ) { |  |
| pri_target_pic_width_minus1 | u(16) |
| pri_target_pic_height_minus1 | u(16) |
| } |  |
| pri_num_resampling_ratios_minus1 | ue(v) |
| for( i = 1; i <= pri_num_resampling_ratios_minus1; i++ ) { |  |
| pri_resampling_width_num_minus1[ i ] | ue(v) |
| pri_resampling_width_denom_minus1[ i ] | ue(v) |
| pri_fixed_aspect_ratio_flag[ i ] | u(1) |
| if( !pri_fixed_aspect_ratio_flag[ i ] ) { |  |
| pri_resampling_height_num_minus1[ i ] | ue(v) |
| pri_resampling_height_denom_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| for( i = 0; i <= pri_num_regions_minus1; i++ ) { |  |
| if( pri_region_id_present_flag ) |  |
| pri_region_id[ i ] | ue(v) |
| if( pri_multilayer_flag ) |  |
| pri_region_layer_id[ i ] | ue(v) |
| pri_region_top_left_in_units_x[ i ] | u(v) |
| pri_region_top_left_in_units_y[ i ] | u(v) |
| pri_region_width_in_units_minus1[ i ] | u(v) |
| pri_region_height_in_units_minus1[ i ] | u(v) |
| if( pri_num_resampling_ratios_minus1 > 0 ) |  |
| pri_resampling_ratio_idx[ i ] | u(v) |
| if( pri_target_pic_params_present_flag ) { |  |
| pri_target_region_top_left_x[ i ] | u(v) |
| pri_target_region_top_left_y[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Where the semantics of the syntax elements are the same as in JVET-AI2032, except for the added ones:

pri_multilayer_flag equal to 1 specifies that the pri_layer_id[ i ] syntax element is present. pri_multilayer_flag equal to 0 specifies that pri_layer_id[ i ] syntax element is not present.

pri_region_layer_id[ i ] specifies the layer id of the picture that the region information pri_region_top_left_in_units_x[ i ], pri_region_top_left_in_units_y[ i ], pri_region_width_in_units_minus1[ i ], pri_region_height_in_units_minus1[ i ] relate to. If pri_region_layer_id[ i ] is not present, it is inferred to be 0.

## Option2: indication of the id of the used layers, and of id of the used layer used by each ROI

In case the bitstream is multi-layer, but the PRI SEI message uses only a subset of the layers (at least one), each used layer j can be identified by its layer ID in the SEI as pri_used_layer_id[ j ]. If more than one layer is used (pri_num_used_layer_minus1 > 0), the layer is identified for each region i with pri_region_used_layer_idx[ i ]. The syntax and semantics are summarized below:

| packed_regions_info( payloadSize) { | Descriptor |
| --- | --- |
| pri_cancel_flag | u(1) |
| if ( !pri_cancel_flag ) { |  |
| pri_persistence_flag | u(1) |
| pri_num_regions_minus1 | ue(v) |
| pri_multilayer_flag | u(1) |
| if( pri_multilayer_flag ) { |  |
| pri_num_used_layer_minus1 | ue(v) |
| for( j = 0; j <= pri_num_used_layer_minus1; j++ ) { |  |
| pri_used_layer_id[ j ] | ue(v) |
| } |  |
| } |  |
| pri_use_max_dimensions_flag | u(1) |
| pri_log2_unit_size | u(4) |
| pri_region_size_len_minus1 | u(4) |
| pri_region_id_present_flag | u(1) |
| pri_target_pic_params_present_flag | u(1) |
| if( pri_target_pic_params_present_flag ) { |  |
| pri_target_pic_width_minus1 | u(16) |
| pri_target_pic_height_minus1 | u(16) |
| } |  |
| pri_num_resampling_ratios_minus1 | ue(v) |
| for( i = 1; i <= pri_num_resampling_ratios_minus1; i++ ) { |  |
| pri_resampling_width_num_minus1[ i ] | ue(v) |
| pri_resampling_width_denom_minus1[ i ] | ue(v) |
| pri_fixed_aspect_ratio_flag[ i ] | u(1) |
| if( !pri_fixed_aspect_ratio_flag[ i ] ) { |  |
| pri_resampling_height_num_minus1[ i ] | ue(v) |
| pri_resampling_height_denom_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| for( i = 0; i <= pri_num_regions_minus1; i++ ) { |  |
| if( pri_region_id_present_flag ) |  |
| pri_region_id[ i ] | ue(v) |
| if( pri_num_used_layer_minus1 > 0 ) |  |
| pri_region_used_layer_idx[ i ] | ue(v) |
| pri_region_top_left_in_units_x[ i ] | u(v) |
| pri_region_top_left_in_units_y[ i ] | u(v) |
| pri_region_width_in_units_minus1[ i ] | u(v) |
| pri_region_height_in_units_minus1[ i ] | u(v) |
| if( pri_num_resampling_ratios_minus1 > 0 ) |  |
| pri_resampling_ratio_idx[ i ] | u(v) |
| if( pri_target_pic_params_present_flag ) { |  |
| pri_target_region_top_left_x[ i ] | u(v) |
| pri_target_region_top_left_y[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Where the semantics of the syntax elements are the same as in section 2.2, except for the added ones:

pri_multilayer_flag equal to 1 specifies that layer syntax elements pri_used_layer_id[ j ] are present. pri_multilayer_flag equal to 0 specifies that layer syntax elements pri_used_layer_id[ j ] are not present. When pri_multilayer_flag equal to 0, pri_used_layer_id[ 0 ] is inferred to be 0, or to the layer_id of the layer the PRI SEI applies to (single layer).

pri_num_used_layer_minus1 plus 1 specifies the number of layers used for the regions. If pri_num_used_layer_minus1 is not present, it is inferred to be 0, pri_used_layer_id[ 0 ] is not signalled and inferred to be 0, or the layer_id of the layer the PRI SEI message applies to

pri_used_layer_id[ j ] specifies the j-th layer id used in the PRI SEI message.

pri_region_used_layer_idx[ i ] specifies index of the used layer Id for the i-th region. When not present, it is inferred to be 0. The layer Id used for region i (priRegionLayerId[ i ]) is inferred as follows:

priRegionLayerId[ i ] = pri_used_layer_id[ pri_region_used_layer_idx[ i ] ]

## Option3: Syntax simplification if a region is a whole layer

If a region covers a whole layer picture, position and size of each region can be omitted. For example, the background is sent as an independent layer and is fully used for the region. For each region, a flag pri_region_is_a_layer is added for each region. It can be used in combination with the two previous options.

| packed_regions_info( payloadSize) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i <= pri_num_regions_minus1; i++ ) { |  |
| if( pri_region_id_present_flag ) |  |
| pri_region_id[ i ] | ue(v) |
| if( pri_multilayer_flag ) { |  |
| pri_region_layer_id[ i ] | ue(v) |
| pri_region_is_a_layer[ i ] | ue(v) |
| } |  |
| if( !pri_region_is_a_layer[ i ] ) { |  |
| pri_region_top_left_in_units_x[ i ] | u(v) |
| pri_region_top_left_in_units_y[ i ] | u(v) |
| pri_region_width_in_units_minus1[ i ] | u(v) |
| pri_region_height_in_units_minus1[ i ] | u(v) |
| } |  |
| if( pri_num_resampling_ratios_minus1 > 0 ) |  |
| pri_resampling_ratio_idx[ i ] | u(v) |
| if( pri_target_pic_params_present_flag ) { |  |
| pri_target_region_top_left_x[ i ] | u(v) |
| pri_target_region_top_left_y[ i ] | u(v) |
| } |  |
| } |  |

Already defined semantics are the same as previously.

pri_region_is_a_layer[ i ] equal to 1 specifies the whole layer is used as the region of index i, and pri_region_top_left_in_units_x[ i ], pri_region_top_left_in_units_y[ i ], pri_region_width_in_units_minus1[ i ], pri_region_height_in_units_minus1[ i ] are not signaled. When pri_region_is_a_layer[ i ] is not signaled it is inferred to be 0.

### Variant where all regions are layers

If all regions are in separate layers, it can be signaled at higher level as follows:

| packed_regions_info( payloadSize) { | Descriptor |
| --- | --- |
| pri_cancel_flag | u(1) |
| if ( !pri_cancel_flag ) { |  |
| pri_persistence_flag | u(1) |
| pri_num_regions_minus1 | ue(v) |
| pri_multilayer_flag | u(1) |
| if( pri_multilayer_flag ) { |  |
| pri_all_regions_are_layers | u(1) |
| } |  |
| … | u(1) |
| for( i = 0; i <= pri_num_regions_minus1; i++ ) { |  |
| if( pri_region_id_present_flag ) |  |
| pri_region_id[ i ] | ue(v) |
| if( pri_multilayer_flag ) { |  |
| pri_region_layer_id[ i ] | ue(v) |
| if( ! pri_all_regions_are_layers) { |  |
| pri_region_is_a_layer[ i ] | ue(v) |
| } |  |
| } |  |
| if( !pri_region_is_a_layer[i] ) { |  |
| pri_region_top_left_in_units_x[ i ] | u(v) |
| pri_region_top_left_in_units_y[ i ] | u(v) |
| pri_region_width_in_units_minus1[ i ] | u(v) |
| pri_region_height_in_units_minus1[ i ] | u(v) |
| } |  |
| … |  |

pri_all_regions_are_layers equal to 1 specifies that all the regions use a full signaled layer. pri_region_is_a_layer[ i ] is inferred to be 1 for every layer i, and is not signaled.

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).