| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0061-v3 |
| --- | --- |

| Title: | EE2-2.5: Timd-Merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli, Ramin G. Youvalari, Frank Plowman, Alexandre Tissier, Yao-Jen Chang, Vadim Seregin, Marta Karczewicz Jiaye Fu, Jiaqi Zhang, Shanshe Wang Siwei Ma (Peking Univ.) | Emails: | mabdoli@xiaomi.com yjchang@qti.qualcomm.com jyfu@stu.pku.edu.cn |
| Source: | Xiaomi Technology, Qualcomm Incorporated, PKU |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-2.5 exploration experiment on the TIMD-merge mode. This test proposes an algorithm for creating a merge list based on the neighboring TIMD blocks. This list is then sorted based on the template cost and one candidate is finally kept in the list. A CU-level flag then indicates whether a TIMD-merge candidate is used for block prediction.

Here is a summary of the EE2-2.5 results:

AI { -0.08%, 0.01%, 0.05%, 100.6%, 100.2%},

RA* { -0.04%, -0.17%, -0.05%, 99.9%, 100.2%}

(*) 12 missing points (class A/B) have been copied from anchor.

# Introduction

The TIMD process is based on a template cost analysis in which several intra modes -- from a candidates list comprising MPMs, wide-angular, DC, horizontal, vertical, and extended modes are tested. Up to three intra modes (IPM1, IPM2,  IPM3) providing the smallest SATD costs are selected and blended in a fusion process according to their respective template cost.

## TIMD-merge mode (Test EE2-2.5)

The TIMD merge mode, originally proposed in JVET-AG0106 [2], inherits the TIMD modes from the previously TIMD coded blocks. For this, the adjacent and non-adjacent spatial neighboring blocks are scanned and if the scanned block is coded in TIMD or TIMD merge mode, then its TIMD information (prediction modes, fusion flag, fusion weights and wide-angle conditions of TIMD modes, MTS transform types) are added to a TIMD merge list. Up to five TIMD candidates are added into the initial list.

### Merge list construction

Figure 1 shows the non-adjacent spatial neighboring blocks that are used in TIMD merge list generation.

Fig 1. Non-adjacent spatial neighboring candidates for TIMD merge mode

Table 1: Initial TIMD merge list

| Cand | Intra modes | Fusion weights |
| --- | --- | --- |
| 1 | (IPM1,1, IPM1,2, IPM1,3) | (W1,1, W1,2, W1,3) |
| 2 | (IPM2,1, IPM2,2, IPM2,3) | (W2,1, W2,2, W2,3) |
| . . . | . . . | . . . |
| 5 | (IPM5,1, IPM5,2, IPM5,3) | (W5,1, W5,2, W5,3) |

An initial list of five TIMD candidates is created from the neighborhood. After forming the TIMD merge list, the merge candidates are sorted based on the SAD cost over the template of the current block. For the cost calculation, the SAD cost of the primary IPM of each TIMD candidate is used.

The template size for sorting of TIMD merge candidates is decided to be one line from the immediate neighbourhood of the block.

The best candidate from the list is then selected to be potentially included in encoder RDO.

The TIMD merge mode is applied to only luma blocks. Moreover, the mode is disabled for 4x4 block sizes.

In the encoder side, the SAD template costs calculation may reuse the SAD costs of the MPM list reordering in order to reduce the encoder runtime.

### Bitstream Signalling

Usage of the mode is signaled with a CABAC coded CU level flag. The TIMD merge mode is used as a sub-mode of TIMD tool.

# Results

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.07% | -0.07% | 100.2% | 99.7% |  |  |
| Class A2 | -0.08% | -0.04% | -0.07% | 100.8% | 100.4% |  |  |
| Class B | -0.09% | 0.07% | 0.11% | 100.9% | 99.7% |  |  |
| Class C | -0.08% | -0.13% | 0.00% | 100.4% | 100.1% |  |  |
| Class E | -0.08% | 0.25% | 0.27% | 100.4% | 101.7% |  |  |
| Overall | -0.08% | 0.01% | 0.05% | 100.6% | 100.2% |  |  |
| Class D | -0.06% | 0.03% | 0.00% | 100.5% | 100.6% |  |  |
| Class F | -0.05% | -0.19% | -0.07% | 100.5% | 100.5% |  |  |
| Class TGM |  |  |  |  |  |  |  |

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |  |
|  | Y | U | V |  | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.25% | -0.21% |  | 100.0% | 100.1% |  |  |
| Class A2 | -0.04% | -0.14% | 0.04% |  | 100.1% | 99.1% |  |  |
| Class B | -0.05% | -0.11% | -0.14% |  | 100.0% | 101.0% |  |  |
| Class C | -0.02% | -0.19% | 0.12% |  | 99.4% | 100.1% |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.17% | -0.05% |  | 99.9% | 100.2% |  |  |
| Class D | -0.02% | -0.36% |  | -0.38% | 99.8% | 99.8% |  |  |
| Class F |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Xiaomi Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).