| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0062-v3 |
| --- | --- |

| Title: | AhG11: Attention based CNN for Chroma Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mouad Aderdor Gosala Kulupana Marc Gorriz Blanch Luka Murn The Lighthouse, 201 Wood Ln, W12 7TQ, London, UK | Tel: Email: | mouad.aderdor@bbc.co.uk gosala.kulupana@bbc.co.uk marc.gorrizblanch@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a new intra prediction mode for Chroma in NNVC. This prediction mode is composed of a Convolutional Neural Network (CNN) that takes as input Y, U, V neighbouring pixels and Y collocated luma pixels. The computed features are then merged by an attention matrix, followed by a prediction head to output both chroma channels. The model is size-agnostic and only supports 4x4, 8x8 and 16x16 block sizes for the moment.

The proposed Attention based CNN for chroma which is implemented on NNVC-10.0, reports below gains with respect to NNVC-10.0 with NN intra prediction mode [1] enabled:

- AI: -0.14%, -0.80% and -0.60% BD-rate PSNR Y, U and V gains, respectively for a complexity of 108% and 158% at the encoder and the decoder respectively.

The proposed method was also tested on a version of NNVC-10 with restricted square CUs partitioning, with dimensions between 8 and 32. With the same configuration as the previous results, this gives:

- AI: -0.34%, -2.75% and -2.14% BD-rate PSNR Y, U and V gains, respectively for a complexity of 158% and 578% at the encoder and the decoder respectively.

# Introduction

Although NN intra prediction exists in NNVC, it does not benefit from cross component information. It has been reported in past contributions like Cross Component Linear Model (CCLM) that Luma information can also be useful in achieving higher gains in Chroma. This contribution uses collocated Luma blocks, and reference samples from all channels to predict chroma blocks, using a CNN with an attention module.

# Description

## Preprocessing

As stated, the model takes in collocated Luma blocks and Y, U, V reference pixels. Assuming encoding in 420 chroma format, the decoded collocated Luma block is extracted and downsampled using the same interpolation filters in NNVC to match the chroma resolution, converted to floating point and normalized to be in the range [0-1].

For each chroma block size of h,w to be predicted, one line of adjacent reference pixels of length 2h and 2w+1 from the left and top of the block, respectively, are extracted from the Y, U and V channels. The resulting vectors for each channel are converted to floating point and normalized to be in the range [0-1]. Unavailable reference pixels are replaced by values of half the range.

## Model architecture

Figure 1: Attention Based CNN Network architecture at inference

Cross component branch extracts cross component features from the input boundary matrix S0 ∈ R3×b by applying I consecutive Di - dimensional 1 × 1 convolutional layers to obtain the Si ∈ RDi×b   output feature maps, where ii = 1, 2 . . . I. By applying 1×1 convolutions, the boundary input dimensions are preserved, resulting in a Di dimensional vector of cross-component information for each boundary location.

Collocated luma branch extracts spatial patterns over the collocated reconstructed luma block X0 by applying convolutional operations. The luma convolutional branch is defined by a 5 × 5 convolutional layer with a stride of 1, to obtain X1 ∈ RC1×N2 feature maps from the N2 input samples.

Attention module: The feature maps (SJ and X1) from both branches are each convolved using a 1 × 1 kernel, to project them into h-dimensional feature matrices F and G. The two matrices are multiplied together to obtain the pre-attention map M. Finally, the attention matrix A ∈ RN2×b is obtained by applying a softmax operation to each element of M, to generate the probability of each boundary location being able to predict a sample location in the block. Each value αj,i in A is obtained as:

αj,i = expmi,jTn=1b-1exp (mn,jT)

where T=0.5 is the softmax temperature. The weighted sum of the contribution of each reference sample in predicting a given sample at a specific location is obtained by computing the matrix multiplication between the cross-component boundary features SI and the attention matrix A. To refine the output, this weighted sum can be multiplied element-wise by the output of the luma branch processed by 1 x 1 convolution filter to output O matrix.

Prediction head is defined by a convolutional layer, applying a 3×3 convolutional filter to O for deriving the two chroma components at once.

The neural network is initially trained with two convolutional filters in the Luma branch and the prediction head branch. For inference, these consecutive convolutions are collapsed into one convolution as can be seen in table 1. This method has reported less complexity with an increase in performance.

| Network Branch | Layer index | Input | Input channels | Output channels | Filter size | Activation |
| --- | --- | --- | --- | --- | --- | --- |
| Cross component branch | 1 | S0 | 3 | 32 | 1×1 | ReLU |
|  | 2 | 1 | 32 | 32 | 1×1 | ReLU |
| Luma branch (training) | 3 | X0 | 1 | 64 | 3×3 | - |
|  | 4 | 3 | 64 | 64 | 3×3 | ReLU |
| Luma branch (inference) | 5 | X0 | 1 | 64 | 5×5 | ReLU |
| Attention module | 6 | 2 | 32 | 16 | 1×1 | ReLU |
|  | 7 | 4 (training) 5 (inference) | 64 | 16 | 1×1 | ReLU |
|  | 8 | 4 (training) 5 (inference) | 64 | 32 | 1×1 | ReLU |
| Prediction head (training) | 9 | O | 32 | 32 | 3×3 | - |
|  | 10 | 9 | 32 | 2 | 1×1 | - |
| Prediction head (inference) | 11 | O | 32 | 2 | 3×3 | - |

Table 1: Network hyperparameters

## Size agnostic architecture

The model is fully convolutional, meaning that it can support multiple block sizes as input. To maximize inference efficiency, the model sees input from each block size at each epoch during training and optimizes the weights as described in figure 2, where Zm(N) is defined as the ground truth chroma blocks for a given {XmN, Sm(N)} input, and M is the number of samples. Supported sizes are {(4,4), (8,8), (16,16)}.

Figure 2: Training algorithm for the proposed multi-model architecture.

## Postprocessing

The output is multiplied by (2b-1), where b is the bit depth, to generate the final chroma blocks.

## Signalling

For the current w ×h chrominance CB, Attention Based CNN chroma intra prediction is signalled in a similar manner to CCLM with the condition that h, w∈{(4,4),  (8,8),  (16,16)}. This new mode is added to the list of modes to undergo rate distortion checks.

# AHG11 template

| Network information in training stage |  |  |
| --- | --- | --- |
| Required | GPU type | NVIDIA GeForce RTX 2080 Ti (12Gb) |
|  | Framework | Tensorflow 1.14.0 |
|  | Number of GPUs per task | 1 |
|  | Epoch | 90 |
|  | Batch size | 16 of 2,532,192 total training samples. |
|  | Training time | 1-2 days |
|  | Training data information | DIV2K with 6 downscaled versions using bilinear and unknown filters. For each data instance, one resolution version was randomly selected and then M blocks of N × N were chosen, making balanced sets between block sizes and uniformed spatial selections within each image. All samples were converted to YCbCr colour space and further normalized to be in the range [0, 1] |
|  | Training configurations for generating compressed training data | Training data has not been compressed. |
|  | Patch size | Currently supports square patches of 4x4, 8x8, 16x16 |
|  | Learning rate | 1e-4 |
|  | Optimizer | ADAM. |
|  | Loss function | MSE |
|  | Preprocessing | Normalization between 0-1. Downsampled Luma Channel Flattened and Stacked neighboring Y, U, V reference pixels |

Table 2: neural network information in the training stage

| Network information in inference stage |  |  |  |
| --- | --- | --- | --- |
| Required | Parameter number | 7074 |  |
|  | Parameter precision | 32F |  |
|  | MACs/pixel | (16,16) | 9218 |
|  |  | (8,8) | 8167 |
|  |  | (4,4) | 8423 |
|  | Total conv. layers | 7 |  |
|  | Total FC layers | 0 |  |
|  | GPU Type | No GPU |  |
|  | CPU Type | AMD Ryzen 9 7950X 16-Core Processor |  |
|  | Framework | MIP |  |

Table 3 neural network information in the inference stage

# Experiment Results

|  |  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over NNVC-10.0 –NnIntraPred=1 –NnlfOption=0 |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 | -0.16% |  | -1.04% |  | -0.92% | 108% | 167% |
| Class A2 | -0.12% |  | -0.88% |  | -0.38% | 108% | 142% |
| Class B | -0.15% |  | -0.95% |  | -0.81% | 105% | 167% |
| Class C | -0.17% |  | -0.61% |  | -0.49% | 111% | 165% |
| Class E | -0.08% |  | -0.51% |  | -0.29% | 109% | 144% |
| Overall | -0.14% |  | -0.80% |  | -0.60% | 108% | 158% |
| Class D | -0.15% |  | -0.82% |  | -0.89% | 106% | 160% |

Table 4: JVET CTC [2] encoding results in All Intra configuration for Attention based CNN for chroma intra prediction over NNVC-10.0

|  |  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over NNVC-10.0 –NnIntraPred=1 –NnlfOption=0 |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.07% |  | -0.97% |  | -0.67% | 106% | 135% |
| Class C | -0.07% |  | -0.36% |  | -0.48% | 106% | 126% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.01% |  | -0.18% |  | -0.66% | 104% | 122% |

Table 5: JVET CTC [2] encoding results in Random access configuration for Attention based CNN for chroma intra prediction over NNVC-10.0

Another test was conducted in All Intra mode where the encoder is constrained to generate only square CU partitions of (32,32), (16,16) and (8,8). This was done to maximize support for the Attention CNN.

|  |  |  |  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over NNVC-10.0 –NnIntraPred=1 –NnlfOption=0 (square constraint) |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | -0.40% |  | -3.69% |  | -2.65% | 164% |  | 530% |  |
| Class A2 | -0.42% |  | -3.34% |  | -1.65% | 166% |  | 572% |  |
| Class B | -0.32% |  | -2.74% |  | -2.56% | 157% |  | 632% |  |
| Class C | -0.40% |  | -2.36% |  | -2.08% | 153% |  | 687% |  |
| Class E | -0.16% |  | -1.75% |  | -1.52% | 152% |  | 438% |  |
| Overall | -0.34% |  | -2.75% |  | -2.14% | 158% |  | 578% |  |
| Class D | -0.39% |  | -2.17% |  | -1.74% | 133% |  | 675% |  |

Table 6: JVET CTC [2] encoding results for Attention based CNN for chroma intra prediction over NNVC-10.0. Both reference and test encoders are constrained to generate only square CUs of sizes between (32,32) and (8,8)

# Conclusion

A new NN intra prediction mode is introduced to NNVC-10.0 for chroma channels, using a CNN with attention. The results report -0.14%, -0.80% and -0.60% BD-rate PSNR Y, U and V gains respectively, and -0.34%, -2.75% and -2.14% BD-rate PSNR Y, U and V gains respectively on square constrained NNVC, suggesting further gains when the model is used more frequently.

# References

- T. Dumas, F. Galpin, P. Bordes. AHG11: combination of the neural network-based intra prediction mode and ISP, JVET-AH0165, 34th JVET meeting, Rennes, 17-24 April 2024.

- S. Liu, A.Segall, E.Alshina, R.Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-U2016, 21st JVET meeting, Teleconference, 6-15 January 2021.

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).