| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting: Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0063-v1 |
| --- | --- |

| Title: | AHG9: On Encoder Optimization Information SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following modifications are proposed related to the encoder optimization information SEI:

- Proposal 1: It is proposed to code the syntax element eoi_object_based_idc using fixed length coding instead of ue(v) coding.

- Proposal 2: It is proposed to define following new values for the bits in eoi_object_based_idc:

  - bitmask=0x08: Areas outside the detected objects have been overwritten in some form but not with a constant sample value. (along with slight redefinition of the value for bitmask 0x04)

  - bitmask=0x10: Areas in the objects have been treated differently based on the object size. For example, objects are pre-sorted in size and large objects use higher QP values than smaller objects during encoding.

- Proposal 3: It is proposed to optionally signal the constant luma, cb and cr sample values used for overwriting areas outside of detected objects when (eoi_object_based_idc & 0x04) > 0.

- Proposal 4: It is proposed to code the eoi_privacy_protection_method_idc syntax element as a bitmask which can allow simultaneous indication of multiple types of protection methods used. It is also proposed to use 8 bits to code this syntax element.

- Proposal 5: An additional privacy protection method indication (“pixelation”) is proposed to be added to the table which lists possible privacy protection methods.

# Introduction

The encoder optimization information (EOI) SEI message is used to indicate if the video has been optimized for human viewing or machine analysis and which types of optimization have been applied in pre-processing or encoding. Some modifications are proposed to EOI SEI.

# Proposals

The proposed modifications are shown below compared to JVET-AI2006 [1] using change marks.

## Proposal 1

It is proposed to code the syntax element eoi_object_based_idc using fixed length coding instead of ue(v) coding.

It is asserted that the syntax element eoi_object_based_idc indicates object-based optimization using a bitmask approach where individual bits indicate certain types of optimizations. It is asserted that because of the use of bitmask, the syntax element eoi_object_based_idc is conceptually similar to the bitmask-based syntax elements eoi_type, eoi_privacy_info_type in the EOI SEI. It is noted that both these bitmask based syntax elements eoi_type and eoi_privacy_info_type are fixed length coded – as u(16) and u(8) resepctively. It is asserted that for design consistency it makes sense to also code the bitmask-based syntax element eoi_object_based_idc using a fixed length coding. Currently 3 bitmask values are defined for eoi_object_based_idc. Additionally, two new bitmask values are proposed in proposal 2 below. As a result it is proposed to code eoi_object_based_idc using u(8).

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v)u(8) |
| … |  |
| } |  |

eoi_object_based_idc, when present, indicates the type of object-based optimization as specified in Table x2, where ( eoi_object_based_idc & bitMask ) not equal to 0 indicates that the object-based optimization type associated with the bitMask value in Table x2 has been applied. When eoi_object_based_idc is greater than 0 and ( eoi_object_based_idc & bitMask ) is equal to 0, the object-based optimization type associated with the bitMask value has not been applied. When eoi_object_based_idc is equal to 0, an application-defined type of object-based optimization has been applied. The value of eoi_object_based_idc shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 65 535255, inclusive, for eoi_object_based_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_object_based_idc is in the range of 8 to 65 535255, inclusive, decoders conforming to this version of this Specification shall ignore eoi_object_based_idc.

## Proposal 2

It is proposed to define following new values for the bits in eoi_object_based_idc:

- bitmask=0x08: Areas outside the detected objects have been overwritten in some form but not with a constant sample value. (along with slight redefinition of the value for bitmask 0x04)

The current bitmask 0x04 indicates the processing where the areas outside the detected objects are overwritten. It is asserted that the area outside may be either overwritten with a constant sample value or may be overwritten using non-constant sample values. It is proposed to distinguish between these two cases by defining an additional bitmask value. It is asserted that knowing that the area outside has been replaced with a constant sample value is beneficial to the receiver, as this information can be used for example for more reliably/ easily detecting which pixels have been overwritten and thus belong to the background. It is thus proposed to define an additional bitmask value of 0x08 to indicate non-constant sample value overwriting and to redefine the bitmask value of 0x04 to be constant sample value overwriting.

- bitmask=0x10: Areas in the objects have been treated differently based on the object size. For example, objects are pre-sorted in size and large objects use higher QP values than smaller objects during encoding

It is asserted that according to JVET-AI2030 TR on Optimization of encoders and receiving systems for machine analysis of coded video content (draft 6) [2]:  As per section 8.1 [6]: “As an extension, it can also be beneficial to encode large objects with slightly higher QP values. As it is generally easier to detect larger objects, reducing the bit rate for large objects usually does not reduce the performance of the machine consumption task.”

Also, Annex A [6] describes software implementation of an encoder for a technique for region of interest-based adaptive QP. This technique classifies a CTU into one of three possible categories. One of the categories listed is: “Overlap only with large objects”, where objects are pre-sorted into large and small objects, which is determined by comparing the size of the area covered by the bounding box of the object to a pre-set threshold. If all objects the CTU overlaps with are large objects, the QP offset can be increased by an intermediate value as large objects are generally easy to detect and thus the detection does not suffer by reducing the quality slightly.

It is proposed to define the bitmask value 0x10 to indicate this type of processing. It is asserted that other optimization techniques defined in JVET-AI2030 TR are captured in the table x2, but this one is missing.

The proposed modifications are shown below.

eoi_object_based_idc, when present, indicates the type of object-based optimization as specified in Table x2, where ( eoi_object_based_idc & bitMask ) not equal to 0 indicates that the object-based optimization type associated with the bitMask value in Table x2 has been applied. When eoi_object_based_idc is greater than 0 and ( eoi_object_based_idc & bitMask ) is equal to 0, the object-based optimization type associated with the bitMask value has not been applied. When eoi_object_based_idc is equal to 0, an application-defined type of object-based optimization has been applied. The value of eoi_object_based_idc shall be in the range of 0 to 731, inclusive, in bitstreams conforming to this version of this Specification. Values of 832 to 65 535, inclusive, for eoi_object_based_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_object_based_idc is in the range of 832 to 65 535, inclusive, decoders conforming to this version of this Specification shall ignore eoi_object_based_idc.

Table x2 – Definition of eoi_object_based_idc

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Areas outside the detected objects have been blurred prior to encoding. |
| 0x02 | Areas outside the detected objects have been encoded with coarser transform-domain quantization than the quantization used for the detected objects. |
| 0x04 | Areas outside the detected objects have been overwritten. For example, an encoding system can overwrite areas outside the detected objects with a constant sample value. |
| 0x08 | Areas outside the detected objects have been overwritten in some form but not with a constant sample value. |
| 0x10 | Areas in the objects have been treated differently based on the object size. For example, objects are pre-sorted in size and large objects use higher QP values than smaller objects during encoding. |

## Proposal 3

It is proposed to optionally signal the constant luma, cb and cr sample values used for overwriting areas outside of detected objects when (eoi_object_based_idc & 0x04) > 0.

When (eoi_object_based_idc & 0x04) > 0, areas outside the detected objects have been overwritten. For example, an encoding system can overwrite areas outside the detected objects with a constant sample value.

It is asserted that when the areas outside the objects are overwritten with a constant sample value indicating that sample value (controlled by a presence flag) may be beneficial. For example, the receiver could then check the sample values at pixels that are contiguous and outside an object, to determine which pixels have been overwritten. It is thus proposed to indicate the constant Y, Cb, Cr sample values used for overwriting (controlled by a presence flag).

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ){ |  |
| eoi_object_based_idc | ue(v) |
| if(eoi_object_based_idc & 0x04 ) > 0 ) { |  |
| eoi_object_overwrite_vals_present_flag | u(1) |
| if( eoi_object_overwrite_vals_present_flag ) { |  |
| eoi_object_luma_overwrite_val | ue(v) |
| eoi_object_cb_overwrite_val | ue(v) |
| eoi_object_cr_overwrite_val | ue(v) |
| } |  |
| } |  |
| } |  |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| … |  |
| } |  |

    - Encoder optimization information SEI message semantics

The encoder optimization information SEI message is used to indicate if the video has been optimized for human viewing or machine analysis and which types of optimization have been applied in pre-processing or encoding.

…

eoi_object_overwrite_vals_present_flag equal to 1 specifies that eoi_object_luma_overwrite_val, eoi_object_cb_overwrite_val, and eoi_object_cr_overwrite_val syntax elements are present. eoi_object_overwrite_vals_present_flag equal to 0 specifies that eoi_object_luma_overwrite_val, eoi_object_cb_overwrite_val, and eoi_object_cr_overwrite_val syntax elements are not present.

eoi_object_luma_overwrite_val indicates the luma value used for overwriting areas outside of detected objects when (eoi_object_based_idc & 0x04) > 0. The value of eoi_object_luma_overwrite_val shall be in the range of 0 to ( 1  <<  BitDepthY ) − 1, inclusive.

eoi_object_cb_overwrite_val indicates the Cb value used for overwriting areas outside of detected objects when (eoi_object_based_idc & 0x04) > 0. The value of eoi_object_luma_overwrite_val shall be in the range of 0 to ( 1  <<  BitDepthC ) − 1, inclusive.

eoi_object_cr_overwrite_val indicates the Cr value used for overwriting areas outside of detected objects when (eoi_object_based_idc & 0x04) > 0. The value of eoi_object_luma_overwrite_val shall be in the range of 0 to ( 1  <<  BitDepthC ) − 1, inclusive.

## Proposal 4

It is asserted that the syntax element eoi_privacy_protection_idc defines values which do not support indicating privacy protection that may have been applied to the picture by using a combination of the defined techniques (e.g. blurring some part of the image and replacing some part of the image for privacy purposes).

It is proposed to code the eoi_privacy_protection_method_idc syntax element as a bitmask which can allow simultaneous indication of multiple types of privacy protection methods used.

It is also proposed to use 8 bits to code this syntax element to allow sufficient future extensibility when using bitmask based approach.

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| … |  |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_method_idc | u(4)u(8) |
| eoi_privacy_info_type | u(8) |
| } |  |
| } |  |

eoi_privacy_protection_method_idc, when present, indicates the privacy protection method / algorithm. eoi_privacy_protection_method_idc greater than 0 and ( eoi_privacy_protection_method_idc & bitMask ) not equal to 0 indicates that the method/ algorithm with the bitMask value in Table x3 has been used to apply privacy protection method. When eoi_privacy_protection_method_idc is equal to 0, the method/ algorithm used for privacy protection is unknown or determined by the application.

The value of eoi_privacy_protection_method_idc shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 255, inclusive, for eoi_privacy_protection_method_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_privacy_protection_method_idc is in the range of 8 to 255, inclusive, decoders conforming to this version of this Specification shall ignore eoi_privacy_protection_method_idc.

Table x3 – Definition of eoi_privacy_protection_method_idc

| bitmask | Interpretation |
| --- | --- |
| 0 | Unknown or determined by the application. |
| 0x011 | Blurring; private information is blurred to make it unidentifiable. |
| 0x022 | Replacing; private information is replaced with something different from the original to make it unidentifiable. |
| 0x043 | Masking; private information is masked so that it cannot be identified |
| 4…15 | Reserved for future use. |

## Proposal 5

It is proposed to indicate “Pixelation” as another type of privacy protection method. Pixelation replaces an area of the image with larger colored blocks, making details impossible to discern.

Thus, privacy protection method indication “pixelation” is proposed to be added to the table which lists possible privacy protection methods.

This can be done by modifying the current table in JVET-AI2006 as follows:

Table x3 – Definition of eoi_privacy_protection_meethod_idc

| eoi_privacy_protection_method_idc | Interpretation |
| --- | --- |
| 0 | Unknown or determined by the application. |
| 1 | Blurring; personal information is blurred to make it unidentifiable. |
| 2 | Replacing; personal information is replaced with something different from the original to make it unidentifiable. |
| 3 | Masking; personal information is masked so that it cannot be identified |
| 4 | Pixelation; personal information is pixelated to make it undiscernible |
| 45…15 | Reserved for future use. |

Or if the proposal 4 above is adopted then as follows:

Table x3 – Definition of eoi_privacy_protection_method_idc

| bitmask | Interpretation |
| --- | --- |
| 0 | Unknown or determined by the application. |
| 0x011 | Blurring; private information is blurred to make it unidentifiable. |
| 0x022 | Replacing; private information is replaced with something different from the original to make it unidentifiable. |
| 0x043 | Masking; private information is masked so that it cannot be identified |
| 0x08 | Pixelation; personal information is pixelated to make it undiscernible |
| 4…15 | Reserved for future use. |

eoi_privacy_protection_method_idc, when present, indicates the privacy protection method / algorithm. eoi_privacy_protection_method_idc greater than 0 and ( eoi_privacy_protection_method_idc & bitMask ) not equal to 0 indicates that the method/ algorithm with the bitMask value in Table x3 has been used to apply privacy protection method. When eoi_privacy_protection_method_idc is equal to 0, the method/ algorithm used for privacy protection is unknown or determined by the application.

The value of eoi_privacy_protection_method_idc shall be in the range of 0 to 15, inclusive, in bitstreams conforming to this version of this Specification. Values of 16 to 255, inclusive, for eoi_privacy_protection_method_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_privacy_protection_method_idc is in the range of 16 to 255, inclusive, decoders conforming to this version of this Specification shall ignore eoi_privacy_protection_method_idc.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AI2006, J. Boyce, S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang, SEI messages for VSEI version 4 (Draft 3), July 2024

- JVET-AI2030, C. Hollmann, J. Chen, S. Liu, Optimization of encoders and receiving systems for machine analysis of coded video content (draft 6), July 2024