| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0064-v1 |
| --- | --- |

| Title: | AHG9: Signaling of Photosensitive Content Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

It is proposed to indicate in an SEI message, photosensitive content information such as strobe lights/ flashing lights in an image and sequence of images, which could cause photosensitive epilepsy/ seizures. Two alternative options are proposed.

- Option 1: It is proposed to use the text description information SEI to indicate a picture (or sequence of pictures) includes photosensitive content which exceeds guidelines in ITU-R BT 1702-3. It is proposed to use txt_descr_purpose equal to 6 in this case and then to specify details about the characteristics of the photosensitive material in the txt_descr_string[ i ] syntax element in the SEI.

- Option 2: A new SEI message to indicate photosensitive content information is proposed.

# Introduction

The VSEI V4 draft 3 [1] includes a text description information SEI.

## It has been reported that photosensitive epilepsy occurs in 1 in 4000 of the population. Video sequences can precipitate photosensitive seizures, typically when the image includes photosensitive content such as strobe lights/ flashing lights.

## This proposal is a follow-up to the proposal JVET-AI0060. For JVET-AI0060 proposal, according to the JVET meeting notes:

“Further study was encouraged to investigate what information is useful to signal according to established practices.”

Also, the JVET meeting notes include suggestion that there may be ITU-R work in the photosensitivity area.

## We studied the standardization work in photosensitivity area and identified relevant standards [2-4]. The proposed approach refers directly to information from these standards.

It is proposed to indicate in an SEI, photosensitive content information such as strobe lights/ flashing lights in an image and sequence of images, which could cause photosensitive epilepsy/ seizures. Two alternative options are proposed.

- Option 1: It is proposed to use the text description information SEI to indicate a picture (or sequence of pictures) includes photosensitive content which exceeds guidelines in ITU-R BT 1702-3. It is proposed to use txt_descr_purpose equal to 6 in this case and then to specify details about the characteristics of the photosensitive material in the txt_descr_string[ i ] syntax element in the SEI.

- Option 2: A new SEI message to indicate photosensitive content information is proposed.

# Proposal

It is proposed to indicate photosensitive content information via SEI. Two alternative options are proposed.

## Option 1:

It is proposed to use the text description information SEI to indicate a picture (or sequence of pictures) includes photosensitive content which exceeds guidelines in ITU-R BT 1702-3. It is proposed to use txt_descr_purpose equal to 6 in this case and then to specify details about the characteristics of the photosensitive material in the txt_descr_string[ i ] syntax element in the SEI.

## The proposed modifications are shown on top of JVET-AI2006 [1].

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table xx.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6 | Photosensitive content information for content with potentially harmful flashing images and/ or patterns exceeding guidelines in ITU-R BT 1702-3 |
| 67-255 | Reserved |

txt_descr_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the txt_descr_purpose.

When txt_descr_purpose is equal to 0, the interpretation of what information is conveyed in the txt_descr_string is application-defined.

When txt_descr_purpose is equal to 1, the txt_descr_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope of this SEI message.

When txt_descr_purpose is equal to 2, the txt_descr_string[ i ] specifies, when not a null string, AI marking information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 2 the string can contain information about machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

When txt_descr_purpose is equal to 3, the txt_descr_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope of this SEI message.

When txt_descr_purpose is equal to 4, the txt_descr_string[ i ] shall specify content advisory rating information conforming to US and Canadian Rating Region Tables (RRT) as defined by CTA-766-D that pertains to the picture(s) in the persistence scope of this SEI message.

When txt_descr_purpose is equal to 5, txt_descr_string[ i ] shall contain a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

The txt_descr_purpose equal to 6 specifies that the picture(s) in the persistence scope of this SEI message contain photosensitive content that includes potentially harmful flashing images and/ or potentially harmful patterns exceeding the guidelines 1 and/ or  2 in ITU-R BT 1702-3 [2] and information about characteristics and description of the photosensitive content is specified by the txt_descr_string[ i ].

## Option 2:

Alternatively, it is proposed to define a new SEI message to indicate photosensitive content information such as strobe light(s) or flashing light(s) present in the image or sequence of images. The syntax and semantics for the new photosensitive content information SEI is shown below.

The main elements in the proposed SEI includes:

- A syntax element pci_content_idc which is a bit mask. Individual bits in the bit mask specify whether the photosensitive content in the picture(s) in the persistence scope of this SEI exceed the photosensitivity content guidelines specified by a corresponding standard.

- pci_descr_string_lang[ i ] and pci_descr_string[ i ] syntax elements which respectively specify the language for the text information and the text information about characteristics and description of photosensitive content in the picture(s) in the persistence scope of this SEI

Photosensitive Content information SEI message

## Photosenstive content information SEI message syntax

| photosensitive_content_information( payloadSize ) { | Descriptor |
| --- | --- |
| pci_cancel_flag | u(1) |
| if(!pci_cancel_flag) { |  |
| pci_persistence_flag | u(1) |
| pci_content_idc | u(14) |
| pci_num_descr_strings | u(8) |
| for( i = 0; i <= pci_num_descr_strings; i++ ) { |  |
| pci_descr_string_lang[ i ] | st(v) |
| pci_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |

## Photosenstive content information SEI message semantics

pci_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous photosensitive content information SEI message in output order. pci_cancel_flag equal to 0 indicates that photosensitive content information follows.

pci_persistence_flag specifies the persistence of the photosensitive content information SEI message for the current layer.

pci_persistence_flag equal to 0 specifies that the photosensitive content information SEI message applies to the current decoded picture only.

pci_persistence_flag equal to 1 specifies that the photosensitive content information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

– A picture in the current layer in an AU associated with a photosensitive content information SEI message is output that follows the current picture in output order.

pci_content_idc indicates the information regarding photosensitive content characteristics as specified in Table x1, where ( pci_content_idc & bitMask ) not equal to 0 indicates that the picture(s) in the persistence scope of this SEI contain photosensitive content which exceeds the photosensitivity content guidelines specified by the corresponding standard. When pci_content_idc is greater than 0 and ( pci_content_idc & bitMask ) is equal to 0, the photosensitive content in the picture(s) in the persistence scope of this SEI does not exceed the photosensitivity content guidelines specified by the corresponding standard. When pci_content_idc is equal to 0, picture(s) in the persistence scope of this SEI include photosensitivity content with information about it as specified by pci_descr_string[ i ] syntax element(s).

Table x1 – Definition of pci_content_idc

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Photosensitive content guidelines specified by ITU-R BT 1702-3 [2] |
| 0x02 | Photosensitive content guidelines specified by ISO 9241-391 [3] |
| 0x04 | Photosensitive content guidelines specified by ISO/ IEC 40500:2012 [4] |

pci_num_descr_strings indicates the number of entries for pci_descr_string_lang[ I ] and pci_descr_string[ i ] that follow.

pci_descr_string_lang[ i ] specifies the language of the pci_descr_string[ i ]. The language of the pci_descr_string[ i ]  shall be given by a language tag as defined by IETF RFC 5646 [5].

pci_descr_string[ i ] specifies text information about characteristics and description of photosensitive content in the associated decoded picture(s).

# Conclusion

It is proposed to adopt the proposed changes to the next draft of VSEI version 4.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AI2006, J. Boyce, S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang, SEI messages for VSEI version 4 (Draft 3), July 2024

- International Telecommunication Union, 2023, Recommendation ITU-R BT.1702-3: Guidance for the reduction of photosensitive epileptic seizures caused by television. https://www.itu.int/rec/R-REC-BT.1702/en

- International Organization for Standardization, ISO 9241-391:2016 Ergonomics of human-system interaction—Part 391: Requirements, analysis and compliance test methods for the reduction of photosensitive seizures, https://www.iso.org/standard/56350.html

- International Organization for Standardization, ISO/ IEC 40500:2012, Information technology — W3C Web Content Accessibility Guidelines (WCAG) 2.0, https://www.iso.org/standard/58625.html

- IETF RFC 5646 (in force), Tags for Identifying Languages.