| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 36th Meeting: Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0071-v1 |
| --- | --- |

| Title: | AHG9: On the importance flag in the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

To simplify the processing importance related signalling, this contribution proposes to merge the flag of po_sei_processing_degree_flag[ i ] into the flag of po_sei_importance_flag[ i ] and rename po_sei_importance_flag[ i ] to po_sei_importance_idc[ i ]. Two-bit signalling is used for po_sei_importance_idc[ i ] to specify the three cases including the case considering  po_sei_processing_degree_flag[ i ] in JVET-AI2006..

# Introduction

In the last JVET meeting, the update on the SEI processing order SEI message is included in JVET-AI2006. The updated syntax structure and the processing importance related text are as follows.

  - SEI processing order and processing order nesting SEI message

    - SEI processing order SEI message

      - SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_for_human_viewing_idc | u(2) |
| po_for_machine_analysis_idc | u(2) |
| po_reserved_zero_4bits | u(4) |
| po_num_sei_messages_minus2 | u(7) |
| po_breadth_first_flag | u(1) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_processing_degree_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(12) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| … |  |
| } |  |

      - SEI processing order SEI message semantics

The SEI processing order (SPO) SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for a group of types of SEI messages that may be present in a CVS.

…

po_sei_importance_flag[ i ] equal to 1 affects the derivation of PoSeiList, which is the list of SEI messages that a decoding system should process for a particular picture picA, as specified below.

po_sei_importance_flag[ i ] equal to 0 specifies that when the decoding system cannot interpret or does not support the functionality indicated by the i-th SEI message type, it shall ignore all data associated with the loop variable value of i and exclude the i-th SEI message type from the processing chain performed by the decoding system.

po_sei_processing_degree_flag[ i ] affects the derivation of PoSeiList as specified below.

PoSeiList is derived as follows:

–	PoSeiList is initially empty.

–	The following applies in a non-decreasing order of po_sei_processing_order[ i ] values for all values of i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive, unless terminated earlier as specified below:

–	When an SEI message seiA associated with the i-th SEI message type persists for picA, the following applies:

–	If the decoding system can interpret and supports the functionality indicated by seiA, seiA is added at the end of PoSeiList.

–	Otherwise, if po_sei_importance_flag[ i ] is equal to 1 and po_sei_processing_degree_flag[ i ] is equal to 0, the derivation of PoSeiList is terminated.

–	Otherwise, if po_sei_importance_flag[ i ] is equal to 1 and po_sei_processing_degree_flag[ i ] is equal to 1, the processing chain specified by this SPO SEI message should not be performed for picA, PoSeiList is set to be empty, and the derivation of PoSeiList is terminated.

…

# Proposal

In the above update in JVET-AI2006, a flag of po_sei_processing_degree_flag[ i ] is added to indicate the degree of processing for the SPO SEI message performed by the decoding system. It is observed the use po_sei_processing_degree_flag[ i ] purely depends on use of po_sei_importance_flag[ i ]. In other words, po_sei_processing_degree_flag[ i ] would not be used unless po_sei_importance_flag[ i ] is equal to 1 in the current text in JVET-AI2006. And the semantics text for po_sei_processing_degree_flag[ i ] is tied with that of po_sei_importance_flag[ i ] as if a third case of the latter.

To simplify the processing importance related signalling, this contribution proposes to merge the flag of po_sei_processing_degree_flag[ i ] into the flag of po_sei_importance_flag[ i ] and rename po_sei_importance_flag[ i ] to po_sei_importance_idc[ i ]. Two-bit signalling is used for po_sei_importance_idc[ i ] to specify the three cases including the case considering  po_sei_processing_degree_flag[ i ] in JVET-AI2006.

The proposed changes are as follows as highlighted in green with the strikethrough text in red.

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_for_human_viewing_idc | u(2) |
| po_for_machine_analysis_idc | u(2) |
| po_reserved_zero_4bits | u(4) |
| po_num_sei_messages_minus2 | u(7) |
| po_breadth_first_flag | u(1) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flagidc[ i ] | u(12) |
| po_sei_processing_degree_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(12) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| … |  |
| } |  |

…

po_sei_importance_flagidc[ i ] equal to 1 or 2 affects the derivation of PoSeiList, which is the list of SEI messages that a decoding system should process for a particular picture picA, as specified below. po_sei_importance_flagidc[ i ] equal to 3 is reserved for future use.

po_sei_importance_flagidc[ i ] equal to 0 specifies that when the decoding system cannot interpret or does not support the functionality indicated by the i-th SEI message type, it shall ignore all data associated with the loop variable value of i and exclude the i-th SEI message type from the processing chain performed by the decoding system.

po_sei_processing_degree_flag[ i ] affects the derivation of PoSeiList as specified below.

PoSeiList is derived as follows:

–	PoSeiList is initially empty.

–	The following applies in a non-decreasing order of po_sei_processing_order[ i ] values for all values of i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive, unless terminated earlier as specified below:

–	When an SEI message seiA associated with the i-th SEI message type persists for picA, the following applies:

–	If the decoding system can interpret and supports the functionality indicated by seiA, seiA is added at the end of PoSeiList.

–	Otherwise, if po_sei_importance_flagidc[ i ] is equal to 2,1, and po_sei_processing_degree_flag[ i ] is equal to 0, the derivation of PoSeiList is terminated.

–	Otherwise, if po_sei_importance_flagidc[ i ] is equal to 1 and po_sei_processing_degree_flag[ i ] is equal to 1, the processing chain specified by this SPO SEI message should not be performed for picA, PoSeiList is set to be empty, and the derivation of PoSeiList is terminated.

…

# References

- “Additional SEI messages for VSEI version 4 (Draft 3)”, JVET-AI2006-v3, JVET 35th (AI) meeting, July 2024, Sapporo, Japan.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).