| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0072-v1 |
| --- | --- |

| Title: | AHG9: Danmaku information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang, Li Zhang 8910 University Center Ln San Diego, CA92122, USA | Email: | xujizheng@bytedance.com yekui.wang@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a new SEI message to carry danmaku information in a video bitstream.

# Introduction

Danmaku (or Danmu) is a subtitle system widely used in online video platforms. Its history can be traced back to 2006 by Niconico, a video-sharing website in Japan. It was popularized in China later. Nowadays, danmaku system is supported by most major video sharing or streaming platforms in China.

Usually, a danmaku is displayed on top of a video, scrolling from right to left. An audience can see the text without shifting one's focus outside of the video. Because it is scrolling from right to left, one can even focus on a fixed spot and still perceive the whole text easily.

For a video shared on a platform, its danmakus are created by different users. Uses can share their comments, feeling, emotions by sending danmakus. One can send a danmaku at any time of the video. It can be in different colours and with different speed to scroll across the video.

Danmakus are stored associated with a video. When a video with danmakus is shown on a device, a group of danmakus within a period are sent to the device and the renderer can choose to how to render danmakus according to the device capability, user options, etc. For example, a renderer can detect human figures in a video and render danmakus outsides of the foreground.

A danmaku system increases involvements among users and give videos social characteristics. For a popular or controversial video, the number of danmakus can be very large. It is not difficult to find videos with millions of danmakus. To our knowledges, the highest number claimed reaches 200 million for a single video.

# Design thoughts for the danmaku SEI message

It is proposed to use a dedicated SEI message to represent danmakus for a video, which has the following characteristics:

- Support widely used danmaku functionality, including supporting different colours, different start times and durations.

- Compact representation.

- Support different languages.

- Support a large number of danmakus.

# Proposed text

The texts below are proposed to be added to the draft VSEI document based on JVET-AI2032-v3.

8.46 Danmaku information SEI message

8.46.1 Danmaku information SEI message syntax

| danmaku_info( payloadSize ) { | Descriptor |
| --- | --- |
| di_cancel_flag | u(1) |
| if( !di_cancel_flag ) { |  |
| di_num_danmaku_minus1 | ue(v) |
| di_num_languages_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| di_bit1_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= di_num_languages_minus1; i++ ) |  |
| di_language[ i ] | st(v) |
| for( i = 0; i <= di_num_danmaku_minus1; i++ ) { |  |
| di_position[ i ] | u(4) |
| di_colour_idc[ i ] | u(4) |
| if( di_colour_idc[ i ] = = 15 ) |  |
| di_colour_rgb[ i ] | u(24) |
| di_start_delta_poc[ i ] | ue(v) |
| di_end_delta_poc[ i ] | ue(v) |
| if( di_num_languages_minus1 != 0 ) |  |
| di_lang_idx[ i ] | ue(v) |
| } |  |
| while( !byte_aligned( ) ) |  |
| di_bit2_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= di_num_danmaku_minus1; i++ ) |  |
| di_text[ i ] | st(v) |
| } |  |
| } |  |

8.46.2 Danmaku information SEI message semantics

The danmaku information (DI) SEI message provides danmaku information.

The DI SEI message persists for the current picture until the next picture in output order that is associated with another DI SEI message or the end of the CLVS, whichever is earlier.

di_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous danmaku SEI message associated with a picture earlier than the current picture in output order. di_cancel_flag equal to 0 indicates that danmaku information follows.

di_num_danmaku_minus1 plus 1 specifies the number of danmaku in this SEI message. The value of di_num_danmaku_minus1 shall be in the range of 0 to 232 − 2, inclusive.

di_num_languages_minus1 plus 1 specifies the number of languages for the danmaku in this SEI message. The value of di_num_languages_minus1 shall be in the range of 0 to 63, inclusive.

di_bit1_equal_to_zero shall be equal to 0.

di_language[ i ] contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the di_language[ i ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

di_position[ i ] specifies the suggested display method for the i-th danmaku. The list of display methods is shown in the following table.

Table x– Mapping of di_position[ i ] to the display method

| di_position[ i ] | display method |
| --- | --- |
| 0 | Unspecified |
| 1 | Top |
| 2 | Bottom |
| 3 | Scrolling from right to left |
| 4 | Scrolling from left to right |
| 5-15 | Reserved |

di_colour_idc[ i ] specifies the colour index for the i-th danmaku. The rgb colour value for index from 0 to 14, inclusive, is shown in the following table.

Table x– Mapping of di_colour_idc to the danmaku colour

| di_colour_idc[ i ] | colour name | colour(0xrrggbb) |
| --- | --- | --- |
| 0 | white | 0xffffff |
| 1 | red | 0xff0000 |
| 2 | green | 0x00ff00 |
| 3 | blue | 0x0000ff |
| 4 | yellow | 0xffff00 |
| 5 | magenta | 0xff00ff |
| 6 | cyan | 0x00ffff |
| 7 | gray | 0x808080 |
| 8 | olive | 0x808000 |
| 9 | dark blue | 0x000080 |
| 10 | turquoise | 0x008080 |
| 11 | purple | 0x800080 |
| 12 | dark green | 0x008000 |
| 13 | dark red | 0x800000 |
| 14 | black | 0x000000 |
| 15 | escape | 0xrrggbb |

di_colour_rgb[ i ] specifies the rgb colour for the i-th danmaku when di_colour_idc[ i ] is equal to 15. The format of the RGB value is as 0xrrggbb, i.e., the 8 MSBs represent the red colour component, the 8 LSBs represent the blue colour component, and the 8 bits in the middle represent the green colour component.

di_start_delta_poc[ i ] plus the POC value of the current picture specifies the value of the varaiable startPoc[ i ]. If there is a picture picA in the CLVS with POC equal to startPoc[ i ], the display of i-th danmaku should start from the picture picA. Otherwise (there is no picture in the CLVS with POC equal to startPoc[ i ]), the display of the i-th danmaku should start from the picture in the CLVS with the smallest POC value among all pictures with POC greater than startPoc[ i ]. The value of di_start_delta_poc[ i ] shall be in the range of 0 to 65 535, inclusive.

di_end_delta_poc[ i ] plus di_start_delta_poc[ i ] plus the POC value of the current picture specifies the value of the variable endPoc[ i ]. If there is a picture picB in the CLVS with POC equal to endPoc[ i ], the display of the i-th danmaku should end at the picture picB. Otherwise (there is no picture in the CLVS with POC equal to endPoc[ i ]), the display of the i-th danmaku should end at the picture in the CLVS with the greatest POC value among all pictures with POC less than endPoc[ i ]. The value of di_end_delta_poc[ i ] shall be in the range of 0 to 65 535, inclusive.

di_lang_idx[ i ] specifies that language of the i-th danmaku is indicated by di_language[ di_lang_idx[ i ] ]. When not present, di_lang_idx[ i ] is inferred to be 0.

di_bit2_equal_to_zero shall be equal to 0.

di_text[ i ] specifies the text for the i-th danmaku.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).