| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0074-v1 |
| --- | --- |

| Title: | AHG9: On the bitdepth range information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang 8910 University Center Ln San Diego, CA92122, USA | Email: | xujizheng@bytedance.com yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes for the bit-depth range information SEI message:

- Use index of a constituent rectangle instead of its ID.

- Add a constraint to avoid referencing an EMPTY constituent rectangle.

- Add a constraint to avoid a subpicture or constituent rectangle being referenced by multiple ranges.

- Fix the constraint on the sum of all range's bit depth.

- Infer the values of bri_nuh_layer_id_present_flag, bri_pic_partition_flag, and bri_partition_type_flag when they are not present.

- Signal the chroma format of the original source picture and clarify the handling of different chroma formats.

- Add a colour component index to specify which colour component may be used for a range.

- Various editorial improvements.

The detailed proposed text changes, marked relative to JVET-AI2032-v3, are included in an attachment to this contribution.

# Explanation and proposed text changes

The following description and text changes are based on the document JVET-AI2032-v3.docx. Forr the text changes, relevant parts that have been added or modified are highlighted in turquoise, and some of the deleted parts are highlighted in yellow strikethrough in red fonts.

Item 1)

When bri_partition_type_flag is equal to 1, bri_partition_id is used to reference a constituent rectangle's ID. However, there is a constraint as "bri_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ bri_nuh_layer_id[ i ] ] – 1". A constituent rectangle's ID can be out of such a range. Thus, it is proposed to reference a constituent rectangle's index instead of its ID. Such a change also unifies the case when bri_partition_type_flag is equal to 0, which indicates subpicture is used.

Item 2)

According to the description of constituent rectangles, a constituent rectangle with EMPTY type should not be used. Thus, it is proposed to have a corresponding constraint when a constituent rectangle is referenced.

Item 3)

For two different ranges, when they have the same bri_nuh_layer_id and the same bri_partition_id, they are referencing a same constituent rectangle or a same subpicture, which shall be disallowed. A related constraint is needed.

The proposed text changes for items 1), 2) and 3) are as below:

bri_partition_idx_len_minus1 + 1 specifies the length of the bri_rect_id[ i ] and bri_subpic_id[ i ] bri_partition_idx[ i ] syntax elements.

bri_partition_idx[ i ], when present and bri_partition_type_flag equal to 1, specifies the cr_rect_id[ j ] constituent rectangle index of the texture component of the i-th display overlay range. bri_partition_idx[ i ], when present and bri_partition_type_flag equal to 0, specifies the subpicture index of the texture component of the i-th display overlay range.  When not present, the value of bri_partition_idx[ i ] is inferred to be equal to 0.

When bri_partition_type_flag is equal to 1, bri_partition_idx[ i ] shall be in the range of 0 .. to cr_num_rects_minus1[ bri_nuh_layer_id[ i ] ] – 1, inclusive. The value of cr_rect_type_idc of the constituent rectangle specified by bri_partition_idx[ i ] shall not be equal to 255. When bri_partition_type_flag is equal to 0, bri_partition_idx[ i ] shall be in the range of 0 to NumSubpics[ bri_nuh_layer_id[ i ] ] – 1, inclusive.

It is a requirement of bitstream conformance that, for any two different values of i and j in the range of 0 to briNumRanges − 1, inclusive, when bri_nuh_layer_id[ i ] is equal to bri_nuh_layer_id[ j ], bri_partition_idx[ i ] shall not be equal to bri_partition_idx[ j ].

Item 4)

It is asserted that the current constraint on the sum of each range's bit depth is incorrect.

The proposed text changes for item 4) are as below:

bri_num_bits_in_range_minus1[ i ] + 1 specifies the number of bits in the i-th range. bri_num_bits_in_range_minus1[ i ] shall be in the range of 0 to BitDepth[ i ] – 1, inclusive.

The variable NumBits[ i ] is set equal to bri_num_bits_in_range_minus1[ i ] + 1.

The sum of bri_num_bits_in_range_minus1[ i ] NumBits[ i ] for i in the range of 0 to bri_num_ranges_minus1, inclusive,  shall be less than or equal to OrigBitDepth.

Item 5)

When the number of ranges is 1, bri_nuh_layer_id_present_flag, bri_pic_partition_flag and bri_partition_type_flag are not signalled but used later. It is proposed to assign inferred values for these flags.

The proposed text changes for item 4) are as below:

bri_nuh_layer_id_present_flag equal to 1 specifies that the bri_nuh_layer_id[ i ] syntax element is present in the SEI message. bri_nuh_layer_id_present_flag equal to 0 specifies that the bri_nuh_layer_id[ i ] syntax element is not present in the SEI message. When not present, the value of bri_nuh_layer_id_present_flag is inferred to be equal to 0.

bri_pic_partition_flag equal to 1 specifies that the bri_partition_idx[ i ] syntax element is present. bri_pic_partition_flag equal to 0 specifies that the bri_partition_idx[ i ] syntax element is not present. When not present, the value of bri_pic_partition_flag is inferred to be equal to 0.

bri_partition_type_flag equal to 1 specifies that each range is coded as a constituent rectangle. bri_partition_type_flag equal to 0 specifies that each range is coded as a subpicture. When not present, the value of bri_partition_type_flag is inferred to be equal to 0.

When bri_partition_type_flag equal to 1, it is a requirement of bitstream conformance that there is a constituent rectangles SEI message preceding the BRI SEI message in decoding order in the current PU.

Item 6)

When the chroma format of the original source picture and the chroma format of each range are different, how to handle is not clear, not to mention that some chroma format combinations do not make sense, e.g. the original source picture is of 4:4:4 colour format and all range's colour format is of 4:0:0. Thus, it is propose to limit colour format combinations and describe how to handle the case of different chroma formats. To support it, the chroma format of the original source picture is also signalled.

The proposed text changes for item 6) are as below:

| bitdepth_range_info( payloadSize ) { | Descriptor |
| --- | --- |
| bri_orig_bit_depth_minus1 | u(4) |
| bri_orig_chroma_format_idc | u(2) |
| ... |  |

bri_orig_chroma_format_idc specifies the chroma format of the original source picture.

...

When bri_orig_chroma_format_idc is not equal to 0, it is a requirement of bitstream conformance that ChromaFormatIdc[ i ], for i in the range of 0 to briNumRanges − 1, inclusive, shall not be 0. The samples SampleToCode[ i ] of CodedRangeRegion[ i ], for i in 0 .. briNumRanges − 1, are indicated to have been pre-processed prior to encoding from an original source picture of resolution CodedWidth[ 0 ] x CodedHeight[ 0 ] with chroma format ChromaFormatIdc after a chroma format conversion process and bit depth OrigBitDepth, according to the process of subclause 8.X.2.1.

Each sample, TargetSample, of a target picture of resolution CodedWidth[ 0 ] x CodedHeight[ 0 ] with chroma format ChromaFormatIdc and bit depth OrigBitDepth, may be reconstructed using the Target sample reconstruction process described in subclause 8.X.2.2, for each sample, DecodedSample[ i ] of CodedRangeRegion[ i ] for i in 0 .. briNumRanges − 1. A chroma format conversion process from the chroma format being ChromaFormatIdc[ i ], for i in the range of 0 to briNumRanges − 1, inclusive, to the chroma format being ChromaFormatIdc may be followed.

Item 7)

When the chroma format of the original source picture is of 4:0:0, for example, depth data, which is the major use case for bitdepth range SEI message, using multple subpictures or constituent rectangles will increase the size of the picture. While the chroma components are not used. By using chroma components for different ranges can keep the resolution. It is noted that using different quantization paramters for different colour components is common, which fits the requirement that different ranges use different quantization paramters. Thus, it is proposed to allow using chroma components to carry depth data in the SEI message. To support it, a colour component index is signalled.

The proposed text changes for item 7) are as below:

| bitdepth_range_info( payloadSize ) { | Descriptor |
| --- | --- |
| bri_orig_bit_depth_minus1 | u(4) |
| bri_orig_chroma_format_idc | u(2) |
| bri_num_ranges_minus1 | u(2) |
| if ( bri_num_ranges_minus1 > 0 ) { |  |
| bri_nuh_layer_id_present_flag | u(1) |
| bri_pic_partition_flag | u(1) |
| if( bri_pic_partition_flag ) { |  |
| bri_partition_type_flag | u(1) |
| bri_partition_id_len_minus1 | u(4) |
| } |  |
| } |  |
| for( i = 0; i <= bri_num_ranges_minus1; i++ ) { |  |
| if ( bri_nuh_layer_id_present_flag ) |  |
| bri_nuh_layer_id[ i ] | u(6) |
| if ( bri_pic_partition_flag ) |  |
| bri_partition_idx[ i ] | u(v) |
| if( bri_orig_chroma_format_idc = = 0 && ChromaFormatIdc[ i ] != 0 ) |  |
| bri_comp_idx[ i ] | u(2) |
| bri_num_bits_in_range_minus1[ i ] | u(4) |
| bri_bit_offset[ i ] | u(4) |
| } |  |
| } |  |

bri_comp_idx[ i ], when bri_orig_chroma_format_idc is equal to 0, specifies the colour component index of the i-th range. The value of bri_comp_idx[ i ] shall be in the range of 0 to 2, inclusive. When not present, the value of bri_comp_idx[ i ] is inferred to be equal to 0.

When bri_orig_chroma_format_idc is equal to 0, the samples SampleToCode[ i ] of the colour component indicated by bri_comp_idx[ i ] of CodedRangeRegion[ i ], for i in the range of 0 to briNumRanges − 1, inclusive, are indicated to have been pre-processed prior to encoding from an original source picture of resolution CodedWidth[ 0 ] by CodedHeight[ 0 ] with chroma format ChromaFormatIdc being 0 and bit depth OrigBitDepth, according to the process of subclause 8.X.2.1. Each sample, TargetSample, of a target picture of resolution CodedWidth[ 0 ] by CodedHeight[ 0 ] with chroma format ChromaFormatIdc being 0 and bit depth OrigBitDepth, may be reconstructed using the target sample reconstruction process described in subclause 8.X.2.2, for each sample, DecodedSample[ i ] of the colour component indicated by bri_comp_idx[ i ] of CodedRangeRegion[ i ] for i in the range of 0 to briNumRanges − 1, inclusive.

Item 8)

Some editorial improvements, e.g. typos fixed, are also proposed.

The enclosed attachment document includes all changes for items 1) to 8).

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).