| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0076-v1 |
| --- | --- |

| Title: | AHG9: On the display overlays SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang 8910 University Center Ln San Diego, CA92122, USA | Email: | xujizheng@bytedance.com yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes for the display overlays SEI message:

- Use index of a constituent rectangle instead of its ID.

- Add a constraint to avoid referencing an EMPTY constituent rectangle.

- Add a constraint to avoid a subpicture or constituent rectangle being referenced by multiple ranges.

# Explanation and proposed text changes

The following description and text changes are based on the document JVET-AI2032-v3.docx. For the text changes, relevant parts that have been added or modified are highlighted in turquoise, and some of the deleted parts are highlighted in yellow strikethrough in red fonts.

Item 1)

When doi_partition_type_flag is equal to 1, doi_partition_id is used to reference a constituent rectangle's ID. However, there is a constraint as "doi_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ bri_nuh_layer_id[ i ] ] – 1". A constituent rectangle's ID can be out of such a range. Thus, it is proposed to reference a constituent rectangle's index instead of its ID. Such a change also unifies the case when doi_partition_type_flag is equal to 0, which indicates subpicture is used.

Item 2)

According to the description of constituent rectangles, a constituent rectangle with EMPTY type should not be used. Thus, it is proposed to have a corresponding constraint when a constituent rectangle is referenced.

Item 3)

For two different ranges, when they have the same doi_nuh_layer_id and the same doi_partition_id, they are referencing a same constituent rectangle or a same subpicture, which shall be disallowed. A related constraint is needed.

The proposed text changes for items 1), 2) and 3) are as below:

doi_partition_idx[ i ], when present and doi_partition_type_flag equal to 1, specifies the constituent rectangle index that represents the texture component of the i-th display overlay is represented by the j-th constituent rectangle when cr_rect_id[ j ] equal doi_partition_id[ i ]. doi_partition_idx[ i ], when present and doi_partition_type_flag equal to 0, specifies the subpicture index of the texture component of the i-th display overlay.  When not present, the value of doi_partition_idx[ i ] is inferred to be equal to 0.

When doi_partition_type_flag equal to 1, doi_partition_idx[ i ] shall be in the range of 0 ..to  cr_num_rects_minus1[ doi_nuh_layer_id[ i ] ] – 1, inclusive. The value of cr_rect_type_idc of the constituent rectangle specified by doi_partition_idx[ i ] shall not be equal to 255. When doi_partition_type_flag is equal to 0, doi_partition_idx[ i ] shall be in the range of 0 ..to NumSubpics[ doi_nuh_layer_id[ i ] ] – 1, inclusive.

It is a requirement of bitstream conformance that, for any two different values of i and j in the range of 0 to doi_num_display_overlays_minus2 + 1, inclusive, when doi_nuh_layer_id[ i ] is equal to doi_nuh_layer_id[ j ], doi_partition_idx[ i ] shall not be equal to doi_partition_idx[ j ].

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).