| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0077 |
| --- | --- |

| Title: | EE2-2.1: Matrix-based position dependent intra prediction for TIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Test 2.1 (matrix-based position dependent intra prediction for TIMD) in the context of Exploration Experiments 2 (JVET-AI2024).

This test studies matrix-based position-dependent intra prediction (PDP) for template-based intra mode derivation (TIMD). PDP has been adopted in ECM-13.0, which replaces part of the conventional intra prediction modes (IPMs) in the regular intra prediction with alternative IPMs using a matrix of weights defined for a block shape and IPMs. TIMD still uses conventional IPMs, so the performance when applying PDP for TIMD is investigated in this test.

On top of the ECM-14.0, simulation results {Y, U, V, EncT, DecT} for overall test sequences in CTC are reported below:

AI: {-0.06%	, -0.06%, -0.04%, 100.0%, 102.2%};

RA: {-0.04%, -0.09%, 0.07%, 100.2%, 100.3%};

LB: {0.03%, 0.08%, -0.58%, 100.2%, 100.6%};

Introduction

In this test, PDP for TIMD is introduced, i.e., the conventional IPMs are replaced with PDP in TIMD. In addition, PDP is employed in template cost calculation for TIMD. Furthermore, the proposed method is leveraged to the inter prediction tools using TIMD, such as CIIP and GPM intra. Details of the proposed method are as follows:

- Signalling is kept as the conventional TIMD.

- Angular modes replaced by PDP in TIMD are different from those in the regular intra prediction since the number of IPMs in TIMD is double of regular intra prediction. Specifically, PDP is only used for mode 0, 1, and (2+4*k) for coding blocks with width and height up to 16 samples; otherwise, PDP is only used for mode 0, 1, and (2+8*k).

- Template cost for TIMD is calculated by applying PDP to reference samples of the template, as shown in Figure 1.

- PDPC is not applied to the PDP samples in TIMD, just as PDPC is not applied to the PDP samples in regular intra prediction.

Figure 1 PDP (left) and PDP for template cost calculation in TIMD (right)

Experimental results

The proposed method was implemented on top of ECM-14.0 [1][2] and tested according to the common test condition of all-intra, random-access and low-delay B configurations as specified in the JVET-AI2017 [4] and JVET-AI2024 [5].

Table 1 EE2-2.1 results over ECM-14.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.05% | -0.12% | 99.8% | 101.0% | 98.4% | 99.9% |
| Class A2 | -0.05% | -0.14% | -0.07% | 100.4% | 101.4% | 99.2% | 99.9% |
| Class B | -0.05% | -0.09% | -0.05% | 100.1% | 102.3% | 102.1% | 100.2% |
| Class C | -0.04% | -0.15% | -0.09% | 99.9% | 103.1% | 100.0% | 100.0% |
| Class E | -0.08% | 0.05% | 0.14% | 99.6% | 103.1% | 100.0% | 100.0% |
| Overall | -0.06% | -0.06% | -0.04% | 100.0% | 102.2% | 100.2% | 100.0% |
| Class D | -0.05% | -0.03% | -0.14% | 100.2% | 103.1% | 100.0% | 100.0% |
| Class F | -0.01% | -0.05% | 0.00% | 99.7% | 101.1% | 98.1% | 100.1% |
| Class TGM | 0.00% | 0.00% | 0.00% | 99.9% | 99.8% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.21% | -0.16% | 100.2% | 100.6% | 100.0% | 100.3% |
| Class A2 | 0.01% | -0.03% | 0.09% | 100.2% | 100.1% | 99.9% | 100.2% |
| Class B | -0.04% | -0.09% | 0.13% | 100.3% | 100.2% | 100.0% | 100.0% |
| Class C | -0.06% | -0.05% | 0.15% | 100.1% | 100.4% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.09% | 0.07% | 100.2% | 100.3% | 100.0% | 100.1% |
| Class D | -0.07% | -0.17% | -0.53% | 99.7% | 100.9% | 100.3% | 100.0% |
| Class F | 0.04% | 0.03% | 0.02% | 100.3% | 100.4% | 100.0% | 100.1% |
| Class TGM | 0.00% | 0.00% | 0.00% | 99.9% | 100.3% | 100.0% | 100.0% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.21% | -0.34% | 100.1% | 100.7% | 100.0% | 100.0% |
| Class C | 0.05% | -0.01% | -0.05% | 100.1% | 100.8% | 100.0% | 100.0% |
| Class E | 0.00% | -0.03% | -1.68% | 100.3% | 100.4% | 99.6% | 100.2% |
| Overall | 0.03% | 0.08% | -0.58% | 100.2% | 100.6% | 99.9% | 100.0% |
| Class D | 0.11% | 0.36% | 0.45% | 100.1% | 99.7% | 100.6% | 100.0% |
| Class F | -0.05% | -0.22% | -0.11% | 100.1% | 100.4% | 100.6% | 99.9% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100.0% | 100.1% | 100.0% | 100.0% |

Conclusion

This contribution proposes a matrix-based position-dependent prediction (PDP) for template-based intra-mode derivation (TIMD). The proposed method brings some coding gains with minor increment of decoding time compared to ECM-14.0 under all-intra and random-access configurations. It is recommended to adopt the proposed method into the next ECM.

Reference

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-14.0

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 14 (ECM 14),” JVET-AI2025, July 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October. 2023.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AH2024, April 2024.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).