| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0079-v1 |
| --- | --- |

| Title: | EE2-2.7: Disabling the combination of TIMD-ISP and TIMD-MRL |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zheming Fan Takeshi Chujoh Tomohiro Ikai | Tel: Email: | fan.zheming@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes disabling the TIMD-related combinations (TIMD-ISP and TIMD-MRL). Test2.7a involves disabling the combination of TIMD and ISP (TIMD-ISP) and the combination of TIMD and MRL (TIMD-MRL) in normative way. Test2.7b involves disabling the combinations of TIMD in non-normative way. This change eliminates ineffective combinations which can reduce encoding time by decreasing the number of RDOs. Test2.7c builds upon Test2.7a by replacing the reduced RDOs with other intra modes to achieve gains without increasing EncT. Test2.7d builds upon Test2.7b by replacing the reduced RDOs with other intra modes.

Test 2.7a:

[0.02%, 0.01%, 0.05%, 97.5%, 99.9%] in AI , [0.01%, -0.02%, -0.03%, 100.2%, 99.8%] in RA

Test 2.7b:

[0.04%, 0.01%, 0.04%, 97.5%, 99.9%] in AI , [0.02%, -0.09%, 0.02%, 100.1%, 99.9%] in RA

Test 2.7c:

[-0.10%, -0.04%, 0.00%, 99.8%, 100.5%] in AI , [-0.05%, -0.16%, 0.02%, 100.4%, 100.2%] in RA

Test 2.7d:

[-0.08%, -0.07%, 0.02%, 99.6%, 100.5%] in AI , [-0.06%, -0.14%, 0.05%, 100.3%, 100.0%] in RA

Proposal

This proposal suggests disabling the combination of TIMD and ISP (TIMD-ISP), and the combination of TIMD and MRL (TIMD-MRL). Disabling these combinations will alter the number of bits required for TIMD encoding and decoding. When timd_flag is true, no further encoding or decoding operations will be performed on the corresponding flags for ISP and MRL. After disabling them, the number of RDO required during the encoder's exploration phase will be reduced. Additionally, the proposal suggests reallocating the saved RDOs to other intra prediction modes, potentially leading to performance gains.

Results

Test results are summarized in follow tables.

Table1. Results of test2.7a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | -0.02% | 97.8% | 99.6% | 101.9% | 99.8% |
| Class A2 | 0.01% | -0.05% | 0.06% | 98.0% | 99.7% | 98.8% | 100.0% |
| Class B | 0.03% | 0.01% | 0.07% | 97.3% | 100.4% | 102.0% | 100.3% |
| Class C | 0.03% | -0.05% | -0.06% | 97.3% | 100.3% | 101.0% | 99.9% |
| Class E | 0.00% | 0.14% | 0.20% | 97.2% | 99.3% | 100.0% | 99.9% |
| Overall | 0.02% | 0.01% | 0.05% | 97.5% | 99.9% | 100.9% | 100.0% |
| Class D | 0.02% | -0.01% | -0.01% | 97.2% | 99.8% | 99.9% | 100.0% |
| Class F | -0.02% | -0.25% | -0.23% | 98.2% | 99.0% | 98.9% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.11% | -0.12% | 100.3% | 99.5% | 100.0% | 100.0% |
| Class A2 | 0.04% | 0.02% | 0.06% | 100.5% | 99.8% | 100.0% | 100.0% |
| Class B | 0.01% | 0.07% | 0.00% | 100.0% | 99.9% | 100.0% | 100.0% |
| Class C | 0.01% | -0.09% | -0.07% | 100.2% | 100.0% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.03% | 100.2% | 99.8% | 100.0% | 100.0% |
| Class D | 0.00% | -0.13% | -0.08% | 99.4% | 100.0% | 100.0% | 100.0% |
| Class F | 0.04% | -0.41% | 0.21% | 99.8% | 99.8% | 100.1% | 100.0% |

Table2. Results of test2.7b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | 0.03% | -0.09% | 97.8% | 100.1% | 100.3% | 100.0% |
| Class A2 | 0.01% | 0.03% | 0.02% | 97.8% | 99.0% | 100.0% | 100.0% |
| Class B | 0.04% | 0.02% | 0.12% | 97.4% | 99.9% | 104.1% | 100.3% |
| Class C | 0.07% | -0.08% | -0.06% | 97.3% | 99.8% | 100.0% | 100.0% |
| Class E | 0.06% | 0.06% | 0.21% | 97.3% | 100.5% | 100.0% | 100.1% |
| Overall | 0.04% | 0.01% | 0.04% | 97.5% | 99.9% | 101.2% | 100.1% |
| Class D | 0.08% | 0.12% | 0.15% | 96.9% | 99.3% | 99.9% | 100.0% |
| Class F | -0.02% | 0.04% | 0.01% | 97.9% | 99.0% | 98.9% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.03% | -0.28% | -0.12% | 100.3% | 99.8% | 100.0% | 100.0% |
| Class A2 | 0.02% | 0.00% | 0.07% | 100.3% | 100.0% | 100.0% | 100.0% |
| Class B | 0.01% | -0.05% | 0.02% | 99.9% | 100.0% | 100.0% | 100.0% |
| Class C | 0.02% | -0.05% | 0.10% | 100.1% | 100.0% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.02% | -0.09% | 0.02% | 100.1% | 99.9% | 100.0% | 100.0% |
| Class D | 0.08% | -0.35% | -0.18% | 99.4% | 100.1% | 99.7% | 100.0% |
| Class F | 0.10% | -0.23% | 0.09% | 99.8% | 99.8% | 100.0% | 100.0% |

Table3. Results of test2.7c.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | 0.11% | -0.08% | 99.8% | 100.1% | 99.9% | 100.0% |
| Class A2 | -0.08% | -0.01% | -0.01% | 100.0% | 100.9% | 99.2% | 100.1% |
| Class B | -0.09% | -0.10% | 0.06% | 99.9% | 101.3% | 102.0% | 100.1% |
| Class C | -0.11% | -0.17% | -0.08% | 99.6% | 100.1% | 100.5% | 100.0% |
| Class E | -0.17% | 0.04% | 0.08% | 99.5% | 99.9% | 100.0% | 100.1% |
| Overall | -0.10% | -0.04% | 0.00% | 99.8% | 100.5% | 100.5% | 100.1% |
| Class D | -0.14% | 0.06% | 0.05% | 99.7% | 101.8% | 100.2% | 100.0% |
| Class F | -0.11% | -0.09% | -0.23% | 98.4% | 99.3% | 101.2% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.31% | -0.14% | 100.7% | 99.8% | 100.0% | 100.0% |
| Class A2 | 0.00% | -0.14% | -0.12% | 100.5% | 99.9% | 100.0% | 100.0% |
| Class B | -0.06% | -0.20% | -0.01% | 100.2% | 100.5% | 100.0% | 100.0% |
| Class C | -0.06% | -0.02% | 0.30% | 100.2% | 100.4% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.16% | 0.02% | 100.4% | 100.2% | 100.0% | 100.0% |
| Class D | -0.04% | -0.74% | -0.25% | 99.3% | 100.1% | 99.7% | 101.0% |
| Class F | 0.01% | -0.32% | -0.10% | 99.5% | 100.2% | 100.0% | 101.0% |

Table3. Results of test2.7d.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | 0.06% | -0.04% | 99.6% | 100.0% | 101.2% | 99.9% |
| Class A2 | -0.08% | -0.10% | -0.04% | 99.7% | 100.3% | 99.3% | 100.1% |
| Class B | -0.08% | -0.08% | 0.09% | 99.9% | 101.3% | 95.9% | 100.1% |
| Class C | -0.09% | -0.12% | -0.06% | 99.6% | 99.8% | 100.0% | 99.9% |
| Class E | -0.11% | -0.11% | 0.14% | 99.0% | 100.8% | 98.9% | 99.9% |
| Overall | -0.08% | -0.07% | 0.02% | 99.6% | 100.5% | 98.7% | 100.0% |
| Class D | -0.05% | 0.12% | -0.02% | 99.2% | 99.5% | 100.1% | 100.0% |
| Class F | -0.12% | -0.04% | 0.00% | 98.6% | 99.1% | 99.7% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.16% | -0.08% | 100.4% | 99.4% | 100.0% | 100.0% |
| Class A2 | -0.07% | -0.17% | -0.11% | 100.6% | 100.1% | 100.0% | 100.0% |
| Class B | -0.06% | -0.07% | 0.21% | 100.3% | 100.2% | 100.1% | 101.0% |
| Class C | -0.06% | -0.20% | 0.05% | 100.2% | 100.2% | 100.4% | 101.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.14% | 0.05% | 100.3% | 100.0% | 100.1% | 100.6% |
| Class D | -0.02% | -0.22% | 0.12% | 99.3% | 100.1% | 99.7% | 101.0% |
| Class F | 0.00% | -0.21% | 0.15% | 99.4% | 100.1% | 100.1% | 101.0% |

Conclusion

This contribution reports 4 tests and it is recommended to adopt test2.7c (preferred) or test2.7a:

Test 2.7a: By disabling two TIMD-related combinations in normative way, the average result is a 0.02% luma loss for a 3% EncT reduction.

Test 2.7b: By disabling two TIMD-related combinations in non-normative way, the average result is a 0.04% luma loss for a 3% EncT reduction.

Test 2.7c: Based on Test2.7a, using other intra modes to replace TIMD-ISP and TIMD-MREF in the RDO list, the result is a 0.10% luma gain without changing EncT and DecT.

Test 2.7d: Based on Test2.7b, using other intra modes to replace TIMD-ISP and TIMD-MREF in the RDO list, the result is a 0.8% luma gain without changing EncT and DecT.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).