| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0080-v1 |
| --- | --- |

| Title: | EE1-1.1: Partial Convolution and Over-Parameterization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingwei Chi, Ao Li, Ce Zhu, Lei Luo, Hongwei Guo, Yongkai Huo, Yutian Liu | Tel: Email: | jingwei-chi@std.uestc.edu.cn aoli@std.uestc.edu.cn eczhu@uestc.edu.cn luolei1010@uestc.edu.cn hwguo@uestc.edu.cn yongkai.huo@transsion.com yutian.liu@transsion.com |
| Source: | University of Electronic Science and Technology of China, Transsion Holdings |  |  |

- Abstract

This contribution reports the results of EE1-1.1, which uses partial convolution to reduce model complexity while employing an over-parameterization training strategy to improve model performance. The proposed model has a complexity of 16.9 kMAC/pixel and 203,969 parameters. Stage 3 training of the proposed model is performed.

The performance of the float model (including NNIntra) compared to the NNVC-10.0 anchor (NNIntra+LOP) is reported to be:

AI {-0.11%, 0.88%, 0.84%, Enc: 94%, Dec: 79%},

RA {-0.03%, 1.08%, 1.13%, Enc: 98%, Dec: 75%},

LDB {0.00%, -1.04%, -1.22%, Enc: 96%, Dec: 75%}.

The performance of the int16 model (including NNIntra) compared to the NNVC-10.0 anchor (NNIntra+LOP) is reported to be:

AI {-0.12%, 0.96%, 0.88%, Enc: 98%, Dec: 95%},

RA {-0.07%, 1.18%, 1.26%, Enc: 99%, Dec: 93%},

LDB {-0.01%, 1.09%, 1.42%, Enc: 101%, Dec: 92%}.

It is proposed to adopt the method into NNVC.

Introduction

The current LOP3 model has been studied and trained in JVET-AH0080 [1] and JVET-AH0081 [2]. At the last meeting, we proposed partial convolution and over-parameterization for LOP in JVET-AI0068 [3]. The proposal introduced a partial convolution block to enhance model efficiency and applied an over-parameterization training strategy to improve model performance.

Figure 1 shows the structure of the proposed partial convolution block. It contains two input parameters, C1 and k. C1 indicates the input channel, and k represents the partial factor. Specifically, the input is split into two branches, and the convolution operation is performed only on one of the branches. The output is then obtained by concatenating the two branches.

Figure 2 elaborates the over-parameterization training strategy. The input parameters consist of C1, C2 and p. C1 and C2 indicate the input and output channels, respectively, and p represents the expansion factor. During the training stage, the block is expanded into two convolution layers with the middle channels being (p* C1). Once the training is completed, the block is collapsed into one convolution layer. The whole process can be formulated as follows:

Training Stage:

As shown in Figure 2, for a given input feature X∈RC1×h×w, where C1 is the number of channels, h is the feature height, and w is the feature width, the sizes of the two convolution kernels are W1∈RC2×C1×3×3 and W2∈RC1×C2×1×1, respectively. The convolution process can be expressed as:

Xout=(X*W1)*W2,                                                               (1)

where ∗ represents the convolution process, and  Xout∈RC1×h×w.

Inference Stage:

According to the property of convolution, the convolution process in Equation (1) can be re-expressed as:

Xout=X*W1*W2=X*Wnew,                                                (2)

where Wnew∈RC1×C1×3×3 indicates the fused weight. Through the above transformation, it is possible to use a new convolution kernel to replace the two previous kernels, thereby completing the over-parameterization process.

Figure 1 Partial Convolution block

Figure 2 Over-Parameterization Block

Proposed method

In this contribution, as it was reported in JVET-AH0077 [3], it is proposed to employ partial convolution and over-parameterization for LOP.

As shown in Figure 3, we integrate the Over-Parameterization Block (OP Block) and Partial Convolution Block (PC Block) and assess the efficacy of these two blocks. Specifically, in the training stage, we expand the 3x3 convolution layer in the PC Block to both 3x3 and 1x1 convolution layers. During the inference stage, these two convolution layers are merged into a single 3x3 convolution layer. The proposed modifications to the model parameters are as follows:

Head block:

- d1: 16

- d2: 8

- d3: 4

- d4: 2

- d5: 2

- d6 (=C): 64

- k: 0.5

Backbone block:

- Luma

  - NY: 14à13

  - CY: 32

  - C1Y: 144

  - C21Y: 32

- Chroma

  - NUV: 4à2

  - CUV: 32

  - C1UV: 128à80

  - C21UV: 32

This results in a complexity of 16.9 kMAC/pixel and 203,969 parameters for EE1-1.1, with 65 convolutional layers. The current LOP3 model from JVET-AH0080 [1] has a complexity of 16.9 kMAC/pixel, where the number of parameters is 205,108 and the number of convolution layers is 94.

Figure 3 LOP3 with OP Blocks and PC Blocks (EE1-1.1).

Training

The training information is shown in Table 1.

| Table 1. Network Information for the model in EE1-1.1 in training. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type |  |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 |
|  | Batch size: | Stage3: 64 |
|  | Loss function: | L1 and MSE |
|  | Training time: | Stage3: 14 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | The LOP stage 3 dataset |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage3: 2e-4, milestones [41, 51, 55] |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

Inference

| Table 2. Network Information for the EE1-1.1 model in inference. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 203969 |
|  | Parameter Precision (Bits) | fp32, int16 |
|  | Memory Parameter (MB) | fp32: 0.4MB, int16: 0.2MB |
|  | Multiply Accumulate (MAC) | 16.9 kMAC/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 65 |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

The number of parameters and complexity is consistent between the excel sheet attached in the contribution and the thop library [4].

Results

The proposed model is tested on top of NNVC-10.0. The changes include the NN model and the parameter selection. The results are recorded according to the NNVC CTC [5].

Comparison to NNVC-10.0 (float) with EE1-1.1

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.04% | 0.92% | 1.68% | 100% | 77% |
| Class A2 | -0.10% | 0.33% | 0.77% | 100% | 75% |
| Class B | -0.03% | 1.60% | 1.17% | 98% | 75% |
| Class C | -0.03% | 1.11% | 0.96% | 95% | 75% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 1.08% | 1.13% | 98% | 75% |
| Class D | -0.05% | 1.01% | 1.79% | 90% | 75% |
| Class F | 0.05% | 1.50% | 0.05% | 93% | 74% |
|  | Low delay B |  |  |  |  |
|  | BD-rate Over NNVC-10.0 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | -0.44% | -0.37% | 97% | 75% |
| Class C | 0.03% | -0.90% | -0.77% | 97% | 75% |
| Class E | -0.22% | -2.22% | -3.25% | 92% | 75% |
| Overall | 0.00% | -1.04% | -1.22% | 96% | 75% |
| Class D | 0.16% | -1.97% | -0.01% | 91% | 76% |
| Class F | 0.20% | 1.30% | 0.38% | 93% | 74% |

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.11% | 1.08% | 1.12% | 93% | 77% |
| Class A2 | -0.04% | 0.45% | 0.52% | 94% | 78% |
| Class B | -0.09% | 0.90% | 0.81% | 93% | 79% |
| Class C | -0.07% | 0.95% | 0.85% | 94% | 81% |
| Class E | -0.26% | 0.99% | 0.90% | 94% | 79% |
| Overall | -0.11% | 0.88% | 0.84% | 94% | 79% |
| Class D | -0.07% | 1.01% | 0.96% | 91% | 81% |
| Class F | -0.12% | 2.22% | 0.20% | 97% | 78% |

The encoding time and decoding time is reliable.

Comparison to NNVC-10.0 (int16) with EE1-1.1

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.09% | 1.06% | 1.65% | 97% | 95% |
| Class A2 | -0.04% | 0.33% | 0.82% | 96% | 92% |
| Class B | -0.09% | 1.64% | 1.44% | 97% | 93% |
| Class C | -0.07% | 1.32% | 1.08% | 104% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 1.18% | 1.26% | 99% | 93% |
| Class D | -0.07% | 1.46% | 1.53% | 100% | 92% |
| Class F | -0.06% | 1.73% | 0.13% | 105% | 94% |
|  | Low delay B |  |  |  |  |
|  | BD-rate Over NNVC-10.0 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.44% | -0.45% | 99% | 93% |
| Class C | -0.01% | -0.34% | -1.08% | 103% | 92% |
| Class E | -0.09% | -3.17% | -3.49% | 102% | 92% |
| Overall | -0.01% | -1.09% | -1.42% | 101% | 92% |
| Class D | -0.03% | -1.10% | -0.88% | 106% | 93% |
| Class F | -0.01% | 2.11% | -2.06% | 102% | 91% |

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.16% | 1.03% | 1.02% | 97% | 94% |
| Class A2 | -0.03% | 0.52% | 0.62% | 98% | 95% |
| Class B | -0.10% | 0.96% | 0.91% | 97% | 95% |
| Class C | -0.09% | 1.07% | 0.83% | 99% | 96% |
| Class E | -0.24% | 1.20% | 1.00% | 98% | 95% |
| Overall | -0.12% | 0.96% | 0.88% | 98% | 95% |
| Class D | -0.12% | 1.57% | 1.23% | 99% | 96% |
| Class F | -0.15% | 2.40% | 0.21% | 98% | 95% |

The encoding time and decoding time is reliable.

Conclusions

This contribution reports the study of EE1-1.1, which employs partial convolution and over-parameterization strategies. The model complexity is 16.9 kMAC/pixel, similar to LOP3, but the number of parameters is decreased from 205,108 to 203,969, and the total convolution layers are reduced from 94 to 65.

Performance comparisons to the NNVC-10.0 anchor show a BD-rate impact of { -0.03%, 1.08%, 1.13%} for RA and {0.00%, -1.04%, -1.22%} for Low delay and {-0.11%,0.88%,0.84%} for AI with the float model. The encoding times compared to the NNVC-10.0 anchor show impacts of {98%, 96%, 94%} for RA, Low delay, AI, respectively. And the decoding times compared to the NNVC-10.0 anchor show impacts of {79%, 75%, 75%} for RA, Low delay, AI, respectively.

For the int16 model, performance comparisons to the NNVC-10.0 anchor show a BD-rate impact of { -0.07%, 1.18%, 1.26%} for RA and {-0.01%,-1.09%,-1.42%} for Low delay and {-0.12%,0.96%,0.88%} for AI. The encoding times compared to the NNVC-10.0 anchor show impacts of {99%, 101%, 98%} for RA, Low delay, AI, respectively. And the decoding times compared to the NNVC-10.0 anchor show impacts of {93%, 92%, 95%} for RA, Low delay, AI, respectively. It is proposed to adopt this method into NNVC.

References

[1] EE1-1.2: Joint LOP model with inputs transformed, JVET-AH0080, D. Liu, J. Ström, M. Damghanian, P. Wennersten, D. Rusanovskyy, Y. Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Bady, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 34th Meeting, Rennes, FR, 17–24 April 2024.

[2] EE1-1.3: Training from scratch for the joint LOP model with inputs transformed, JVET-AH0081, D. Liu, J. Ström, M. Damghanian, P. Wennersten, D. Rusanovskyy, Y. Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 34th Meeting, Rennes, FR, 17–24 April 2024.

[3] EE1-1.2: Partial Convolution and Over-Parameterization, JVET-AI0068, A. Li, J. Chi C. Zhu, L. Luo, Y. Huo, Y. Liu, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024.

[4] https://pypi.org/project/thop/

[5] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-AF2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, 13–20 October 2023.

Patent rights declaration(s)

University of Electronic Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Shenzhen Transsion Holdings Co., Ltd. (Transsion) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).