| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0081-v1 |
| --- | --- |

| Title: | EE2-2.18: Chroma TMRL |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hang Huang, Yue Yu, Haoping Yu, Dong Wang | Tel: Email: | huanghang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the results for EE2-1.18. In EE2-2.18, TMRL (template-based multiple reference line intra prediction) is extended to chroma as a new chroma prediction mode, which was initially proposed in JVET-AI0168. The process of this new chroma prediction mode is similar to TMRL. A candidate list that includes combinations of reference line and intra prediction mode is constructed and reordered. A flag and a list index are signaled to indicate which combination candidate is used to predict the current chroma block if the proposed chroma TMRL mode is selected. The impact on coding efficiency and runtimes over ECM-14.0 is reported as {Y, U, V, EncT, DecT} below:

AI(Y/U/V): 0.00%/-0.30%/-0.27%, EncT 100.2%, DecT 99.7%

RA(Y/U/V): 0.00%/-0.28%/-0.13%, EncT xxx%, DecT xxx%

Introduction

The template-based multiple reference line intra prediction (TMRL) mode combines reference line and prediction mode and uses a template matching method to construct a list of candidate combinations. An index to the candidate combination list is coded to indicate which combination of reference line and prediction mode is used in coding the current block.

Proposed method

In this contribution, a new non-CCP chroma intra prediction mode is proposed.

The proposed chroma TMRL mode includes the following aspects like TMRL:

- the extended reference-line candidate list and the intra-prediction-mode candidate list.

The extended reference-line candidate list consists of {1, 3, 5}.

The size of the intra-prediction-mode candidate list is 8, and the construction process follows the order shown below:

- The non-CCP chroma intra prediction modes in the current ECM including DM, chroma DIMD, DC;

- The collocated luma modes: the intra prediction modes from luma blocks in the positions of TL, TR, BL and BR in the collocated luma block as shown in Figure 1;

- The adjacent chroma modes: the intra prediction modes from adjacent chroma blocks in the positions of L', T', BL', TR', and TL' as shown in Figure 1;

Some default intra modes will be added if the intra-prediction-mode candidate list is not filled fully with the above chroma modes. The default intra modes are the first angular mode in the intra-prediction-mode candidate list shifted with {-1, +1, -2, +2, …}. To remove redundant intra prediction mode in the list, a pruning operation is applied. It is noted that the CCP mode, planar mode, and DBV mode are excluded from the intra-prediction-mode candidate list.

Figure 1 The collocated luma block and adjacent chroma block

- construction of the chroma TMRL combination list.

There are 3x8=24 combinations of the reference line and the allowed intra-prediction modes for a block.

Then the combination candidates in the list are used to predict the collocated luma block and the template of the current chroma block. This template includes one top row and one left column of the current chroma block as shown in Figure 2. The SATD cost between the prediction and the reconstructed pixels is then calculated. The candidates in the list are reordered based on a combined SATD cost, calculated as follows:

cost=8*costY+costCbTop+costCrTop≪logH+2+costCbLeft+costCrLeft≪logW+2

For the collocated luma block, the reference line {1, 3, 5} will be set as {2, 6, 10} accordingly. Only the first N combinations with the smallest cost can be allowed for the current chroma block, N is equal to 6 in this proposal.

Figure 2 Illustration of template area

- signaling of chroma TMRL

A chroma TMRL flag is signaled to indicate whether the chroma TMRL mode is applied or not. If the chroma TMRL mode is applied, an index is signaled to indicate which combination candidate in the list is used.

Simulation results

The proposed method is implemented on top of the ECM-14.0 software [1] and tested based on the common test condition [2]. The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.18% | -0.01% | 100.5% | 98.9% | 99.6% | 100.0% |
| Class A2 | -0.01% | -0.23% | -0.23% | 100.8% | 99.6% | 100.0% | 100.1% |
| Class B | 0.00% | -0.21% | -0.27% | 100.2% | 99.9% | 98.8% | 99.8% |
| Class C | 0.02% | -0.30% | -0.40% | 100.0% | 100.0% | 100.2% | 100.0% |
| Class E | 0.00% | -0.65% | -0.40% | 99.7% | 100.2% | 99.7% | 99.9% |
| Overall | 0.00% | -0.30% | -0.27% | 100.2% | 99.7% | 99.6% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.39% | -0.20% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | 0.03% | -0.28% | -0.22% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | -0.31% | -0.02% | 100.3% | 100.1% | 100.0% | 100.1% |
| Class C | 0.00% | -0.16% | -0.15% | 100.4% | 99.9% | 100.0% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.28% | -0.13% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Conclusions

This contribution proposes a chroma TMRL mode. The results show that the performance of non-CCP chroma intra prediction can be further improved with a minor impact on complexity. It is suggested to adopt the proposed method into ECM.

References

- H. Huang, Y. Yu, H. Yu, D. Wang, “Non-EE2: Chroma TMRL”, JVET-AI0168, July, 2024.

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AI2017, July, 2024.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).