| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0082-v1 |
| --- | --- |

| Title: | EE2-2.19a: Multi-model EIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lai Zhang, Zhihuang Xie, Yue Yu, Haoping Yu, Dong Wang | Tel: Email: | zhanglai@oppo.com xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-2.19a. In test 2.19a, the multi-model EIP method is investigated. On top of ECM-14.0, simulation results are reported as follows:

EE2-2.19a:

AI { -0.07% Y, -0.01% U, -0.03% V, 100.2% EncT, 99.7% DecT}

RA { -0.03% Y, -0.11% U, -0.01% V, 100.1% EncT, 99.9% DecT}

Introduction

Extrapolation filter-based Intra Prediction (EIP) is an adopted coding tool in the current ECM. In the EIP mode, the samples in a CU are predicted from the top-left position to the bottom-right position by applying an extrapolation filter to neighboring reconstructed samples or predicted samples. The EIP mode uses a 15-tap filter for prediction as shown below:

predx,y=i=013(ci×tx-offsetXi, y-offsetYi)+c14×2bitdepth-1

where predx,y is the predicted value at position (x, y) in the CU, ci is the filter coefficient, and the tx-offsetXi, y-offsetYi is the reconstructed sample or predicted sample. Predicted sample values are clipped to the range of the reference samples instead of the full sample value range. The EIP filter can be derived from the neighboring reconstructed samples or can be inherited from the previous EIP-coded blocks (EIP merge mode).

In JVET-AI0167[1], two methods, the multi-model EIP and the block-vector guided EIP, were proposed to further improve the coding efficiency of the EIP mode. Both methods and their combination are investigated in the EE.

Proposed method

In EE2-2.19a, the multi-model EIP method (MM-EIP) is tested. In the MM-EIP mode, the EIP training samples are classified into two groups, and the samples in both groups are used as training sets to derive two separate extrapolation filters, respectively. The classification process is based on the reference value of the EIP training sample and a threshold. The threshold is set as the average value of all the samples in the neighboring reconstructed area, and the reference value of the current EIP training sample is calculated by one or two input samples. An EIP training sample with reference <= threshold is classified into Group1; while an EIP training sample with reference > threshold is classified into Group2. Two extrapolation filters are formulated as below:

predx,y=i=013(c1_i×tx-offsetX1_i, y-offsetY1_i)+c1_14×2bitdepth-1 if reference ≤thresholdpredx,y=i=013(c2_i×tx-offsetX2_i, y-offsetY2_i)+c2_14×2bitdepth-1 if reference >threshold

In addition, the regularization parameters λ is adaptively selected according to the number of training samples when solving the coefficients of MM-EIP filter. The same classification rule is applied when predicting the current block. The prediction sample is calculated by the above formula according to the reference value of the current inputs and the threshold.

The MM-EIP mode is signalled by a flag when the current block’s EIP mode flag is true and EIP-merge mode flag is false. The number of derived EIP filter candidates are modified so that the total number of derived EIP filter candidates (including both EIP and MM-EIP) doesn’t exceed the previous number. The maximum template size of EIP filter derivation process is set to 8.

Simulation results

The proposed method is implemented on top of the ECM-14.0 software [2] and tested based on the common test condition [3]. The test results are summarized below.

EE2-2.19a:

Table 1: Test result of EE2-2.19a over ECM-14.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | -0.05% | -0.24% | 100.3% | 99.8% | 100.4% | 100.5% |
| Class A2 | -0.06% | -0.08% | -0.08% | 100.2% | 99.7% | 100.4% | 100.5% |
| Class B | -0.04% | 0.00% | 0.11% | 100.2% | 99.9% | 102.1% | 100.4% |
| Class C | -0.04% | 0.08% | -0.04% | 100.0% | 99.3% | 101.2% | 100.4% |
| Class E | -0.12% | -0.01% | 0.04% | 100.3% | 99.7% | 102.1% | 100.4% |
| Overall | -0.07% | -0.01% | -0.03% | 100.2% | 99.7% | 101.3% | 100.4% |
| Class D | 0.00% | 0.00% | -0.03% | 100.3% | 100.2% | 100.3% | 100.3% |
| Class F | -0.11% | -0.17% | -0.13% | 98.9% | 99.5% | 100.3% | 100.4% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.21% | -0.15% | 99.9% | 99.9% | 100.1% | 100.6% |
| Class A2 | -0.02% | -0.05% | -0.05% | 99.9% | 99.7% | 100.5% | 100.7% |
| Class B | -0.03% | -0.11% | -0.02% | 100.2% | 100.2% | 100.2% | 100.4% |
| Class C | -0.03% | -0.07% | 0.12% | 100.1% | 99.7% | 100.3% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.11% | -0.01% | 100.1% | 99.9% | 100.3% | 100.5% |
| Class D | -0.06% | -0.18% | -0.22% | 99.7% | 99.9% | 100.6% | 100.3% |
| Class F |  |  |  |  |  |  |  |

Conclusions

It is recommended to adopt the proposed mothed into the next version of ECM.

References

- L. Zhang, Z. Xie, Y. Yu, H. Yu, D. Wang, “Non-EE2: Non-EE2: Improvements on EIP”, JVET-AI0167, July 2024

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AI2017, July 2024

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).