| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0083-v2 |
| --- | --- |

| Title: | EE2-2.19b: Block Vector guided EIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-2.19b. In EE2-2.19b, a block-vector guided EIP method is introduced to further improve the coding performance of EIP. On top of ECM-14.0, the test results are summarized as follows:

AI: -0.05%(Y), -0.03%(U), -0.01(V), 100.5%(EncT), 101.0%(DecT)

RA: -0.03%(Y), -0.08%(U), -0.07%(V), 100.1%(EncT), 100.1%(DecT)

Introduction

Extrapolation filter-based Intra Prediction (EIP) is an adopted coding tool in the current ECM. In the EIP mode, the samples in a CU are predicted from the top-left position to the bottom-right position by applying an extrapolation filter to neighboring reconstructed samples or predicted samples. The EIP mode uses a 15-tap filter for prediction as shown below:

predx,y=i=013(ci×tx-offsetXi, y-offsetYi)+c14×2bitdepth-1

where predx,y is the predicted value at position (x, y) of the CU, ci is the filter coefficient, and the tx-offsetXi, y-offsetYi is the reconstructed sample or predicted sample. Predicted sample values are clipped to the range of the reference samples instead of the full sample value range. The reference sample area used for determining the range is the same area that is used to generate the filter coefficients.

The EIP filter can be derived from the neighboring reconstructed samples or can be inherited from the previous EIP-coded blocks (EIP merge mode). There are three EIP filter shapes and three types of reconstructed area supported in ECM as shown in Figure 1 and Figure 2, respectively.

Figure 1. Three EIP filter shapes.

Figure 2. Three types of reconstructed areas for EIP filter.

For a CU coded in the EIP mode, an EIP merge flag is first signaled to indicate whether the EIP filter is inherited from previous blocks coded in the EIP mode. When the EIP merge flag is true, an EIP merge list is constructed from the spatial adjacent, spatial non-adjacent, temporal, and history-based candidates. The position and inclusion order of these candidates are the same as those used in the CCP merge candidate list. An EIP merge index is further signaled to indicate which EIP merge candidate is selected. The filter shape and the filter coefficients of the selected candidate are then inherited to code the CU.

When the EIP merge flag is false, the EIP filter is derived from the neighboring reconstructed samples, and a relevant syntax element is signaled to indicate which one of the three types of reconstructed area and which one of the three filter shapes are used for the current CU. The selected filter moves in the selected reconstructed area either horizontally or vertically with a one-pixel step to construct the auto-correlation matrix and the cross-correlation vector. The calculation of coefficients from the auto-correlation matrix and the cross-correlation vector is the same as that in CCCM.

Proposed method

The block-vector guided EIP (BV-EIP) method[1] uses a block vector to determine the reference area for calculating the EIP filter parameters instead of directly using the adjacent spatial reference area. Then the current coding unit utilizes the calculated EIP filter parameters and neighboring reconstructed samples to create the prediction. Figure 3 illustrates the reference area in the BV-EIP method. It takes one flag to indicate the usage of this sub-mode of EIP and the block vector used in this method is derived from the rough searching process of the intraTMP in the current ECM.

Additionally, the square-shaped EIP model calculated from the reference area is added into the initial EIP merge list, where the reference area used for this model calculation is guided by the BV of the first candidate of the intraTMP BV merge list. The EIP model candidates reordering process and the final size of the EIP merge list are kept unchanged.

Figure 3. Illustrates reference area for BV-EIP

Simulation results

The proposed method was implemented on top of ECM-14.0[2]. The simulations were performed following the common test conditions[3].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.18% | 0.00% | 100.9% | 101.1% | 100.0% | 100.1% |
| Class A2 | -0.04% | -0.06% | -0.06% | 100.3% | 100.9% | 100.8% | 100.1% |
| Class B | -0.04% | 0.00% | 0.01% | 100.7% | 101.4% | 98.7% | 99.9% |
| Class C | -0.02% | -0.02% | -0.04% | 100.5% | 100.7% | 100.8% | 100.0% |
| Class E | -0.08% | 0.11% | 0.06% | 100.3% | 100.7% | 101.1% | 100.0% |
| Overall | -0.05% | -0.03% | -0.01% | 100.5% | 101.0% | 100.1% | 100.0% |
| Class D | -0.01% | -0.08% | 0.04% | 100.4% | 100.2% | 100.1% | 100.0% |
| Class F | -0.04% | 0.07% | 0.04% | 99.6% | 101.0% | 96.6% | 100.1% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.18% | -0.16% | 99.9% | 99.9% | 99.9% | 100.4% |
| Class A2* | 0.01% | 0.07% | 0.03% | 99.9% | 100.0% | 99.7% | 100.3% |
| Class B | -0.03% | -0.04% | -0.26% | 100.3% | 100.1% | 100.1% | 100.1% |
| Class C | -0.04% | -0.15% | 0.16% | 100.3% | 100.4% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.08% | -0.07% | 100.1% | 100.1% | 99.9% | 100.2% |
| Class D | 0.03% | -0.48% | 0.00% | 100.0% | 100.4% | 99.8% | 100.1% |
| Class F | -0.06% | -0.14% | -0.02% | 100.2% | 100.6% | 100.0% | 100.0% |

*: two QP points are copied from anchor data.

Conclusion

This contribution reports the test results of EE2-2.19b, which is the block-vector guided EIP. The simulation results show that the proposed method can improve the coding efficiency of ECM with a minor complexity increase. It is suggested to adopt the EE2-2.19b into the next version of ECM.

Reference

- L. Zhang, Z. Xie, Y. Yu, H. Yu, D. Wang, “Non-EE2: Non-EE2: Improvements on EIP”, JVET-AI0167, July 2024

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AI2017, July 2024

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).