| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0084-v2 |
| --- | --- |

| Title: | EE2-2.19c: a combination of EE2.19a and EE2.19b |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lai Zhang Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | zhanglai@oppo.com xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-2.19c, which is a combination of EE2-2.19a and EE2-2.19b. Both EE2-2.19a and EE2-2.19b aim to improve the coding performance of EIP, where EE2-2.19a proposes a multi-model method for EIP and may allow applying two extrapolation filters in a coding unit, and EE2-2.19b introduces a block-vector guided EIP method.

Simulation results of the combination test on top of ECM-14.0 are reported as follows:

AI: -0.10% (Y), -0.06% (U), -0.05% (V), 100.9% (EncT), 101.1% (DecT)

RA: -0.04%(Y), -0.13%(U), -0.06%(V), 100.2% (EncT), 99.9%(DecT)

Introduction

In EE2-2.19a, it is proposed that using the multi-model EIP method derives two extrapolation filters and applies them to an EIP-coded CU.

In EE2-2.19b, it is proposed that a block vector is used to determine a reference area instead of directly using the adjacent spatial reference area for calculating the EIP filter parameters

EE2-2.19c tests a combination of EE2-2.19a and EE2-2.19b.

Simulation results

The proposed method was implemented on top of ECM-14.0. The simulations were performed following the common test conditions.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0.15% | -0.12% |  | -0.10% |  | 102.1% |  | 101.3% |  | 100.0% |  | 100.5% |  |
| Class A2 | -0.10% | -0.01% |  | -0.06% |  | 101.4% |  | 101.0% |  | 100.4% |  | 100.5% |  |
| Class B | -0.07% | -0.05% |  | -0.03% |  | 100.0% |  | 100.8% |  | 100.2% |  | 100.5% |  |
| Class C | -0.06% | -0.09% |  | -0.08% |  | 100.3% |  | 101.4% |  | 100.8% |  | 100.4% |  |
| Class E | -0.15% | -0.04% |  | 0.02% |  | 101.2% |  | 101.0% |  | 101.0% |  | 100.2% |  |
| Overall | -0.10% | -0.06% |  | -0.05% |  | 100.9% |  | 101.1% |  | 100.5% |  | 100.4% |  |
| Class D | -0.03% | 0.08% |  | -0.12% |  | 100.5% |  | 100.5% |  | 100.4% |  | 100.3% |  |
| Class F | -0.10% | 0.00% |  | 0.12% |  | 99.9% |  | 100.9% |  | 101.1% |  | 100.4% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1* | -0.06% |  | -0.12% |  | -0.12% |  | 100.2% |  | 100.1% |  | 100.1% |  | 100.3% |
| Class A2* | -0.04% |  | -0.19% |  | -0.13% |  | 99.8% |  | 99.5% |  | 100.0% |  | 100.0% |
| Class B | -0.02% |  | -0.17% |  | -0.10% |  | 100.3% |  | 100.1% |  | 100.1% |  | 100.3% |
| Class C | -0.06% |  | -0.06% |  | 0.08% |  | 100.4% |  | 100.0% |  | 100.3% |  | 100.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.04% |  | -0.13% |  | -0.06% |  | 100.2% |  | 99.9% |  | 100.1% |  | 100.2% |
| Class D | 0.04% |  | -0.40% |  | -0.01% |  | 99.9% |  | 99.7% |  | 100.2% |  | 100.3% |
| Class F | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |

*: several QP points are copied from anchor data.

Conclusion

This contribution reports the test results of EE2-2.19c, which is a combination of EE2-2.19a and EE2-2.19b. The combination results show that the proposed method can bring more than 0.1% luma coding performance for AI with a minor complexity increase. It is suggested to adopt the EE2-2.19c into the next version of ECM.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).