| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0085-v1 |
| --- | --- |

| Title: | EE2-3.7: Sub-block merge mode extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Yue Yu Haoping Yu Dong Wang | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-3.7. In this test, the block size restriction for sub-block merge mode is modified to allow more CUs applicable for sub-block merge mode. In EE2-3.7a, a sub-block merge mode extension with normative conditioning is investigated. In EE2-3.7b, a sub-block merge mode extension with non-normative conditioning is investigated.

On top of ECM-14.0, simulation results are reported as follows:

EE2-3.7a:

RA {x % Y, x % U, x % V, x % EncT, x % DecT}

LDB {x % Y, x % U, x % V, x % EncT, x % DecT}

EE2-3.7b:

RA {x % Y, x % U, x % V, x % EncT, x % DecT}

LDB {x % Y, x % U, x % V, x % EncT, x % DecT}

Introduction

In VVC and ECM, sub-block merge mode includes affine merge mode and SBTMVP mode. During the development of ECM, both the affine merge mode and the SBTMVP mode are enhanced. The CU size restriction of sub-block merge mode is specified as “cbWidth >= 8 && cbHeight >= 8”. Several tools in ECM have extended their usage with less restriction but the one for sub-block merge mode remains unchanged.

It is proposed to extend the usage of sub-block merge mode in JVET-AI0169 [1] and the proposed method is investigated in the EE.

Proposed method

It is proposed to modify the CU size restriction of sub-block merge mode as “cbWidth * cbHeigh >= 32”. With this modification, CUs of size 4*8, 4*16, 4*32… and 8*4, 16*4, 32*4… can apply sub-block merge mode.

It is proposed to use neighboring blocks’ coding information to determine whether sub-block merge mode is allowed for blocks whose block size is in the grey area in Figure 1. Seven adjacent neighboring blocks and five non-adjacent neighboring blocks, as illustrated in Figure 2, are checked. If at least one of the checked neighboring blocks is coded with sub-block merge mode or inter affine mode or GPM-affine mode, sub-block merge mode is allowed for current block. Otherwise, sub-block merge mode is forbidden for current block. Separate context models are used for the sub-block merge flag for blocks whose block size is in the grey area in Figure 1.

In EE2-3.7a, the sub-block merge flag is not signaled if sub-block merge mode is forbidden by the condition check. In EE2-3.7b, the sub-block merge flag is signaled regardless of the condition check.

Figure 1                                   Figure 2

Simulation results

The proposed method is implemented on top of the ECM-14.0 software [2] and tested based on the common test condition [3]. The test results are summarized below.

EE2-3.7a:

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.08% | 0.02% | 0.06% | 100.6% | 100.3% | 100.0% | 100.0% |
| Class C | -0.14% | 0.02% | 0.11% | 101.1% | 101.0% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.21% | -0.27% | -0.01% | 101.6% | 101.8% | 99.8% | 100.0% |
| Class F | -0.12% | -0.15% | -0.18% | 100.3% | 101.0% | 100.1% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.09% | -0.11% | 0.29% | 101.8% | 100.6% | 100.0% | 100.0% |
| Class F | -0.08% | -0.41% | 0.22% | 100.9% | 101.8% | 99.3% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |

EE2-3.7b:

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.08% | -0.03% | 100.4% | 100.1% | 100.0% | 100.3% |
| Class C | -0.06% | -0.01% | 0.07% | 101.1% | 101.6% | 100.0% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.10% | -0.11% | -0.21% | 100.7% | 101.0% | 99.8% | 100.0% |
| Class F | 0.01% | 0.02% | 0.01% | 99.4% | 100.2% | 100.1% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.08% | 0.25% | -0.12% | 100.1% | 99.8% | 99.2% | 99.9% |
| Class E | 0.04% | -0.05% | -0.90% | 100.4% | 101.8% | 99.8% | 100.2% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.05% | 1.20% | 0.92% | 101.6% | 100.4% | 100.4% | 100.0% |
| Class F | 0.14% | -0.02% | -0.13% | 99.9% | 99.9% | 98.2% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |

Conclusions

It is recommended to adopt EE2-3.7a into ECM.

References

- F. Wang, Y. Yu, H. Yu, D. Wang, “Non-EE2: On sub-block merge mode”, JVET-AI0169, July 2024

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AI2017, July 2024

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).