| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, Turkey, 01–08 November 2024 | Document: JVET-AJ0088-v2 |
| --- | --- |

| Title: | Non-EE2: On MPM with matrix-based position dependent intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zexing Sun Yue Yu Haoping Yu Luhang Xu Dong Wang | Tel: Email: | sunzexing@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes modifications to the construction of the most probable mode (MPM) list relating to matrix-based position-dependent intra-prediction (MPDIP), where some non-MPDIP modes in the primary MPM may be converted to the MPDIP modes. Furthermore, for MPDIP modes in the primary MPM, the MPDIP prediction instead of the conventional angular prediction will be used to calculate the template matching cost. Simulation results of the proposed method on top of ECM-14.0 are reported below:

AI: {-0.07%, -0.06%, -0.04%, 100.0%, 100. 6%};

RA: {-0.02%, -0.15%, 0.05%, 100.0%, 99.9%};

Introduction

Matrix-based position-dependent intra-prediction (MPDIP) is an adopted coding tool in the current ECM-14.0 [1]. MPDIP replaces some of the conventional intra-prediction modes (IPMs). More specifically, MPDIP is used for mode 0, 1, and (2+2*k) angular mode for coding blocks with width and height up to 16 samples; otherwise, MPDIP is only used for mode 0, 1, and (2+4*k) as shown in Table 1,.

Table 1. The modes replaced by PDP

| Coding block size | IPMs replaced by PDP |
| --- | --- |
| W≤16 && H≤16 | 0, 1,(2+2×k) |
| Otherwise | 0, 1,(2+4×k) |

The pre-stored matrix weights are applied to the samples in an L-shaped reference area to generate the prediction samples.

Proposed method

In the current MPM list, there are 6 modes in the primary MPM and 16 modes in the secondary MPM. There are also 45 modes maintained in the non-MPM list. Among the six modes in the primary MPM, some may not be MPDIP modes. In general, if the current block is eligible for MPDIP prediction, there is a better chance to select an MPDIP mode because MPDIP may provide a better prediction. If there are more MPDIP modes in the MPM, coding of the current MPDIP mode may take fewer bits. Based on this observation, this contribution first proposes that if the current block is eligible MPDIP, i.e., the decoded left and the above reference samples are available and the current block size supports MPDIP, some of the non-MPDIP modes in the primary MPM list are converted to MPDIP modes. The processes for constructing the secondary MPM and the non-MPM lists are kept unchanged.

This contribution further proposes that for the MPDID modes in the primary MPM list, the MPDIP prediction will be used in the calculation of the template cost, instead of using the conventional angular prediction. The costs of the top and left templates, shown in Figure 1, are calculated separately and the sum of the two costs is taken as the final template cost.

Figure 1. MPDIP template cost calculation

Experimental results

The proposed method was implemented on top of ECM-14.0 [1] and tested according to the common test condition of all intra (AI) and random access (RA) configurations as specified in the JVET-AF2017 [2]. Experimental results are summarized below. More detailed results can be found in the attached spreadsheets.

Table 2 Experimental results of the proposed method over ECM-14.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.20% | -0.14% | 99.4% | 99.2% | 98.8% | 100.0% |
| Class A2 | -0.04% | -0.05% | -0.08% | 99.6% | 99.9% | 100.0% | 100.0% |
| Class B | -0.06% | 0.05% | 0.04% | 100.5% | 101.0% | 102.2% | 99.9% |
| Class C | -0.06% | -0.15% | -0.12% | 100.0% | 101.8% | 98.8% | 100.1% |
| Class E | -0.10% | -0.02% | 0.05% | 100.4% | 100.8% | 99.7% | 100.0% |
| Overall | -0.07% | -0.06% | -0.04% | 100.0% | 100.6% | 100.1% | 100.0% |
| Class D | -0.10% | 0.05% | -0.16% | 100.0% | 103.1% | 100.0% | 100.0% |
| Class F | -0.08% | -0.09% | 0.07% | 100.6% | 101.8% | 100.0% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 3 Experimental results of the proposed method over ECM-14.0.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.26% | -0.21% | 100.3% | 100.3% | 100.0% | 99.8% |
| Class A2 | 0.01% | -0.17% | -0.12% | 99.8% | 99.7% | 99.8% | 99.7% |
| Class B | -0.03% | -0.07% | 0.16% | 99.7% | 99.8% | 100.0% | 100.1% |
| Class C | -0.04% | -0.15% | 0.24% | 100.3% | 99.8% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.15% | 0.05% | 100.0% | 99.9% | 100.0% | 100.0% |
| Class D | -0.05% | -0.16% | 0.26% | 100.4% | 100.2% | 99.9% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Conclusion

This contribution proposes modifications to the construction of the MPM list. The proposed methods bring some coding gains compared to ECM-14.0 with no noticeable complexity increase. It is suggested to further study this method in the next round of EE2.

Reference

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/jvet-ai-ee2/ECM

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October. 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).