| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 Nov 2024 | Document: JVET-AJ0090-v1 |
| --- | --- |

| Title: | AHG12: temporal ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Yue Yu, Haoping Yu, Dong Wang | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

- Abstract

This contribution proposes an adaptive loop filter that uses temporal reconstructed information as the input to a filter whose output will enhance the current picture. The proposed temporal adaptive loop filter (TALF) is implemented on top of ECM-14.0 reference software. The simulation results show that the proposed method can achieve the following BD-rates gain over the anchor:

RA:   x.xx%/x.xx%/x.xx%, xxx.x%/xxx.x%EncT/DecT

LDB: x.xx%/x.xx%/x.xx%, xxx.x%/xxx.x%EncT/DecT

Introduction

In ECM-14.0, ALF uses spatial reconstructed information, residual information, deblocking information, and fixed filtered information as input to improve the current reconstructed picture. The online-trained ALF coefficients and the information on filter precision and non-linear clipping are transmitted in the bitstream. The history ALF filters can also be used in the current picture’s filtering.

This contribution proposes a temporal adaptive loop filter (TALF) that uses temporal reconstructed information to improve the current picture’s quality. Similar to ALF, the filter coefficients and related information are transmitted in the bitstream and the history of TALFs can also be used in the current picture’s filtering.

Proposed method

The proposed TALF is placed at the last in the loop filters. The use or not for TALF for each CTB is dependent on a CTB-level flags.

Figure-1. Example of TALF in loop filters.

Filtering mode

The proposed TALF has six filtering modes.

- The first filtering mode is an uni-filtering mode, which uses the integerized MV0 from the reference picture 0 to locate the filter inputs. The positions with no existing MV0 will be skipped in TALF’s first mode when the current CTB uses TALF.

- The second filtering mode is an uni-filtering mode, which uses the integerized MV1 from the reference picture 1 to locate the filter inputs. The positions with no existing MV1 will be skipped in TALF’s second mode when the current CTB uses TALF.

- The third filtering mode is a bi-filtering mode, which uses the integerized MV0 from the reference picture 0 and MV1 from the reference picture 1 to locate the filter inputs. The positions with no existing MV0 and MV1 will be skipped in TALF’s third mode when the current CTB uses TALF.

- The forth filtering mode is an uni-filtering mode, it uses the collocated reconstructed samples in the closest reference picture as the filter inputs.

- The fifth filtering mode is an uni-filtering mode, it uses the collocated reconstructed samples in the second closest reference picture as the filter inputs.

- The sixth filtering mode is a bi-filtering mode, it uses the collocated reconstructed samples in the first and the second closest reference pictures as the filter inputs.

Figure-2. The inputs of the uni-filtering mode are located by the uni-MV.

Figure-3. The inputs of the bi-filtering mode are located by the MV0 and MV1.

Filter shape

The proposed TALF has two uni-filtering shapes and two bi-filtering shapes. Filters for uni-filtering have 13 coefficients and filters for bi-filtering have 14 coefficients. A syntax element is signaled to indicate the filter shape applied for the current slice.

Figure-4. The filter shape 0 for bi-filtering (left) and uni-filtering (right)

Figure-5. The filter shape 1 for bi-filtering (left) and uni-filtering (right)

Other features

There are two history tables for storing the history uni-TALF filters and bi-TALF filters and up to 8 sets of filters can be stored in each table. The current slice can reuse the history filters in the history tables.

The non-linear clip method is adopted in TALF. The clip values are same to the ALF.

The adaptive precision method is adopted in TALF. The precision for the absolute TALF coefficients can be 6, 7, 8 or 9 bits.

Simulation results

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |
| Class B | -0.19% | 0.30% | 0.39% |  |  | 100.5% | 100.5% |
| Class C | -0.08% | 0.22% | 0.29% |  |  | 100.9% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |
| Class D | -0.08% | 0.25% | 0.31% |  |  | 100.9% | 100.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |
| Class C | -0.16% | 0.23% | -0.07% |  |  | 100.6% | 100.1% |
| Class E | -0.08% | 1.04% | 0.76% |  |  | 102.3% | 100.2% |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |
| Class D | -0.10% | 0.38% | 0.54% |  |  | 101.0% | 100.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! |  |  | #DIV/0! | #DIV/0! |

Conclusions

The proposed TALF can bring x.xx%/x.xx%/x.xx% BD-rates gain for RA and x.xx%/x.xx%/x.xx% BD-rates change for LDB on top of ECM-14.0. It is suggested to further study the proposed method in the next round of EE2.

References

- ECM14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).