| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1-8 Nov. 2024 | Document: JVET-AJ0091-v2 |
| --- | --- |

| Title: | AHG9/AHG13: Updates on Film grain regions characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shaowei Xie, Ping Wu, Ying Gao, Yaxian Bai, Cheng Huang | Email: | xie.shaowei@zte.com.cn, ping.wu@zte.com.cn, huang.cheng5@zte.com.cn |
| Source: | ZTE |  |  |

_____________________________

- Abstract

While the model adaptation method proposed in JVET-AH0166 [1] was adopted into the VSEI TuC doc, i.e., JVET-AI2032 [2], some necessary modifications to the related syntax and semantics in the Film Grain Regions Characteristics (FGR) SEI message are proposed in this contribution to ensure the standard usage of adaptive film grain technology.

Introduction

At the 33rd JVET meeting, the Film Grain Regions Characteristics (FGR) SEI message was documented into the VESI TuC, where a list of film grain regions are described and each region is associated with a set of intensity intervals. Furthermore, for improving the adaptive ability of the FGR SEI message, the film grain model adaptation allowing separate film grain model_ids to be applied in different regions (fgr_region_model_id), proposed in JVET-AH0166, was adopted into the FGR SEI message. In this contribution, some necessary modifications to the FGR SEI message are made to ensure the standard usage of the regional and model adaptive film grain technology.

Proposed modifications to the FGR SEI message

When fgr_single_model_flag is equal to 1, the identifications of the film grain simulation models used in current picture have the same value, named as “fgr_model_id”. When fgr_single_model_flag is equal to 0, the identifications of the film grain simulation models used in different regions of the current picture may have different values, named as “fgr_region_model_id[ i ]” for the i-th region.

While “fgr_model_id” and “fgr_region_model_id[ i ]” are both used to indicate the film grain simulation models, but in different situations, they share the same identification value definition of the film grain simulation models as specified in Table 5. In order to unify the roles of both in calculating film grain parameters or values, we propose to add the variable “fgrModelidValue”.

Let variable fgrModelidValue specify the identification value of the film grain simulation model, and is derived as follows:

–	If fgr_single_model_flag is equal to 1, the following applies:

fgrModelidValue = fgr_model_id

–	Otherwise (fgr_single_model_flag is equal to 0), the following applies:

fgrModelidValue = fgr_region_model_id[ i ]

Table 5 - fgr_model_id or fgr_regions_model_id[ i ] values

| Value | Description |
| --- | --- |
| 0 | Frequency filtering |
| 1 | Auto-regression |

- Along with above proposal, we basically replace the wording “fgr_model_id” with “fgrModelidValue”, apart from its definition of “fgr_model_id”. Refer to the latest VSEI TuC doc, i.e., JVET-AI2032, we can see more detailed revisions in the attached content.

Summary

To ensure the simultaneous application of more than one film grain simulation model in a picture, it is recommended to check and make some necessary modifications to the related semantics in the Film Grain Regions Characteristics (FGR) SEI message.

Reference

- Shaowei Xie, Ping Wu, Ying Gao, Yaxian Bai, Menghu Jia, Weihong Niu, Cheng Huang (ZTE), AHG9: Improvements to the film grain regions characteristics SEI message, JVET-AH0166, Apr. 2024.

- Sean McCarthy, Jill Boyce, Jie Chen, Sachin Deshpande, Miska M. Hannuksela, Hendry Tan, Ye-Kui Wang, Technologies under consideration for future extensions of VSEI (version 5), JVET-AI2032, July 2024.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).