| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1-8 Nov. 2024 | Document: JVET-AJ0092-v2 |
| --- | --- |

| Title: | AHG9/AHG13: Comments on Film grain regions characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shaowei Xie, Ping Wu, Ying Gao, Yaxian Bai, Cheng Huang | Email: | xie.shaowei@zte.com.cn, ping.wu@zte.com.cn, huang.cheng5@zte.com.cn |
| Source: | ZTE |  |  |

_____________________________

- Abstract

While the model adaptation method proposed in JVET-AH0166 [1] was adopted into the VSEI TuC doc, i.e., JVET-AI2032 [2], some model-adaptive modifications to the related syntax and semantics in the Film Grain Regions Characteristics (FGR) SEI message are proposed in this contribution to ensure the standard usage of adaptive film grain technology.

Introduction

At the 33rd JVET meeting, the Film Grain Regions Characteristics (FGR) SEI message was documented into the VESI TuC, where a list of film grain regions are described and each region is associated with a set of intensity intervals. Furthermore, for improving the adaptive ability of the FGR SEI message, the film grain model adaptation allowing separate film grain model_ids to be applied in different regions (fgr_regions_model_id), proposed in JVET-AH0166, was adopted into the FGR SEI message. In this contribution, besides a variable “fgrModelidValue” is added to unify film grain calculation, some model-adaptive modifications to the related syntax and semantics in the FGR SEI message are made to ensure the standard usage of the regional and model adaptive film grain technology.

Proposed modifications to the FGR SEI message

In the latest VSEI TuC doc JVET-AI2032, “fgr_regions_model_id[ i ]” is introduced to indicate the film grain simulation models used in different regions of the current picture. Relatively, the original “fgr_model_id” is used to indicate the single film grain model, if only one model is applied in the current picture. While “fgr_model_id” and “fgr_regions_model_id[ i ]” share the same identification of the film grain simulation models, we propose to add a variable “fgrModelidValue” to unify their roles in calculating film grain parameters or values. Following the above explanation, we have made detailed revisions in JVET-AI2032, referring to the attached file.

Furthermore, based on our analysis and understanding, applying more than one film grain simulation model in a picture simultaneously will affect the assignment of the syntax in the FGR SEI message. Specifically, as shown in Figure 1, the blue highlighted contents originally can only indicate the parameters of a single film grain simulation model. Therefore, we propose to modify this part of the FGR SEI message syntax to support multi-model application.

Figure 1 - Original Part of the FGR SEI message syntax

Firstly, we propose to add the flag “fgr_model_present_flag[ m ]” to indicate the presentation of any specific film grain simulation model (m-th model). “fgr_model_present_flag[ m ]” equal to 0 indicates that the film grain simulation model with fgrModelidValue equal to m is not applied, where m equal to 0 refers to the frequency Filtering based film grain simulation model, m equal to 1 refers to the Auto-regression based film grain simulation model, as specified in Table 5. “fgr_model_present_flag[ m ]” equal to 1 indicates that the syntax elements specifying modelling of film grain with fgrModelidValue equal to m are present in the SEI message.

Secondly, we propose to add the variable “fgrModelidMax” to represent the maximum index of the film grain simulation models supported in this version of this Specification, as specified by Table 5. Specifically, while the value of fgrModelidValue shall be in the range of 0 to 1, inclusive, the value of fgrModelidMax shall be set equal to 1 in this version of this Specification.

The modified FGR SEI message syntax is shown in Figure 2. And we can find more detailed revisions in the attached content (JVET-AI2032).

Figure 2 - Modified part of the FGR SEI message syntax

Summary

To ensure the simultaneous application of more than one film grain simulation model in a picture, it is recommended to check and make some model-adaptive modifications to the related syntax and semantics in the Film Grain Regions Characteristics (FGR) SEI message.

Reference

- Shaowei Xie, Ping Wu, Ying Gao, Yaxian Bai, Menghu Jia, Weihong Niu, Cheng Huang (ZTE), AHG9: Improvements to the film grain regions characteristics SEI message, JVET-AH0166, Apr. 2024.

- Sean McCarthy, Jill Boyce, Jie Chen, Sachin Deshpande, Miska M. Hannuksela, Hendry Tan, Ye-Kui Wang, Technologies under consideration for future extensions of VSEI (version 5), JVET-AI2032, July 2024.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).