| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0093-v1 |
| --- | --- |

| Title: | AHG11: A Neural Network Downscaling Filter for RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yiqing Zhu, Wenxuan He, Xu Li, Jiedong Ye, Qiong Liu | Email: | yiqingzhu@hust.edu.cn q.liu@hust.edu.cn |
| Source: | Huazhong University of Science and Technology |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network (CNN) downscaling filter to improve the  reference picture resampling (RPR) downsampling. When RPR option on, the proposed technique use the proposed downsacling network to produce low-resolution frames replacing than RPR downsampled frames. The architecture is designed with low complexity, reducing computational resource consumption while maintaining satisfactory performance.

BD-rate changes on top of NNNC-8.0 anchor are reportedly summarized as below:

RA: -%, -%, -%, AI: -%, -%, -%

Complexity: 5.2 kMAC/pixel

- Introduction

In this contribution, the RPR downsampled frames are replaced by those produced by the proposed CNN downscaling filter at the input stage. As illustrated in Fig. 1, the proposed network takes the original frame and the low-resolution RPR frame as input to generate a reconstructed frame. This reconstructed frame replaces the low-resolution RPR frame for subsequent coding processes.

Fig. 1. Illustration of the integration for the CNN-based downscaling filter for RPR.

- Proposed method

The proposed downscaling network architecture is illustrated in Fig. 2.

Fig. 2. Proposed downscaling network architecture.

  - Inference

SADL is used for performing the inference of the proposed CNN downscaling filter in VTM. The network information in the inference stage is provided in Table 1.

Table 1. Network Information in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 0.022M |
|  | Parameter Precision (Bits) | float |
|  | Memory Parameter (MB) | Total Parameters Number × 2 |
|  | Multiply Accumulate (kMAC/pixel) | 5.2 |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 256×256 |
|  | Peak Memory Usage |  |
|  | Other information: |  |

  - Training

The network information in the training stage is provided in Table 2.

Table 2. Network Information in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 10 |
|  | Batch size: | 128 |
|  | Training time: | 10h |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn on NNLF-LOP2 and NNIntra, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |

- 3	Experimental results

The proposed model is integrated into NNVC-8.0 and tested according to the common test conditions defined in [1]. As shown in Table 3, the proposed method brings on average {-%, -%, -%} and {-%, -%, -%} BD-rate changes for {Y, Cb, Cr} compared with NNVC-8.0 anchor under RA and AI configurations.

Table 3. Coding performance over NNVC8.0 anchor

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -% | % | % |
| Class A2 | -% | % | % |
| Class B | -% | % | % |
| Class C | -% | % | % |
| Class E |  |  |  |
| Overall | -% | % | % |
|  | All Intra Main10 |  |  |
|  | BD-rate Over NNVC-8.0 Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.63% | 3.05% | 4.04% |
| Class A2 | -1.51% | 7.31% | 9.37% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| Overall | -0.52% | 1.73% | 2.23% |

- 4	Conclusion

This contribution proposes a CNN-based downscaling filter integrated within the RPR framework, designed to substitute the RPR downsampled frames with high-quality downscaled frames, which reduce the stream flow.

The Proposed architecture achieves better performance than default RPR by AI {-0.52%, 1.73%, 2.23%} and RA {-%, %, %} with only 5.2KMAC/pixel rate raising. It is recommended that the proposed method to be adopted into NNVC.

- 5	References

- Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AF2016, E. Alshina, R. -L. Liao, S. Liu, A. Segall, Hannover, DE, October 2023.

- 6	Patent rights declaration(s)

Vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).