| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0095 |
| --- | --- |

| Title: | EE2-3.6: GPM-affine with TM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Na Zhang1, Zhipin Deng1, Wenbin Yin1, Lei Zhao1, Mehdi Salehifar2, Li Zhang2 1Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results for EE2 Test 3.6 on GPM-affine harmonized with GPM-TM. On top of ECM-14.0, the simulation results are reported as below:

RA: {%, %, %; %, %};

LDB: {%, %, %; %, %}.

Introduction

GPM-affine was proposed to be harmonized with GPM-TM (GPM-affine-TM) in JVET-AI0083 [1] and established as Test 3.6 in JVET-AI EE2 [2]. In Test 3.6, an affine candidate could be refined by template matching (TM) using the same method as affine mode. A GPM sub-partition can be predicted by GPM-affine-TM or GPM-TM.

Simulation results

EE2 Test 3.6 was implemented on top of ECM-14.0 [3]. Simulations were performed following the common test conditions of EE2 [4]. The simulation results compared to ECM-14.0 are shown in the following table.

Table 1: Test results of Test 3.6

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.03% | -0.10% | -0.02% | 100.1% | 99.9% |  |  |  |  |  |
| Class C | -0.07% | -0.01% | 0.09% | 100.6% | 100.0% | -0.11% | 0.21% | -0.57% | 100.8% | 100.4% |
| Class E |  |  |  |  |  | -0.27% | 0.00% | -0.43% | 101.7% | 99.4% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.04% | -0.09% | -0.02% | 99.2% | 100.0% | -0.06% | 0.35% | 0.74% | 101.3% | 99.3% |
| Class F | -0.02% | -0.09% | 0.01% | 100.0% | 100.3% |  |  |  |  |  |

Conclusions

In this contribution, the test results of EE2 Test 3.6 on GPM-affine with TM are reported. It is recommended to adopt Test 3.6 into ECM.

Reference

- Y. Wang, K. Zhang, and L. Zhang, “EE2-3.8: GPM-affine with MMVD and TM,” Document of Joint Video Experts Team, JVET-AI0083, Sapporo, JP, 12–19 July 2024.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Strom, F. Wang, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-AI2024, Sapporo, JP, 12–19 July 2024.

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, Sapporo, JP, 12–19 July 2024.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).